### 3.3.4 Portable Fuel Containers (PFC) emission inventories

As part of the MSAT rule, emissions for portable fuel containers (PFC) would be processed in EMS-HAP, ASPEN, and subsequent HAPEM exposure modeling.  In order to create the emissions inventories for the MSAT HAPs, two main steps were taken.  First, state level VOC PFC emissions were allocated to counties and to several SCC codes.  Secondly, after allocation of the VOC emissions, HAP specific emissions were developed.  This section describes the processes in both steps.

#### 3.3.4.1 VOC allocation

VOC total PFC reference (uncontrolled) emissions were available for 1990, 2005, 2010, 2015, 2020, and 2030 by state.  In addition to the reference inventories, there were control emissions for 2010, 2015, 2020, and 2030.  In addition to the years listed, a 1999 reference inventory was needed.  The 1999 inventory would be created based on linear interpolation between the 1990 and 2005 inventories.

For both the reference and control inventories, the state VOC emissions needed to be allocated to counties and to SCC codes related to PFC emissions.  The following steps were used to allocate the VOC emissions:

- For each year, the reference inventories were read into SAS®.  For 2010, 2015, 2020, and 2030, the control inventories were read into SAS®.

- The state level VOC emissions for each year and emissions scenario, reference or control, were allocated to residential and commercial components for six categories:  1) vapor displacement while refilling containers at the pump, 2) spillage while refilling at the pump, 3) spillage during transport, 4) vapor displacement while refueling equipment, 5) spillage while refueling equipment, and 6) permeation and evaporation.  Total state level PFC emissions were allocated to the categories by using national level residential and commercial emissions (Table 30) for each of the categories using the following equations:

Eresidential,XXXX,YY=E×ResRes+Com						       (9)

Ecommercial,XXXX,YY=E×ComRes+Com 					     (10)

where E was the emissions of the category being split, XXXX was year, YY was state, and Res and Com were the emissions shown in Table 30.

- After allocating the VOC emissions to the six categories, the commercial and residential permeation and evaporation categories were split into commercial permeation, commercial evaporation, residential permeation, and residential evaporation by

EAAA,XXXX,YY,perm=EAAA,XXXX,YY,perm∧evap×0.3387				     (11)

EAAA,XXXX,YY,evap=EAAA,XXXX,YY,perm∧evap×(1−0.3387)				     (12)

The fraction 0.3387 represents the fraction of combined permeation and evaporative emissions attributable to permeation, based on data from the California Air Resources Board.

- Once the state VOC emissions were allocated to the residential and commercial components of the categories, they were assigned SCC codes for later processing in EMS-HAP.  These codes are shown in Table 31.

.

- After creating the SCC level state emissions for the years and emission scenarios, a 1999 reference inventory was created by interpolating from the 1990 to 2005 emissions.  The interpolation was done for each state and SCC combination and the equation was:

E1999,YY,SCC=E1990,YY,SCC+9×E2005,YY,SCC−E1990,YY,SCC15			     (13)

where E1999,YY,SCC, E1990,YYY,SCC, and E2005,YY,SCC were the 1999, 1990, and 2005 emissions for state YY and SCC shown in Table 31.

- After creating the 1999 state VOC inventory, the state emissions were allocated to the counties by using the ratio of county to state fuel consumption.  State emissions were multiplied by the county specific ratio to yield a county specific VOC emissions number for each SCC.  This equation is shown as Equation 14.  EXXXX,YYYYY,AAA,SCC=EXXXX,YY,AAA,SCC×ConsumptionYYYYYConsumptionYY			     (14)

where EXXXX,YYYYY,AAA,SCC were the emissions for year XXXX, county with FIPS code YYYYY, emission scenario AAA (reference or control) and SCC shown in Table 31, EXXXX,YY,AAA,SCC were the state level emissions for year XXXX, state YY, emission scenario AAA, and SCC in Table 31, ConsumptionYYYY was the county fuel consumption and ConsumptionYY was the state fuel consumption.

- As for the nonroad emissions, Broomfield County emissions were allocated to surrounding counties.

Figure 9 shows the flow of steps for allocation of VOC emissions.

Figure 1.  Steps in allocation of state VOC PFC emissions to counties.

Table 1.  PFC categories with national level residential and commercial emissions.

| Category | Residential Emissions | Commercial Emissions |
| --- | --- | --- |
| Vapor displacement while refilling at the pump | 4,328 | 8,341 |
| Spillage displacement while refilling at the pump | 382 | 735 |
| Spillage during transport | 13,519 | 18,442 |
| Vapor displacement while refueling equipment | 4,328 | 8,341 |
| Spillage while refueling equipment | 21,340 | 41,747 |
| Permeation and evaporation | 187,757 | 5,997 |

Table 2.  SCC codes of PFC categories.

| SCC code | Description |
| --- | --- |
| 2501011011 | Storage and Transport; Petroleum and Petroleum Product Storage; Residential Portable Gas Cans; Permeation |
| 2501011012 | Storage and Transport; Petroleum and Petroleum Product Storage; Residential Portable Gas Cans; Evaporation |
| 2501011013 | Storage and Transport; Petroleum and Petroleum Product Storage; Residential Portable Gas Cans; Spillage During Transport |
| 2501011014 | Storage and Transport; Petroleum and Petroleum Product Storage; Residential Portable Gas Cans; Refilling at the Pump - Vapor Displacement |
| 2501011015 | Storage and Transport; Petroleum and Petroleum Product Storage; Residential Portable Gas Cans; Refilling at the Pump - Spillage |
| 2501011016 | Storage and Transport; Petroleum and Petroleum Product Storage; Residential Portable Gas Cans; Refueling Equipment - Vapor Displacement |
| 2501011017 | Storage and Transport; Petroleum and Petroleum Product Storage; Residential Portable Gas Cans; Refueling Equipment - Spillage |
| 2501012011 | Storage and Transport; Petroleum and Petroleum Product Storage; Commercial Portable Gas Cans; Permeation |
| 2501012012 | Storage and Transport; Petroleum and Petroleum Product Storage; Commercial Portable Gas Cans; Evaporation |
| 2501012013 | Storage and Transport; Petroleum and Petroleum Product Storage; Commercial Portable Gas Cans; Spillage During Transport |
| 2501012014 | Storage and Transport; Petroleum and Petroleum Product Storage; Commercial Portable Gas Cans; Refilling at the Pump - Vapor Displacement |
| 2501012015 | Storage and Transport; Petroleum and Petroleum Product Storage; Commercial Portable Gas Cans; Refilling at the Pump - Spillage |
| 2501012016 | Storage and Transport; Petroleum and Petroleum Product Storage; Commercial Portable Gas Cans; Refueling Equipment - Vapor Displacement |
| 2501012017 | Storage and Transport; Petroleum and Petroleum Product Storage; Commercial Portable Gas Cans; Refueling Equipment - Spillage |

#### 3.3.4.2 Creation of HAP PFC inventories

Once the state VOC PFC emissions were allocated to counties and SCC codes, PFC emissions for MSAT HAPs could be developed.  Two methods were used to create the emissions, one for benzene, and the second for other HAPs.  For benzene, the county level light duty gasoline vehicle (LDGV) refueling emissions for benzene and VOC were used to create the PFC emissions.  At the county level, the benzene refueling emissions were divided by the VOC refueling emissions, to yield a ratio that would be multiplied with the PFC VOC emissions.  Benzene fuel control refueling emissions would be used for refueling control emissions while no fuel controls would be used for 1999 and all the future year reference inventories.  Several combinations of PFC and benzene fuel control refueling ratios would be used.  These combinations were composed of the PFC emissions with no controls, and with controls, and with benzene refueling emissions with and without controls.  Table 32 lists the combinations and years for which they were used.

Table 3.  PFC and benzene fuel control inventory scenarios.

| PFC emissions | Benzene refueling emissions | Years |
| --- | --- | --- |
| No controls | No controls | 1999, 2010, 2015, 2020, 2030 |
| Controls | Controls | 2015, 2020, 2030 |
| No controls | Controls | 2015, 2020, 2030 |
| Controls | No controls | 2010, 2015, 2020, 2030 |

To calculate the benzene emissions for each PFC SCC in each county the following formulas were used.  For all SCC emissions except for permeation (residential and commercial) the benzene emissions were calculated as:

BenzeneAAA,XXXX,YYYYY,SCC=VOCAAA,XXXX,YYYYY,SCC×Benzenerefuel,XXXX,YYYYY,BBBVOCrefuel,XXXX,YYYYY,BBB×0.36	     (15)

For permeation emissions, the equation was

BenzeneAAA,XXXX,YYYYY,SCC=VOCAAA,XXXX,YYYYY,SCC×Benzenerefuel,XXXX,YYYYY,BBBVOCrefuel,XXXX,YYYYY,BBB×0.36×1.77	     (16)

where XXXX was the year, YYYYY was the FIPS code of the county, and SCC was an SCC code shown in Table 31.  AAA represents no controls or controls for PFC emissions, and BBB represents whether refueling emissions are control or uncontrolled as in Table 32.  Note that 1999 and 2010 uncontrolled benzene refueling emissions were calculated from 2015 as done in the onroad emissions processing.

In the equations the factor 0.36 represents an adjustment based on the nationwide percentage of benzene in gasoline vapor from gasoline distribution with an RVP of 10 psi at 60˚F (Hester, 2006).  The percentage is 0.27%, in contrast to 0.74% benzene in vehicle refueling emissions from highway vehicles.  The ratio or factor of 0.36 was applied to the refueling emissions.  A second ratio was used for permeation emissions since recent research suggests that the ratio of benzene from permeation is higher than for evaporation, vapor displacement or spillage.  A recent study (Haskew et al., 2004) suggests that the ratio of benzene from permeation to total VOC from permeation is about 1.7727 times higher than the ratio associated with evaporation.

For all other HAPs, the PFC emissions were created by multiplying the PFC VOC emissions by the county level ratio of HAP LDGV evaporative emissions by the VOC LDGV evaporative emissions for the county or:

HAPAAA,XXXX,YYYYY,SCC=VOCAAA,XXXX,YYYYY,SCC×HAPLDGV,XXXX,YYYYYVOCLDGV,XXXX,YYYYY				     (17)

where the subscripts are as denoted previously.  Using the LDGV evaporative emissions means only HAPs in the onroad inventory with LDGV evaporative emissions would have PFC emissions.  For all other HAPs, the same formula was used for all SCC codes.  Naphthalene was also multiplied by a factor of 0.0054 to reduce the emissions.  Table 33 lists the emissions summaries for the no controls inventories and Table 34 lists the emissions summaries for the controlled inventories.  Steps used in creating the HAP inventories are shown in Figure 10.

Figure 2.  Steps in creating HAP PFC emissions.

Table 4.  PFC emissions with and without controls for no benzene fuel controls.

| PFC type | HAP | Year |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1999 | 2010 |  | 2015 |  | 2020 |  | 2030 |  |
|  |  | PFC: no controls | PFC: no controls | PFC: with controls | PFC: no controls | PFC: with controls | PFC: no controls | PFC: with controls | PFC: no controls | PFC: with controls |
| Commercial PFC: Evaporation | Benzene | 9.30x100 | 9.03x100 | 8.62x100 | 9.55x100 | 7.72x10-1 | 1.02x101 | 8.24x10-1 | 1.15x101 | 9.33x10-1 |
|  | All HAPs | 4.61x102 | 3.92x102 | 3.71x102 | 4.13x102 | 3.93x101 | 4.37x102 | 4.14x101 | 4.92x102 | 4.65x101 |
| Commercial PFC: Permeation | Benzene | 8.43x100 | 8.18x100 | 7.82x100 | 8.66x100 | 7.00x10-1 | 9.25x100 | 7.47x10-1 | 1.05x101 | 8.46x10-1 |
|  | All HAPs | 2.40x102 | 2.04x102 | 1.94x102 | 2.15x102 | 2.04x101 | 2.28x102 | 2.15x101 | 2.57x102 | 2.42x101 |
| Commercial PFC: Refilling at the Pump: Spillage | Benzene | 1.61x100 | 1.80x100 | 1.80x100 | 1.96x100 | 1.96x100 | 2.12x100 | 2.12x100 | 2.45x100 | 2.45x100 |
|  | All HAPs | 8.42x101 | 8.73x101 | 8.73x101 | 9.44x101 | 9.44x101 | 1.01x102 | 1.01x102 | 1.16x102 | 1.16x102 |
| Commercial PFC: Refilling at the Pump: Vapor Displacement | Benzene | 1.83x101 | 2.05x101 | 2.05x101 | 2.23x101 | 2.23x101 | 2.41x101 | 2.41x101 | 2.79x101 | 2.79x101 |
|  | All HAPs | 9.57x102 | 9.90x102 | 9.90x102 | 1.07x103 | 1.07x103 | 1.14x103 | 1.14x103 | 1.31x103 | 1.31x103 |
| Commercial PFC: Refueling Equipment: Spillage | Benzene | 1.00x102 | 8.95x101 | 5.30x101 | 9.66x101 | 5.72x101 | 1.05x102 | 6.20x101 | 1.21x102 | 7.16x101 |
|  | All HAPs | 5.17x103 | 4.09x103 | 2.58x103 | 4.40x103 | 2.78x103 | 4.72x103 | 2.97x103 | 5.41x103 | 3.41x103 |
| Commercial PFC: Refueling Equipment: Vapor Displacement | Benzene | 1.83x101 | 2.05x101 | 2.05x101 | 2.23x101 | 2.23x101 | 2.41x101 | 2.41x101 | 2.79x101 | 2.79x101 |
|  | All HAPs | 9.57x102 | 9.90x102 | 9.90x102 | 1.07x103 | 1.07x103 | 1.14x103 | 1.14x103 | 1.31x103 | 1.31x103 |
| Commercial PFC: Spillage During Transport | Benzene | 4.10x101 | 4.44x101 | 4.35x101 | 4.81x101 | 4.29x101 | 5.20x101 | 4.64x101 | 5.99x101 | 5.34x101 |
|  | All HAPs | 2.12x103 | 2.11x103 | 2.07x103 | 2.28x103 | 2.06x103 | 2.43x103 | 2.20x103 | 2.78x103 | 2.52x103 |
| Residential PFC: Evaporation | Benzene | 2.91x102 | 2.83x102 | 2.70x102 | 2.99x102 | 2.42x101 | 3.19x102 | 2.58x101 | 3.62x102 | 2.92x101 |
|  | All HAPs | 1.44x104 | 1.23x104 | 1.16x104 | 1.29x104 | 1.23x103 | 1.37x104 | 1.30x103 | 1.54x104 | 1.46x103 |
| Residential PFC: Permeation | Benzene | 2.64x102 | 2.56x102 | 2.45x102 | 2.71x102 | 2.19x101 | 2.90x102 | 2.34x101 | 3.28x102 | 2.65x101 |
|  | All HAPs | 7.50x103 | 6.39x103 | 6.06x103 | 6.75x103 | 6.40x102 | 7.14x103 | 6.75x102 | 8.04x103 | 7.58x102 |
| Residential PFC: Refilling at the Pump: Spillage | Benzene | 8.37x10-1 | 9.37x10-1 | 9.37x10-1 | 1.02x100 | 1.02x100 | 1.10x100 | 1.10x100 | 1.27x100 | 1.27x100 |
|  | All HAPs | 4.38x101 | 4.54x101 | 4.54x101 | 4.91x101 | 4.91x101 | 5.25x101 | 5.25x101 | 6.02x101 | 6.02x101 |
| Residential PFC: Refilling at the Pump: Vapor Displacement | Benzene | 9.51x100 | 1.07x101 | 1.07x101 | 1.16x101 | 1.16x101 | 1.25x101 | 1.25x101 | 1.45x101 | 1.45x101 |
|  | All HAPs | 4.97x102 | 5.14x102 | 5.14x102 | 5.55x102 | 5.55x102 | 5.94x102 | 5.94x102 | 6.82x102 | 6.82x102 |
| Residential PFC: Refueling Equipment: Spillage | Benzene | 5.11x101 | 4.57x101 | 2.71x101 | 4.94x101 | 2.93x101 | 5.35x101 | 3.17x101 | 6.18x101 | 3.66x101 |
|  | All HAPs | 2.64x103 | 2.09x103 | 1.32x103 | 2.25x103 | 1.42x103 | 2.41x103 | 1.52x103 | 2.77x103 | 1.74x103 |
| Residential PFC: Refueling Equipment: Vapor Displacement | Benzene | 9.51x100 | 1.07x101 | 1.07x101 | 1.16x101 | 1.16x101 | 1.25x101 | 1.25x101 | 1.45x101 | 1.45x101 |
|  | All HAPs | 4.97x102 | 5.14x102 | 5.14x102 | 5.55x102 | 5.55x102 | 5.94x102 | 5.94x102 | 6.82x102 | 6.82x102 |
| Residential PFC: Spillage During Transport | Benzene | 3.00x101 | 3.26x101 | 3.19x101 | 3.53x101 | 3.15x101 | 3.81x101 | 3.40x101 | 4.39x101 | 3.92x101 |
|  | All HAPs | 1.56x103 | 1.55x103 | 1.52x103 | 1.67x103 | 1.51x103 | 1.78x103 | 1.62x103 | 2.04x103 | 1.85x103 |

Table 5.  PFC emissions with and without controls for benzene fuel controls.

| PFC type | HAP | Year |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2015 |  | 2020 |  | 2030 |  |
|  |  | PFC: no controls | PFC: with controls | PFC: no controls | PFC: with controls | PFC: no controls | PFC: with controls |
| Commercial PFC: Evaporation | Benzene | 5.79x100 | 5.05x10-1 | 6.19x100 | 5.39x10-1 | 7.00x100 | 6.10x10-1 |
|  | All HAPs | 4.10x102 | 3.91x101 | 4.33x102 | 4.12x101 | 4.88x102 | 4.62x101 |
| Commercial PFC: Permeation | Benzene | 5.25x100 | 4.58x10-1 | 5.61x100 | 4.89x10-1 | 6.35x100 | 5.53x10-1 |
|  | All HAPs | 2.12x102 | 2.02x101 | 2.24x102 | 2.13x101 | 2.53x102 | 2.39x101 |
| Commercial PFC: Refilling at the Pump: Spillage | Benzene | 1.25x100 | 1.25x100 | 1.35x100 | 1.35x100 | 1.56x100 | 1.56x100 |
|  | All HAPs | 9.37x101 | 9.37x101 | 1.00x102 | 1.00x102 | 1.15x102 | 1.15x102 |
| Commercial PFC: Refilling at the Pump: Vapor Displacement | Benzene | 1.42x101 | 1.42x101 | 1.54x101 | 1.54x101 | 1.77x101 | 1.77x101 |
|  | All HAPs | 1.06x103 | 1.06x103 | 1.14x103 | 1.14x103 | 1.30x103 | 1.30x103 |
| Commercial PFC: Refueling Equipment: Spillage | Benzene | 6.02x101 | 3.66x101 | 6.52x101 | 3.97x101 | 7.53x101 | 4.58x101 |
|  | All HAPs | 4.36x103 | 2.76x103 | 4.68x103 | 2.95x103 | 5.37x103 | 3.38x103 |
| Commercial PFC: Refueling Equipment: Vapor Displacement | Benzene | 1.42x101 | 1.42x101 | 1.54x101 | 1.54x101 | 1.77x101 | 1.77x101 |
|  | All HAPs | 1.06x103 | 1.06x103 | 1.14x103 | 1.14x103 | 1.30x103 | 1.30x103 |
| Commercial PFC: Spillage During Transport | Benzene | 3.05x101 | 2.74x101 | 3.30x101 | 2.96x101 | 3.80x101 | 3.41x101 |
|  | All HAPs | 2.26x103 | 2.05x103 | 2.41x103 | 2.19x103 | 2.76x103 | 2.50x103 |
| Residential PFC: Evaporation | Benzene | 1.81x102 | 1.58x101 | 1.94x102 | 1.69x101 | 2.19x102 | 1.91x101 |
|  | All HAPs | 1.28x104 | 1.22x103 | 1.36x104 | 1.29x103 | 1.53x104 | 1.45x103 |
| Residential PFC: Permeation | Benzene | 1.64x102 | 1.43x101 | 1.76x102 | 1.53x101 | 1.99x102 | 1.73x101 |
|  | All HAPs | 6.64x103 | 6.33x102 | 7.02x103 | 6.67x102 | 7.91x103 | 7.49x102 |
| Residential PFC: Refilling at the Pump: Spillage | Benzene | 6.50x10-1 | 6.50x10-1 | 7.03x10-1 | 7.03x10-1 | 8.13x10-1 | 8.13x10-1 |
|  | All HAPs | 4.87x101 | 4.87x101 | 5.21x101 | 5.21x101 | 5.97x101 | 5.97x101 |
| Residential PFC: Refilling at the Pump: Vapor Displacement | Benzene | 7.36x100 | 7.36x100 | 7.97x100 | 7.97x100 | 9.20x100 | 9.20x100 |
|  | All HAPs | 5.51x102 | 5.51x102 | 5.90x102 | 5.90x102 | 6.76x102 | 6.76x102 |
| Residential PFC: Refueling Equipment: Spillage | Benzene | 3.08x101 | 1.87x101 | 3.33x101 | 2.03x101 | 3.85x101 | 2.34x101 |
|  | All HAPs | 2.23x103 | 1.41x103 | 2.39x103 | 1.51x103 | 2.74x103 | 1.73x103 |
| Residential PFC: Refueling Equipment: Vapor Displacement | Benzene | 7.36x100 | 7.36x100 | 7.97x100 | 7.97x100 | 9.20x100 | 9.20x100 |
|  | All HAPs | 5.51x102 | 5.51x102 | 5.90x102 | 5.90x102 | 6.76x102 | 6.76x102 |
| Residential PFC: Spillage During Transport | Benzene | 2.24x101 | 2.01x101 | 2.42x101 | 2.17x101 | 2.78x101 | 2.50x101 |
|  | All HAPs | 1.66x103 | 1.50x103 | 1.77x103 | 1.60x103 | 2.02x103 | 1.83x103 |