Data Review Checklist for Contractors updated Aug 2025

Responsibility of tool outputs lies on contractor who develops tools.

Goal is a complete set of County, SCC, Pollutant emissions that are reasonable in magnitude.

# Overview of Data Checks

| No. | Data Element | QA Check/Method | Provide to EPA |
| --- | --- | --- | --- |
|  | Number of Counties outputted | Completeness | Table of comparison of counties (see Data Check 1 table below) with Analysis Discussion |
|  | Number of SCCs outputted | Completeness | Table of comparison of SCCs (see Data Check 2 table below) with Analysis Discussion |
|  | Emissions on County SCC level | Zeroes | Provide a list of any SCC Pollutant combinations that are zero emissions and explain. |
|  | Emissions Magnitude | Compare on a county level to 2017NEI and 2017EPA | Provide a list of any SCC Pollutant combinations where 20% different and explain, if possible. |
|  | Suite of Pollutants | Completeness when compared to our expected pollutants list. | Provide a list of corrections to expected pollutants list if needed. |
|  | Impossible Sums | Make sure HAP-VOCs don’t exceed total VOCs, make sure PM sums make sense | Correct as needed. |
|  | Check full list of SCCs | Check for retired SCCs, accurate SCCs, new SCCs when compared to Synaptica and NEMOs | Provide list of corrections to EPA as needed. |
|  | Rankings | Check rankings of highest and lowest | Determine if it’s significantly different. |
|  | Verification | Verify that all outputs of tools (SCC, county, pollutant) actually make it into the right dataset in EIS. For final data that includes SLT inputs, this should be 2023EPA_Nonpoint. Default data without any SLT inputs should be in 2023EPA_WWDefault. | A list of what verifications were completed. |
|  | Input Templates | Verify that the NP Survey and the input templates listed as received are consistent, and that the emission comment field also correlates with the notation that these inputs were included. If not, was a template missed? | Provide a list of SLT agencies that inputs were included, and a comparison list to the NP Survey. If no templates were missed, provide the list of inconsistencies so EPA can follow up with those SLT. |
|  | Uniqueness Check | Verify that there is no more than one record for each unique combination of FIPS, SCC, and pollutant code | A table of all such occurrences. |

# Examples.

The following data check examples should be used as template format for contractors when providing QA analyses.

## Data Check 1. Number of Counties provided by EPA Tool

Example table to provide when providing final tool.

| SCCs provided by WW | Number of Counties Generated in 2020 Tool | Number of Counties Generated in 2023 Tool | Analysis Discussion |
| --- | --- | --- | --- |
| 2505040120 | 1698 | 1649 | Fewer counties in 2023 CBP with employees in associated NAICS code |
| 2630020000 | 3224 | 2908 | Removed counties with 0 flow and 0 emissions |
| 2801000003 | 3219 | 3149 | Fewer counties in ag census dataset than in 2020 |
| 2805100010 | 3224 | 3225 | New CT FIPS adds an extra county in 2023 |

## Data Check 2. Number of SCCs provided by EPA Tool.

Provide a complete table, and only provide analysis where different or there’s clearly a problem.

| State and County FIPS | Number of SCCs Generated in 2020 Tool | Number of SCCs Generated in 2023 Tool | Analysis Discussion |
| --- | --- | --- | --- |
| 01001 | 85 | 120 | There are more SCCs in each county because we've added several new SCCs to the Wagon Wheel in 2023 |
| 01003 | 94 | 129 | There are more SCCs in each county because we've added several new SCCs to the Wagon Wheel in 2023 |
| 09110 |  | 125 | New county in CT |

## Data Check 3. Explanation of Zero Emissions Created.

Example where FIPS/SCC/pollutant combination has zero emissions.

| State and County FIPS | SCC | Pollutant | Throughput | Emissions | Analysis Discussion |
| --- | --- | --- | --- | --- | --- |
| 01001 | 2104001000 | Mercury | 0 | 0 | This county did not have activity data, therefore emissions are 0. |
| 05007 | 2401020000 | VOC | 590.8739344 | 0 | This county did have activity data, and an emission factor, but 0 emissions due to default point source emissions subtraction |
| 06075 | 2801000003 | PM25-PRI | 15151.6667 | 0 | This county did have activity data, but the emission factor 0, therefore emissions are 0. |

## Data Check 4. Emissions Magnitude Analysis.

Maps and comparisons should be provided with this datacheck.

Example subset of large magnitude differences across FIPS/tool/pollutants.

| FIPS | Tool | Key Pollutant | 2020 Emissions | 2023 Emissions | % Difference | Analysis Discussion |
| --- | --- | --- | --- | --- | --- | --- |
| 01001 | Cooking, commercial and residential | PM25-PRI | 169.2 | 77.1 | -54.43% | A new methodology for calculating emissions for this category resulted in a large difference in emissions. |
| 26163 | Residential Wood Combustion | PM25-PRI | 558.6736593 | 257.444838 | -53.92% | Methodology changes like the urban rural adjustment resulted in significantly higher emissions in some counties and significantly fewer in others. |
| 01015 | Road Dust | PM25-PRI | 388.3442984 | 62.83135995 | -83.82% | A new methodology for calculating emissions for this category resulted in a large difference in emissions. |

## Data Check 5. Suite of Pollutants.

Example: We checked against the expected pollutants list dated XX/XX/2X and it matches except for <SCC> <pollutant code> which we asked Lindsay to add to the EPL.  For 2023, there are no pollutants in the 2023 results that are not in the expected list.

## Data Check 6. Impossible Sums.

Example: We summed HAP VOC to VOC and found no impossibilities. We summed all PM species and found no discrepancies.

## Data Check 7. Full List of SCCs.

Example: We checked the SCC list in the NEMOs and the Synaptica list and found no unknown SCCs. If any are found, then we will follow up with EPA to add these SCCs to the NEMOS and Synaptica.

## Data Check 8. Rankings.

Example: Summary findings show rankings are consistent with previous inventories for key pollutants.

| State | Tool | Pollutant | 2020 Emissions | 2023 Emissions | 2020 Rank | 2023 Rank | Analysis Discussion |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NJ | Landfills | Mercury | 12.3 | 39.8 | 26 | 16 | LMOP reported 31 more landfills in NJ in 2023 than in 2020, resulting in more emissions and a higher rank than other states. |
| NV | Construction Dust | PM25-PRI | 6303.1675 | 1673.6633 | 5 | 28 | Road construction spending changed significantly in NV, resulting in a change in rank. |
| TX | Residential Wood Combustion | PM25-PRI | 4,495.7 | 9,271.3 | 31 | 16 | SEDS data increased significantly and decreased significantly in others. This resulted in different SEDS adjustment factors, larger differences in emissions, and changes in rank. |

## Data Check 9. Verification into EIS.

Example: There are 25,000 datapoints exported from the WW tool, and 25,000 are now imported into EIS to the dataset 2020EPA_Nonpoint. We’ve reviewed the feedback report and verified that this is the correct number with no critical errors or warnings.

## Data Check 10. All SLT Inputs Included.

Example: We received input template data from ID, WY, and TX for the Cooking tool. We’ve verified that the emission comment field for output includes a note that says input templates were included, and that the emissions are altered accordingly.  AL said they submitted an input template on their NP survey but did submit one to EIS. EPA should follow up with this SLT.

## Data Check 11. Uniqueness Check.

There are no cases where there are multiple records for each unique combination of FIPS, SCC, and pollutant code.