MEMORANDUM

DATE:	October 7, 2008

SUBJECT:	Summary of Steel Minimills Data for Establishing AP-42 Emission Factors

FROM:	Bradley Nelson, EC/R Inc.

TO:		Michael Ciolek, EPA/OAQPS/SPPD/MPG

1.0	INTRODUCTION

This memorandum presents a summary of the current test data available for determining emission factors for Section 12.5.1 of the Compilation of Air Pollutant Emission Factors, Volume I: Stationary Point and Area Sources, Fifth Edition (AP-42).  The current test data includes older test data that were used to determine the previous emission factors in addition to new data collected during the current revisions to Section 12.5.1.

- 2.0	BACKGROUND

- Section 12.5.1 of AP-42 of Chapter 12 was revised and sent out for public comment in 2004.  The main issue that was raised from these public comments was that the published emission factors were based on “C” and “D” rated data, and hence the emission factors were “D” or “E” rated emission factors.  Based on the public comments, EPA contacted numerous State agencies requesting complete test reports in 2007.  During that period, we received 45 complete test reports, which included the appendices.  After review, it was determined that 30 of these test reports could be used to calculate emission factors for steel minimills.  Some of the reports were not used because the facility was not a steel minimill, or the report did not provide any process data that could be used to calculate an emission factor.  The new test data was added to the existing database of emission data, and new emission factors were calculated.

- 3.0	EMISSION FACTOR CHANGES

- The revised emission factors are based on 52 “A” or “B” rated test reports from 17 steel minimill facilities.  The most significant changes are the addition of new test data and the consolidation of the melt shop processes into a single process emission factor.  This was done since many of the processes are performed in the same building and are controlled by the same control device.  Therefore this consolidates many of the process emission factors from the previous version of the AP-42 section into a single emission factor for Electric Arc Furnaces (EAF) and other processes in the melt shop.  The revised emission factors also include emission factors for arsenic, cadmium, chromium, mercury, manganese, and nickel.  A summary of the changes to the emission factors for EAFs are presented in Table 1.

- Table 1.  Summary of Emission Factor Changes

| - Pollutant | - Revised EF Rating | - Revised EF | - Previous EF Rating | - Previous EF |
| --- | --- | --- | --- | --- |
| - Filterable PM | - B | - 0.020 | - B | - 0.083 |
| - Condensable PM – Aqueous | - C | - 0.029 | - D | - 0.073 |
| - Condensable PM - Organic | - C | - 0.010 |  |  |
| - Total PM | - C | - 0.059 | - C | - 0.064 |
| - NOX | - B | - 0.20 | - D | - 0.22 |
| - CO | - B | - 2.0 | - C | - 1.6 |
| - SO2 | - C | - 0.22 | - D | - 0.090 |
| - Lead | - C | - 0.00056 | - E | - 0.00066 |
| - VOC | - C | - 0.023 | - E | - 0.17 |
| - Arsenic | - E | - 6.2E-06 | - N/A | - N/A |
| - Beryllium | - D | - 2.8E-07 | - E | - 7.4E-08 |
| - Cadmium | - E | - 5.0E-06 | - N/A | - N/A |
| - Chromium | - E | - 3.5E-06 | - N/A | - N/A |
| - Mercury | - D | - 1.1E-04 | - N/A | - N/A |
| - Manganese | - E | - 3.0E-04 | - N/A | - N/A |
| - Nickel | - E | - 5.5E-06 | - N/A | - N/A |
| - Fluoride | - D | - 0.059 | - E | - 0.075 |

As the table shows, there were only a few significant changes to the emission factors from the December 20, 2006 published emission factors in comparison to the revised September 2008 emission factors.  The most significant changes were to the filterable and condensable PM emission factors and the VOC emission factors.  These changes are due to the addition of new test data for each of these emission factors.