CC                       Program OBODM

CC     'Dugway Proving Ground Open Burn/Detonation Model'

CC                 'Version 01.3 - 07/24/97'                            072497

CC

CC     OBODM Version 01.3 program by -

CC     Jay R. Bjorklund, James F. Bowers & Greg C. Dodd

CC

CC     H.E. Cramer Co. Inc., P.O. Box 411, Sandy, UT 84091-0411,

CC     (801) 561-4964 or 561-7206

CC                     and

CC     Meteorological & Modeling Division

CC     West Desert Test Center, Dugway, Utah 84022-5000, (801) 831-5101

CC

CC**********************************************************************

CC     Last Update -

CC     03/05/96 - In sub. IDISC.FOR, call to IFNUL.FOR was corrected to

CC                a function reference rather than a call.

CC                In IOSCN.FOR, ibackt was used in a CLRPG.FOR call, but

CC                should have been int*2.  int*2 iback was added and

CC                replaced ibackt in the call.

CC                In SPDSP.FOR, irow and icol were corrected to int*2

CC                type variables.

CC     03/06-96 - In OBODM.FOR, linetl and ifrdwt were double defined.

CC     03/13/96 - In RLABL.FOR seasonal labels were changed from

CC                season-1, season-2, etc. to winter, spring, summer

CC                and fall.

CC     07/09/96 - In OBODM.DOC, under constv, the definitions of ppb    070996

CC                and ppt were corrected.  Also, in subroutine DISPR    070996

CC                the conversion factors to convert ppm to ppb or ppt   070996

CC                were changed from 1.0e-3 to 1.0e3.  Also, corrected   070996

CC                an error in PLUMI and PLUMC.  When distance to final  070996

CC                plume rise is specified, the program uses the first   070996

CC                x,y point to determine the distance to final rise.    070996

CC                The program was erroneously using the distance to the 070996

CC                receptor, when the receptor distance was less than the070996

CC                maximum rise distance.  This affected only the first  070996

CC                x,y point calculated.                                 070996

CC     07/11/96 - Modified IDISC and STATB, so would turn off menu      071196

CC                function BACK if at the start of the files list and/or071196

CC                turn of NEXT if at the end of the files list.         071196

CC                iswopt(32) is now used to control these special cases.071196

CC     11/21/96 - Modified DISPR, was only checking to see if zpdist >  112196

CC                hmdpth if complex terrain was used.  Changed to check 112196

CC                always.                                               112196

CC                Also, when an old input file was read and had an x,y  112196

CC                coordinate data file specified, you could not turn it 112196

CC                off.  Now, if file name field is blank, the file name 112196

CC                is blanked.                                           112196

CC     02/07/97 - Modified command line options and added command line  020797

CC                option to swap y and z receptor axes.  Also, changed  020797

CC                debug (ifdbug) code.                                  020797

CC     03/07/97 - Added description of ifmode to obodm.doc.  Also,      030797

CC                error return code was improved from routine IFSRD.    030797

CC     03/12/97 - Corrected problem in PLTSQ and JFCTC where variable   031297

CC                should be character*2.                                031297

CC     07/11/97 - Corrected a problem where source inputs were not      071197

CC                checked for errors in RINPH.                          071197

CC     07/24/97 - Corrected errors in source data defaults and expanded 072497

CC                the maximum number of sources from 10 to 50.  These   072497

CC                required changes to the data save file contents,      072497

CC                requiring a change from version 1.2 to 1.3.  The      072497

CC                effected subroutines are: OBODM, RINPH, RINPK, RPRTB, 072497

CC                RMDLA, RPRTD, RMDLB, RINPC, RINDB, RPRTC, RINPL,      072497

CC                RINDA, RINPQ, GETID, SAVID, RPLTA.  Also, include     072497

CC                files OCALCS.INC, OCNTRL.INC, ONAMES.INC and          072497

CC                OWNTRL.INC and OBODM.DOC were changed.                072497

CC                moddat=07/24/97, ivrmod=0                             072497

CC     08/24/97 - Corrected code in GETD6 that was checking for input   082497

CC                saved data errors even though no input saved data was 082497

CC                being used.  This resulted in ERR being displayed for 082497

CC                data categories in the main menu when NONE should have082497

CC                been displayed.                                       082497

CC                Also, in GETD2 the first argument to ipack was a      082497

CC                constant and should have been set from the array      082497

CC                nsubsc for the correct number of menus for the data   082497

CC                item.                                                 082497

CC                moddat=08/24/97, ivrmod=1                             082497

CC     02/10/98 - Corrected code in DISPR so that initial source        021098

CC                dimensions (sigxop, sigyop and sigzop) are equal      021098

CC                for buoyant quasi-continuous sources to comply with   021098

CC                the derivation of the plume rise models.              021098

CC                moddat=02/10/98, ivrmod=2                             021098

CC     07/30/98 - Corrected output code that showed emission strength as073098

CC                G/S or per time.  Now shows only G or total units.    073098

CC                moddat=07/30/98, ivrmod=3                             073098

CC     08/12/98 - Corrected problem where program was not checking for  081298

CC                missing date/hours and sequence errors.  Also, added  081298

CC                program date to menu1.  Also, removed the word on from081298

CC                warning messages 16 and 17 (missing date, sequence    081298

CC                error) and changed missing date to llyrs rather than  081298

CC                iniyrs or expected date.                              081298

CC                moddat=08/12/98, ivrmod=4                             081298

CC     09/01/98 - Fixed error that did not recognize the character      090198

CC                abrevations for some units, subroutine LUNIT.         090198

CC                moddat=09/01/98, ivrmod=5                             090198

CC     09/04/98 - Corrected error that allowed iofhrs to be 1 for hour  090498

CC                limits 0100-2400, should have been 0.  Moved update   090498

CC                date to lower right corner of screen in menu 1.  Moved090498

CC                code after ROUTA and ROUTB that check to see if input 090498

CC                data was to be intialized to area near front of code  090498

CC                so after all of routines RMDLA, RMDLB, RMDLC, ROUTA   090498

CC                ROUTB were executed a check would be made.  This was  090898

CC                changed because if an error occurred in other routines090498

CC                data would not be reinitialized and some changed      090498

CC                could be within out to a user .OUT file, changing user090498

CC                data.  Corrected bug where would not append .inp to   090498

CC                batch input control (=) file when no extension was    090498

CC                entered.                                              090498

CC                moddat=09/04/98, ivrmod=6                             090498

CC     11/01/98 - Corrected code that failed to unpack concentration    110198

CC                arrays correctly, depending on data values and array  110198

CC                size.  Corrected parameter ifcalc so -1 indicates     110198

CC                source not operational, but zeros are included in     110198

CC                averages.  Corrected strange problem where the highest110198

CC                and second highest in array vmxhi were not being      110198

CC                printed correctly in OUTPT, removed vmxhi from OUTPT  110198

CC                arguments and used calcsa instead.  vmxhi is          110198

CC                equivalenced to calcsa in ROUTA and ROUTB.  Also,     110198

CC                corrected some minor labeling problems with table     110198

CC                headings, page numbers and table numbers.             110198

CC                moddat=11/01/98, ivrmod=7                             110198

CC     11/10/98 - Corrected code in ISODA that errored the program off  110198

CC                when hourly source data was used.  Corrected code in  110198

CC                GETD1 that read Version 1.0 data incorrectly.         110198

CC                Changed code in ROUTA so that tables are not produced 111098

CC                for user excluded hours and OUTPT prints a message    111098

CC                that all values are zero when all receptors are zero, 111098

CC                rather than printing several pages of zeros.  Also,   111098

CC                corrected several minor labeling problems.            111098

CC                moddat=11/10/98, ivrmod=8                             111098

CC     12/06/98 - Corrected labeling problem where time-average calcs   120698

CC                time units were not labeled correctly when producing  120698

CC                graphics.                                             120698

CC                moddat=12/06/98, ivrmod=9                             120698

CC     12/10/98 - Corrected screen for displaying print files, was not  121098

CC                displaying "OBODM (Version" in the heading and had    121098

CC                Entr in the heading rather than Rtrn.  RINPC and STATB121098

CC                respectively were corrected.  Also, discovered if the 121098

CC                user scaled output using ascale and bscale and a .SOL 121098

CC                file and then went back and specified an input file   121098

CC                and executed the input file data, all in the same run,121098

CC                the ascale and bscale values would scale all printed  121098

CC                results.  Changed this so ascale and bscale are always121098

CC                reset to 1.0 and 0.0, respectively.  Corrected bug    121098

CC                where hour counters were not being reset when running 121098

CC                batch brute force runs under the summary mode.  The   121098

CC                net effect of this error was the annual average for   121098

CC                the second batch run would be 1/2 what it should, the 121098

CC                third batch run would be 1/3 what it should, etc.     121098

CC                moddat=12/10/98, ivrmod=10                            121098

CC     12/21/98 - Corrected problem where the first 9 lines of date/time122198

CC                output in the highest and second highest tables were  122198

CC                printed as all zeros (routines ROUTA, ROUTB and OUTPT)122198

CC                moddat=12/21/98, ivrmod=11                            122198

CC     01/25/99 - Corrected problem where when sources were deleted or  012599

CC                duplicated (copied), some of the source parameters    012599

CC                were not copied correctly.  This was caused by        012599

CC                incorrect dimensioning of equivalenced source arrays  012599

CC                in subroutine RINPG.  This would affect any run where 012599

CC                any source except the last was deleted and any run    012599

CC                where any source was copied from another.             012599

CC                moddat=01/25/99, ivrmod=12                            012599

CC     03/04/99 - Corrected problem where results for batch runs using  030499

CC                hourly meteorological data did not compare to results 030499

CC                when run outside of a batch run.  Program was not     030499

CC                opening a new met. input data file or a new source    030499

CC                input data file if the previous batch file had already030499

CC                opened either file type.  Program listed name of met. 030499

CC                and source file, but did not open it because it       030499

CC                assumed it had already been opened.                   030499

CC                moddat=03/04/99, ivrmod=13                            030499

CC     01/04/00 - Added code to RMDLA to help identify data format      010400

CC                errors.  The program was allowing the friction        010400

CC                velocity and Monin-Obukhov length read from ISC3      010400

CC                dry deposition format data to be read as the wind     010400

CC                profile exponent and vertical potential temperature   010400

CC                gradient, respectively.  If the program was directed  010400

CC                to read ISC3 CARD format data, but the data were      010400

CC                ISC3 dry deposition format, this would cause the      010400

CC                program to calculate an incorrect wind speed and      010400

CC                incorrect plume rise, resulting in erroneous          010400

CC                concentration or deposition calculations.  The OBODM  010400

CC                program does not process ISC3 dry or wet deposition   010400

CC                format data.                                          010400

CC                Also, added function LEAPS to expand leap year        010400

CC                detection.  There was no problem in this area, the    010400

CC                code was expanded for completeness.                   010400

CC                Also, corrected bug in RINPQ where the program was not010400

CC                detecting an overflow of calculation points and       010400

CC                calculation quantities when operated in the summary   010400

CC                mode.                                                 010400

CC                moddat=01/04/00, ivrmod=14                            010400

CC     01/07/00 - Continued with related areas for updates under 010400 010700

CC                above.  Also, added more information to the help      010700

CC                menus.  Increased screens from 78 to 81.              010700

CC                moddat=01/07/99, ivrmod=15                            010700

CC     02/28/00 - Under certain model conditions time-average concen-   022800

CC                trations exceeded maximum peak concentrations.  This  022800

CC                has changed to not allow time-average concentration   022800

CC                to exceed maximum peak concentration.  Also, corrected022800

CC                a problem where time-average concentration was being  022800

CC                under-estimated for quasi-continuous sources when     022800

CC                dosage was also being calculated.                     022800

CC                moddat=02/28/00, ivrmod=16                            022800

CC     07/17/00 - When input rectangular discrete coordinates were input071700

CC                in feet, the y coordinate was not being converted to  071700

CC                meters, resulting in incorrect results. Sub=RPRTD.FOR 071700

CC                moddat=07/17/00, ivrmod=17                            071700

CC     09/05/00 - Added option (ISWOPT(33)) to turn off printing of calm090500

CC                wind warning messages.                                090500

CC                moddat=09/05/00, ivrmod=17a                           090500

CC     05/18/01 - Corrected error where volume units of liters were     051801

CC                being calculated incorrectly.  The prog was using     051801

CC                1.0e3 as a conversion factor, but should have been    051801

CC                using 1.0e-3.                                         051801

CC                moddat=05/18/01, ivrmod=18                            051801

CC     12/07/02 - Modified program so user can now enter a flagpole     120702

CC                height for each grid receptor.  Previously you could  120702

CC                only specify one flagpole height for the entire grid  120702

CC                system.                                               120702

CC     01/14/03 - Corrected problem where program would not allow annual011403

CC                or seasonal calculations under certain conditions     011403

CC                associated with flagpole receptors or misstaking      011403

CC                flagpole receptors for complex terrain.               011403

CC                Also, corrected problem where date/time was printed   011403

CC                incorrectly for highest and second-highest tables     011403

CC                when a large grid was used and multiple quantities    011403

CC                (dos., con., etc.) were calculated.  Also, added      011403

CC                counters to processing display that indicate number of011403

CC                calms, missing data and data errors.                  011403

CC                moddat=01/14/03, ivrmod=19 (Combines mods from 12/7/02011403

CC     05/15/03 - Added two options. You can now specify if the wind    051503

CC                direction in user specified met. file formats is from 051503

CC                or towards (vector) when using hourly data.  Single   051503

CC                hour runs are always direction from.  Preprocessed    051503

CC                ISCST3 data are always towards (vector).  You can now 051503

CC                turn off printing of met. data error and warning      051503

CC                messages.  Routines changed are obodm.for, rindb.for  051503

CC                rinpq.for, rmdla.for, rinpm.for, rprtc.for            051503

CC                moddat=05/15/03, ivrmod=20                            051503

CC     09/29/04 - Corrected error where met. data limits were not       092904

CC                allowed to change for wind speed, direction, power law092904

CC                and turbulent intensities.                            092904

CC                moddat=09/29/04, ivrmod=21                            092904

CC     02/14/05 - Corrected problem in RMDLA, ISODA, and RPLTF where the021405

CC                field width in calls to IFSRD was too small for actual021405

CC                data fields being0 read and was changed from 10 to 30.021405

CC                if the actual field width was greater than or equal to021405

CC                to 10 and the field contained a single trailing zero  021405

CC                with all other characters in the field blank and this 021405

CC                was the last data field in the record IFSRD returned  021405

CC                the value as missing (-1.0E20) rather than zero.      021405

CC                However, in calculations the missing value was treated021405

CC                as zero.  The field width of 30 will prevent this from021405

CC                reoccurring.                                          021405

CC                moddat=02/14/05, ivrmod=22                            021405

CC     04/18/06 - Corrected problem in RPLTF where it was not allowing  041806

CC                negative coordinates and was not converting x,y units 041806

CC                correctly when plotting special map features.         041806

CC                Also, units conversion in area and volume have been   041806

CC                correctedin OBODM and RPLTD                           041806

CC                moddat=04/18/06, ivrmod=23                            041806

CC     06/06/07 - Corrected problem in RINPK, RINDB where the code was  060607

CC                checking array IVARSO for values, but was checking by 060607

CC                summing (2) through (13), but should have been        060607

CC                checking only (2) through (4). This caused hourly     060607

CC                input source data to show NONE for source data in the 060607

CC                main menu.  Also, source greater than mixing depth was060607

CC                causing a user reponse to the warning on each         060607

CC                occurrance, so changed to no user response.           060607              060607

CC                moddat=06/06/07, ivrmod=24                            060607

CC

CC

CC**********************************************************************

CC

CC     This program is designed to calculate -

CC        1) Dosage

CC        2) Concentration

CC        3) Time-average concentration

CC        4) Gravitational deposition

CC     Downwind of multiple -

CC        1) Instantaneous

CC        2) Quasi-continuous square wave

CC     Sources -

CC        1) Volume/point

CC        2) Line

CC     Program capabilities include -

CC        1) The ability to model multiple sources.

CC        2) The ability to model multiple types of sources.

CC        3) The ability to use either a rectangular or polar receptor

CC           grid system.

CC        4) The ability to specify source locations in either rectang-

CC           ular or polar coordinates.

CC        5) The ability to specify input and output data files in

CC           standard or user specified formats.

CC        7) The ability to provide multiple graphics displays in

CC           addition to printed output of calculations.

CC        8) The ability to use complex terrain.

CC

CC     All calculations in this program are relative to a rectangular

CC     coordinate system where 0 degrees north is the positive y axis

CC     and 90 degrees east is the positive x axis. All sources

CC     and calculation receptor points are relative to the origin

CC     of this coordinate system. All vertical measures referenced

CC     in this program are relative to the ground (zero).

CC

CC**********************************************************************

CC

CC             - Program Language and Application -

CC

CC     This program is written in FORTRAN 77 and C and is designed for use

CC     on IBM compatible PC computer systems.

CC

CC     Program assumptions -

CC        1) The numerical result of real number operations is

CC           truncated.

CC        2) Underflows do not cause an error trap, but result

CC           in zero.

CC        3) Integers are size integer*4.

CC        4) Double precision are greater than or equal to

CC           size real*8.

CC        5) All character type parameters begin with the letter

CC           m (except the single letter m or double mm which

CC           are used as integer indices).

CC           All integer type parameters begin with i, j, k, l or

CC           n and include a single m or double mm.

CC           All real or double type parameters begin with the

CC           letters a through h or o through z.

CC        6) All common variable names are six letters long. all local

CC           variable names are less than six letters long.

CC        7) Variable names that are all caps are global names.

CC        8) All common variables are contained in common blocks

CC           ONAMES.INC, OCNTRL.INC, OCDSPM.INC, OCALCS.INC, and

CC           OWNTRL.INC.  All common blocks are compiled via the

CC           INCLUDE statement.

CC

CC     Sections of coding within this program that are computer,

CC     compiler or graphics dependent begin and end with special

CC     sentinels in the first few characters of the line image.

CC     These sentinels are -

CC

CC     -Start           -End            -Application

CC      -----           -----           ------------------------------

CC      CCOMP1S         CCOMP1E         Hewlett Packard HP9000/800.

CC      CCOMP1S-HP2645A CCOMP1E-HP2645A HP2645A & 700/92 terminals.

CC      CCPCITS         CCPCITE         IBM compatible PC.

CC      CC-DEBUGS       CC-DEBUGE       debug only.

CC      CC>                             code not presently used

CC

CC      Variables within this program that are computer or graphics

CC      dependent are - 'isystm' in common block 'OCNTRL.INC',

CC      see BLOCK DATA routine below for current values.

CC

CC**********************************************************************

CC

CC                - OBODM Programs and Subroutines -

CC

CC     The main program is OBODM. The primary routines are - OBODM

CC     GMENU, RINPA, RINPB, RINDA, RINDB, RINPC, RINPD, RINPE, RINPF,

CC     RINPG, RINPH, RINPI, RINPJ, RINPK, RINPL, RINPM, RINPN, RINPO,

CC     RINPP, RINPQ, RINPR, RPRTA, RPRTB, RPRTC, RPRTD, RMDLA, RMDLB,

CC     RMDLC, ROUTA, ROUTB, RPLTA, RPLTB, RPLTC, RPLTD, RPLTE, RPLTF,

CC     RPLTG, RPLTH, RPLTI, RPLTJ

CC     These primary routines are designed to be segments, overlays or

CC     subroutines depending on the limitations of the computer where

CC     used.

CC     The secondary routines are all subroutines referenced by the

CC     primary routines and are -

CC       Screen/menu related:

CC     BLOCK DATA, DTSET, FNAME, GETD1, GETD2, GETD3, GETD4, GETD5,

CC     GETD6, IDISC, IDSPL, IFRSP, IFULS, IHLPS, IPACK, IRDSC, IWHER,

CC     IWTSC, JWHER, LUNIT, STODT, UNITS, IOSCN, OBSTR, OBEND, IWRSC,

CC     SPDSP, IBELL, IFORM, NFORM,

CC     (PC: OBSIO, IOCRT, INCRT, JFCTL, KFCTL, SHDOW, FILES, IFFLD,

CC          QSMOD, QRMOD, QVIDB, QLINE, QCLER, QSCLR, QUSCL, QCOLR,

CC          QPREG, QOVSC, QINKY, QRTXT, QSTXT, CLRPG, DISPG, QCMOV,

CC          SETMS, RESMS, CLSMS, RDALL, POPBX, INBAR, STATB, PRBAR,

CC          CRTYP, LFCTL, FRAME, IFCTC, JFCTC, KFCTC, IRSBK, GTBRK,

CC          STBRK, WRBUF, JFJMP, KNCRT

CC          SETDM.C, GETDM.C, GETBD.C, SETPX.C, CLRSC.C, SCRUP.C,

CC          SCRDN.C, MPCGA.C, MPVGA.C, SETOV.C, RDKEY.C, RDCHR.C,

CC          WRCHR.C, CURSP.C, SETPG.C, GETFL.C, ISMOS.C, SHOWM.C,

CC          HIDEM.C, GETML.C, SETML.C, GETCL.C, WAITM.C, RDSTA.C,

CC          CSCAN.C, GTRGC.C, WRKEY.C

CC     (PC OBJ: IOERROR, SEGREAD, FINDFIRS, INT86, SOUND, GETCBRK

CC     (HP: OBSIO.C, FILES.C)

CC       Dispersion related:

CC     ARTAN, CONCD, CONCE, CRSSA, CRSSE, DECAA, DELTD, DELTU, DISPR,

CC     DISPS, ERFXF, ERFXS, FSEXP, FGAMA, GRVDP, PLUMC, PLUMI, PWLAW,

CC     SGXYZ, TZ1Z2, VERT1, VERT2, VERT3, VRXYZ, YLATR

CC       Meteorology related:

CC     AIRDN, AIRVS, DFMET, IPSQL, IRNDX, ISODA, IUSDA, JULAN, NETRX,

CC     NMXDY, PRSVP, SUNAN, WSBAR, UVTWD,

CC       Particle related:

CC     DDST1, DRPAV, TRMVS,

CC       Utility related:

CC     ACCUM, ICNTR, IFALF, IFCHR, IFCMP, IFEQU, IFNBR, IFNUM, IFSRD,

CC     IGTDT, INONE, IOPUT, IPKBK, IRDBF, ISUMI, ISVDT, IVGET, ADEXT,

CC     IVSTO, IWTBF, IZGET, JINIT, JPACK, JRNIT, KERRS, MBLK1, KERRP,

CC     MVCH1, NUMC1, NUMLB, ORDER, OUTPT, RLABL, LRJST, ANTRU, CMXMN,

CC     IANYI, JANYI, IFNUL, ISEAS, IDATR, IFFOP, SAVID, GETID, LOCAL,

CC     IFWER, INRAN, IORAN, INCPT, IOCPT, INITD, JDSKR, JACUM, JHAVG,

CC     JDSKW, CHXYZ, IFCHS

CC       Graphics related:

CC     BKGDQ, CHCKQ, CLRCQ, CMXNQ, CNRXQ, CURVQ, DIFNQ, DMXMQ, FLWKQ,

CC     HD01Q, HD02Q, HD03Q, HD04Q, ICLRQ, IDOLQ, IFBKQ, IFBXQ, IFOTQ,

CC     ILXEQ, ITRPQ, JCLRQ, LABLQ, LBLPQ, LSCLQ, NBXPQ, PLT1Q, PLT2Q,

CC     PLT3Q, PLTSQ, PMAPQ, PNTSQ, SCRLQ, SPCLQ, SPLNQ, STRPQ, SWCHQ,

CC     SYMBQ, XINTQ, FILLQ, LBITQ, IFANQ, IFSPQ, GLABL, PLABL

CC

CC**********************************************************************

CC

CC                  - OBODM Common Variables -

CC

CC     Note: units shown with parameter names are model calculation

CC           units and are not necessarily input or print units.

CC

CC     airden - air density (g/cm**3)

CC     airhum - air humidity (%).

CC     airmol - molecular weight of air (g/mol)

CC     airprs - air pressure (mb).

CC     airtmp - average air temperature (deg c).

CC     airvsc - air viscosity (poise or g/cm.sec).

CC     alatrl - lateral term of the concentration equation.

CC     alongw - alongwind term of the concentration equation.

CC     alphad - model crosswind cloud dispersion coefficient.

CC     alphas - array of source crosswind dispersion coefficients.

CC     ascale - scale factor multiplier for dispersion model

CC              calculations under the reprint option. ascale*dispersion+

CC              bscale.

CC     betadf - model vertical cloud dispersion coefficient.

CC     betale - ratio of lagrangian to eulerian time-scales used in the

CC              correction factor on sigmeq and sigatr for crossing

CC              trajectory effects of heavy particles (default

CC              =0.0 or no correction). the suggested value for betale,

CC              if  used is 1.0.

CC              if betale = 0.0 or vspart/(sigmeq*wndspd) <= .2386 the

CC                       correction is not applied to sigmep or sigmap.

CC              if betale > 0.0 and vspart/(sigmeq*wndspd) > .2386 the

CC                       correction factor is applied to sigmeq and

CC                       sigatr.

CC

CC            sigepr=sigmeq/(1.0+(betale*vspart/(wndspd*sigmeq))**2)

CC                   **.25

CC            sigapr=sigatr/(1.0+4.0*(betale*vspart/(wndspd*sigmeq))**2)

CC                   **.25

CC              where:

CC                  sigmeq = modified input value

CC                  sigatr = modified input value

CC                  vspart = input particle settling velocity

CC                  wndspd = transport speed

CC     betass - array of source vertical dispersion coefficients.

CC     botmrg - bottom margin of hard copy plot (inch).

CC     brnrat - array containing the fuel burn rate (g/s)

CC     bscale - scale factor added to dispersion model calculations

CC              under the reprint option. ascale*dispersion+bscale.

CC     burndf - default fuel burn rate (g/s).

CC     calcsa - scratch array containing all dispersion calculations.

CC     calhgt - array of release heights used in dispersion, same       072497

CC              as relhgt unless relhgt defaulted.                      072497

CC     cldang - angle from 0 degrees north to long side of initial

CC              source dimension cldlng (deg).

CC     cldcov - cloud cover (8ths of celestial dome)

CC     clddpt - source initial vertical depth or diameter (m).

CC     clddst - distance to maximum plume rise (m).

CC     clddxe - array of end of line source x coordinates (m).

CC     clddxs - array of source x coordinates or start of line source

CC              x coordinates (m).

CC     clddye - array of end of line source y coordinates (m or deg).

CC     clddys - array of source y coordinates or start of line source

CC              y coordinates (m or deg).

CC     clddze - array of end of line source z elevations (m).

CC     clddzs - array of source z elevations or start of line source

CC              z elevations (m).

CC     cldfct - fraction of source strength contributing to dispersion

CC              when part of cloud exceeds mixing layer height.

CC     cldhgt - cloud ceiling (m).

CC     cldlng - source initial length of long side (m).

CC     cldmax - array containing the maximum plume rise calculated for

CC              each source (m).

CC     cldmxd - array containing the distance to maximum plume rise for

CC              each source (m).

CC     cldris - plume rise height increment (m).

CC     cldsht - source initial length of short side (m).

CC     clearx - clearing index read from input hourly meteorological

CC              data file or defaulted via hm(ft)*spd(kts)/100.0.

CC     clrinx - threshold clearing index.  no calculations take place

CC              if clearx is less than or equal to clrinx.

CC     consta - array of constants to convert square meters to output

CC              area units -

CC              square - meters, kilometers, foot, yards, miles,

CC              centimeters, millimeters, micrometers, inches, acres,

CC              hectares.

CC              area(units) = consta(i) * area(square meters)

CC     constc - array of constants to convert temperature to degrees c.

CC              Celsius, Kelvin, Fahrenheit, Rankine.

CC              temperature(Celsius) = constc(i,2) * (temperature -

CC                                     constc(i,1))

CC     constg - array of constants to convert angle to degrees -

CC              degrees, radians.

CC              angle(degrees) = constg(i) * angle

CC     consth - array of constants to convert heat to Joules -

CC              calorie, kilocalorie, British thermal unit, Joule, erg.

CC              heat(Joules) = consth(i) * heat

CC     constl - array of constants to convert length units to meters.

CC              meters, kilometers, feet, yards, miles, centimeters,

CC              millimeter, micrometers, inches + knots for speed to

CC              meters/sec.

CC              length(meters) = constl(i) * length

CC     constm - array of constants to convert mass to grams.

CC              gram, kilogram, pound, ounce, ton, milligram, microgram,

CC              particles.

CC              mass(grams) = constm(i) * mass

CC     constp - array of constants to convert pressure to millibars.

CC              millibar, pascal, inches mercury, millimeters mercury.

CC              pressure(millibars) = constp(i) * pressure

CC     constt - array of constants to convert time to seconds.

CC              seconds, minutes, hours.

CC              time(seconds) = constt(i) * time

CC     constv - array of constants to convert cubic meters to output

CC              volume units -

CC              cubic - meters, kilometers , foot, yards, miles,

CC              centimeters, millimeters, micrometers, inches and

CC              liters, ppm, ppb, ppt.

CC              volume(units) = constv(i) * volume(cubic meters)

CC              for ppm constv(i) = (1.0e3*22.4*1013.25/273.15) or

CC              ppb = (1.0e6*22.4*1013.25/273.15) or                    070996

CC              ppt = (1.0e9*22.4*1013.25/273.15) multiplied by -       070996

CC              (temperature(k)/(pressure(mb)*molecular wt.)

CC     contrs - array of contour levels for surface 1 and surface 2.

CC     convrt - array of units conversion constants for met. input.

CC     cscale - scale factor multiplier for dispersion model

CC              calculations under plotting. cscale*dispersion+dscale.

CC     csmaxs - array containing current maximum value of surface 1

CC              and surface 2.

CC     csmins - array containing current minimum value of surface 1

CC              and surface 2.

CC     dcutof - minimum possible particle diameter (micrometers) below

CC              which is assumed gaseous.

CC     decays - coefficient of time dependent exponential decay for

CC              the removal of material due to chemical or physical

CC              processes (/s).

CC     dekayd - decay depletion term of the concentration equation.

CC     delthp - wind direction shear over the transport layer (radians).

CC     densty - density of the pollutant/species (g/cm**3).

CC     dhincs - saved height increment between line source sources.     110198

CC     dhssss - height of current volume source in line source calc.    110198

CC     dirshr - rate of change of wind direction with height in the

CC              surface layer (degrees/meter). dirshr is positive to the

CC              right of wnddir and negative to the left when looking

CC              downwind along wnddir.

CC     dphidz - vertical gradient of virtual potential temperature

CC              (deg k/m).

CC     dqincs - saved emission increment between line source sources.   110198

CC     drplwr - array of input lower limits of the particle diameters

CC              (micrometers).

CC     drpmmd - particle mass-median diameter (micrometers).

CC     drpsig - geometric standard deviation of the particle

CC              distribution (micrometers).

CC     drpupr - array of input upper limits of the particle diameters

CC              (micrometers).

CC     dscale - scale factor added to dispersion model calculations

CC              under plotting. cscale*dispersion+dscale.

CC     dtorad - conversion from degrees to radians (radians/degrees).

CC     dxincs - saved x coordinate increment between line source sources110198

CC     dxssss - x coord of current volume source in line source calc.   110198

CC     dyincs - saved y coordinate increment between line source sources110198

CC     dyssss - y coord of current volume source in line source calc.   110198

CC     dzincs - saved z coordinate increment between line source sources110198

CC     dzssss - z coord of current volume source in line source calc.   110198

CC     factrs - factor used to modify sigatr to sigapr.

CC     functx - array containing the x points used to plot y as a

CC              function of x (m).

CC     functy - array containing the y points used to plot y as a

CC              function of x.

CC     gammac - air entrainment coefficient for quasi-continuous plume

CC              rise (0.60).

CC     gammad - array of particle reflection coefficients for

CC              each size category.

CC     gammai - air entrainment coefficient for instantaneous plume

CC              rise (0.64).

CC     gammat - array of source air entrainment coefficients.

CC     geolat - latitude in degrees of location being modeled. north

CC              latitude is positive.

CC     geolon - longitude in degrees of location being modeled. west

CC              longitude is positive.

CC     gravty - acceleration of gravity (9.80665 m/s**2).

CC     grdang - calculation grid system orientation angle.

CC              if the positive y axis of the rectangular grid receptor

CC              system points to the right (east) of zero degrees north

CC              when viewed from the south, the angle is positive. if

CC              it points to north the angle is zero.

CC     gxendp - end x coordinate for automatic grid generation.

CC     gxincr - increment in x for automatic grid generation.

CC     gxstrt - start x coordinate for automatic grid generation.

CC     gyendp - end y coordinate for automatic grid generation.

CC     gyincr - increment in y for automatic grid generation.

CC     gystrt - start y coordinate for automatic grid generation.

CC     hcminc - minimum character height possible due to device

CC              resolution (in).

CC     heatdf - default heat content of the fuel (cal/g).

CC     hetcnt - array containing the fuel heat content (cal/g).

CC     hgtmax - maximum source height (m).

CC     hgtmet - elevation of ground at meteorological data measurement

CC              location (m).

CC     hightl - effective source emission height (m).

CC     hmdpth - mixing layer depth (m).

CC     htlchr - height of axes labels and legend characters in inches.

CC     htnchr - height of axis coordinate label characters in inches.

CC     htpchr - height of special characters in inches.

CC     ialpnt - array containing the first and last angle of slope of

CC              each contour segment for the placement of contour

CC              labels.

CC     ibackd - menu data field background color index.

CC     ibackt - menu text background color index.

CC     ibaudr - baud rate. default = 1200.

CC     iboard - PC video card index. Type of text/graphics board.

CC              same as isvbrd.

CC              1 - CGA card.

CC              2 - EGA card.

CC              3 - VGA card.

CC     ibpage - display page number.

CC     iceunt - ceiling height (cldhgt) units index, length units.

CC     ichars - index of special character to use for plotting points,

CC              see subroutine SPCLQ for indices.

CC     icnclr - array containing the color index of each contour line

CC              see icolor for color indices for surface 1 and surface 2

CC     icndsh - array containing the index of the type of contour line

CC              to draw, solid, dashed, etc. see ifdash for surface 1

CC              and surface 2.

CC     icmbeg - array with column offsets for each command

CC     icmcol - start column number for command bar

CC     icmkey - array containing ascii numbers associated with 'alt' keys

CC     icmlen - array with length of labels for each command field

CC     icmmap - array that maps command bar commands

CC     icmnum - number of commands in command bar

CC     icmrow - row number for command bar

CC     icmsta - array containing current status of commands

CC     icolmx - maximum columns on crt screen.

CC     icolor - current plot color index.  hue     brightness saturation

CC               0 - black                  0           0          0

CC               1 - blue                   0          57        100

CC               2 - green                240          42        100

CC               3 - cyan                 300          71        100

CC               4 - red                   90          57        100

CC               5 - magenta               60          71        100

CC               6 - yellow               180          57        100

CC               7 - dark white             0          66          0

CC               8 - gray                   0          33          0

CC               9 - light blue            20          71        100

CC              10 - light green          260          71        100

CC              11 - light cyan           330          57        100

CC              12 - light red            120          42        100

CC              13 - light magenta         60          85        100

CC              14 - light yellow         180          85        100

CC              15 - light white            0         100          0

CC     icontr - contour index

CC              1 - surface 1.

CC              2 - surface 2.

CC     icpont - pointer used to control calculation path between        110198

CC              subroutines RMDLB and RMDLC.                            110198

CC     ibcfil - logical unit number of batch processing file.

CC     ibcrec - record counter for batch processing file.

CC     idbfil - logical unit number of database screen menu template

CC              file.

CC     idcunt - time units index of pollutant/species half-life.

CC     idpage - display page number.

CC     idrect - array containing the source dimension length units

CC              indices. a) length units of long and short side of

CC              source in 1's position b) length units of source

CC              vertical dimension in 10's position.

CC     idsunt - wind direction shear angle units.

CC     idubrn - default mass units of fuel burn rate.

CC     iduhet - default heat units of fuel heat content.

CC     idunit - mass units of pollutant/species density.

CC     ierfil - logical unit number of error data file OBODMERR.OBD.

CC     ifanul - print annual tables.

CC              0 - no.

CC              1 - yes.

CC     ifavrg - averaging time/type for plotting.

CC              1 - single N-hour average event.

CC              2 - seasonal average.

CC              3 - quarterly average.

CC              4 - annual average.

CC              5 - highest N-hour average.

CC              6 - second highest N-hour average.

CC     ifbndx - plotting out of bounds flag (subroutine PLT2Q only)

CC              see xbndxs and ybndxs.

CC              0 - point out of plot boundary

CC              1 - point inside of plot boundary

CC              2 or 3 - curve crossed over plot boundary

CC                       2 - curve going out.

CC                       3 - curve coming in.

CC     ifbtch - batch process flag.

CC              0 - no batch processing.

CC              1 - program processing batch data files.

CC     ifcalc - calculate dispersion/deposition flag (not used for

CC              single met. obs).

CC              0 - no calculations occur, dispersion and deposition are

CC                  zero for that hour, because the wind speed is calm

CC                  or data are missing.  these hours are subtracted

CC                  from the total hours according to EPA's Guideline

CC                  on Air Quality Models, 1990, Section 9.3.4.2. if

CC                  the number of hours accumulated for the averaging

CC                  period is less than 75% of the total possible hours

CC                  in the period, then 75% of the total possible hours

CC                  is used.

CC              1 - yes calc, all data are good or run is single met.

CC                  obs., all hours are included in averaging.

CC             -1 - no calculations occur, dispersion and deposition are

CC                  zero, because the event could not occur due to a

CC                  clearing index less than or equal to the threshold,

CC                  a non-daytime hour, or the day was excluded from

CC                  calculations.  all hours are included in averages,

CC     ifccok - flag used to indicate if CRTL-C is ok.

CC             -1 - kills program imediately.

CC              0 - no, cannot interupt.

CC              1 - yes, input menus only.

CC         2 or 3 - general processing, plotting only.

CC     ifcolr - plot hard copy in color.

CC              0 - no.

CC              1 - yes.

CC     ifchlb - flag to indicate whether or not to change the default

CC              plot legend.

CC              0 - no.

CC              1 - yes.

CC     ifcnvt - was grid system, etc. converted to output units.

CC              0 - not converted.

CC              1 - converted to output units.

CC     ifctlc - flag used to indicate CRTL-C was typed.

CC              0 - no

CC              1 - yes, in input menu.

CC              2 - yes, in general processing.

CC     ifdash - solid, dash, dotted line flag.

CC               0 - curve plotted as solid line

CC               1 - curve plotted as dashed line

CC               2 - curve plotted as dotted line

CC               3 - curve plotted as dash-dot line

CC               4 - curve plotted as dash-2 dots line

CC               5 - curve plotted as dash-3 dots line

CC               6 - curve plotted as dash-4 dots line

CC               7 - curve plotted as dash-5 dots line

CC               8 - curve plotted as dash-6 dots line

CC               9 - curve plotted as dash-7 dots line

CC              10 - curve plotted as dash-8 dots line

CC              11 - curve plotted as dash-9 dots line

CC              12 - curve plotted as dash-10 dots line

CC              13 - curve plotted as dash-11 dots line

CC              14 - curve plotted as dash-12 dots line

CC              15 - curve plotted as dash-13 dots line

CC     ifdbug - print flag for debug print output or special options.   020797

CC              1  - /a  See below for options /a through /z.           020797

CC              2  - /b                                                 020797

CC              3  - /c                                                 020797

CC              4  - /d                                                 020797

CC              5  - /e                                                 020797

CC              6  - /f                                                 020797

CC              26 - /z                                                 020797

CC              Enter a command line argument /a, /b, /c, /d, /e or /f. 020797

CC              /a - Prints the minimum amount of debug information,    020797

CC              /f - Prints the maximum amount of debug information.    020797

CC              Also, special processing options are set via this       020797

CC              command line argument.                                  020797

CC              /m - Generate a new menu database file from files       020797

CC                   SCREEN01, SCREEN02, etc.                           020797

CC              /z - Swap Y and Z input grid system points.  Assumes the020797

CC                   Y axis coordinate points specified are to be used  020797

CC                   as Z heights and the single Z value specified is to020797

CC                   be used as Y.  Also, assumes the wind direction is 020797

CC                   specified as 270 degrees.  This allows for the     020797

CC                   calculation of concentrations in the X,Z plain at a020797

CC                   lateral distance of Y from the maximum centerline. 020797

CC                   If Y is zero, maximum centerline values are        020797

CC                   calculated.  Complex terrain cannot be used with   020797

CC                   this option.  If the input data or solution data   020797

CC                   are saved, this command line argument must be used 020297

CC                   in subsequent runs, because it is not saved with   020297

CC                   the data or solution.                              020297

CC     ifdfil - logical unit number of input fuels/explosives default

CC              data file.

CC     ifdstm - standard versus daylight savings time.

CC              0 - standard

CC             -1 - daylight savings

CC     iferrm - array indicating if data in each data field of the

CC              current menu is erroneous or not. if iferrm(k,l) =

CC              0 - no error.

CC              1 - erroneous, highlight this field.

CC     iferrs - i/o and/or program status flag.

CC               0 - enter key typed, check cursor position for action

CC                   if any.

CC               1 - terminate program.

CC               2 - f2 key typed, return to main menu.

CC               3 - f3 key typed, return to base or current menu.

CC               4 - f4 key typed, display program help menus.

CC               5 - f5 key typed, display units abbrev. menus.

CC               6 - f6 key typed, display fuel/explosive and pollutant/

CC                   species menus.

CC               7 - f7 key typed, display current disc files.

CC               8 - f8 key typed, n/a.

CC               9 - f9 key typed, n/a.

CC              10 - f10 key typed, n/a.

CC              11 - pgup key typed, go back to previous menu.

CC              12 - pgdn key typed, go to next menu.

CC     ifgend - generate particle distribution from drpmmd and drpsig.

CC              0 - no

CC              1 - yes

CC     ifgeng - generate grid system flag.

CC              <=0 - no.

CC                1 - yes.

CC     ifgpol - rectangular versus polar grid system flag

CC              0 - rectangular

CC              1 - polar

CC     ifgpwn - type of material index.

CC              0 - gaseous

CC              1 - particulate

CC     ifgrph - graphics flag.

CC              0 - no graphics desired.

CC              1 - generate graphics.

CC              2 - same as 1, but specify all plot details.

CC     ifhcpy - hard copy flag.

CC              0 - no

CC              1 - HP LaserJet xx.

CC              2 - HP RuggedWriter 480.

CC              3 - IBM Proprinter.

CC              4 - Epson FX.

CC              5 - Epson LQ.

CC              6 - Alps P2400C.

CC              7 - IBM XL24 AGM.

CC              8 - Okidata.

CC              9 - Canon BJ200e.

CC             10 - HP7475A bed plotter.

CC             11 - DEC Inkjet.

CC             12 - plot file (logical unit iplfil).

CC     ifhiof - flag used to turn on or off highlighting for the

CC              characters [ and ].

CC              0 - highlight on.

CC              1 - highlight off.

CC     ifhrly - single meteorological observation or continuous hourly

CC              meteorological data.

CC              0 - single obs.

CC              1 - hourly.

CC     ifinit - initialize program data from old work file.

CC              0 - no.

CC              1 - yes.

CC     iflend - end disc record for fuels/explosives and pollutant/

CC              species default data in file mwknam.

CC     iflrjs - flag indicating if menu editing is left or right

CC              justified.

CC              0 - right.

CC              1 - left.

CC     iflstr - start disc record for fuels/explosives and pollutant/

CC              species default data in file mwknam.

CC     ifhsly - source data are single set or some source data varies

CC              with the hourly meteorological data.

CC              0 - source data are single set.

CC              1 - some source data varies hourly.

CC     ifmode - Program calculation and save solution mode.             030797

CC              0 - Detailed - Saves all calculations in the output     030797

CC                  solution file.  The solution file can be read       030797

CC                  and new tables, averages, source combinations,      030797

CC                  plots, etc. can be generated.                       030797

CC              1 - summary, only saves summary calculations in the     030797

CC                  output solution file.  Only the original tables     030797

CC                  can be printed, scaled or plotted.                  030797

CC     ifmous - presence of mouse flag.

CC              0 - no mouse.

CC              1 - yes mouse.

CC     ifmxpc - flag used in limiting maximum stable plume rise

CC              to maximum adiabatic rise for quasi-continuous sources.

CC              0 - determine if max. stable rise will exceed max.

CC                  adiabatic rise.

CC              1 - maximum stable rise is greater than maximum

CC                  adiabatic rise. when stable rise exceeds adiabatic

CC                  rise, the adiabatic rise is used. also, the

CC                  distance to the maximum adiabatic rise is used.

CC              2 - maximum stable rise is less than maximum adiabatic

CC                  rise.

CC     ifmxpi - flag used in limiting maximum stable plume rise

CC              to maximum adiabatic rise for instantaneous sources.

CC              if iswopt(25) is > 0, only stable rise is used for

CC              instantaneous sources.

CC              0 - determine if max. stable rise will exceed max.

CC                  adiabatic rise.

CC              1 - maximum stable rise is greater than maximum

CC                  adiabatic rise. when stable rise exceeds adiabatic

CC                  rise, the adiabatic rise is used. also, the

CC                  distance to the maximum adiabatic rise is used.

CC              2 - maximum stable rise is less than adiabatic rise.

CC     ifnewp - flag indicating type of new plot.

CC              0 - initialize.

CC              1 - plot y as a function of x.

CC              2 - plot isopleths or contours.

CC     ifnext - program logic pointer.  points to major program segments

CC     ifored - menu data field foreground color index.

CC     iforet - menu text foreground color index.

CC     ifpass - flag used to control index of quantity currently being

CC              plotted.

CC               <0 - go back to previous quantity.

CC              >=0 - go to next quantity.

CC     ifprlp - controls the looping over particle categories in model

CC              calculations.

CC               0 - either no particle categories or nvsprt

CC                   > 0 & betale = 0.0.

CC                   if nvsprt > 0 loop over categories in VERT2

CC                   or in VERT3.

CC               1 - nvsprt > 0 & betale > 0.0 the

CC                   loop over categories starts in function DISPR.

CC     ifprno - printer number for graphics hard copies.

CC              1 - printer LPT1.

CC              2 - printer LPT2.

CC              3 - printer LPT3.

CC              4 - plot file OBD#####.PLT

CC              5 - COM port 1

CC              6 - COM port 2

CC             <0 - if abs value -1 to -10, then abs value = LPT

CC                  number.  if abs value -11 to -20, then value =

CC                  COM number.

CC     ifpsol - reprint solution file flag.

CC                   0 - not done or not equal to 2 or 6.

CC              2 or 6 - done.

CC     ifrddm - flag to read hourly meteorological data record or not.

CC              0 = no.

CC              1 = yes.

CC              also, used for starting hour index in plotting.

CC     ifrdwt - array set to indicate which of the control/option,

CC              receptor, source, meteorological and solution/graphics

CC              data sets are initialized or not.

CC              -2 - warning, fatal.

CC              -1 - error, fatal

CC               0 - not initialized or incomplete.

CC               1 - no warnings, no errors, data are complete.

CC               2 - warning, data are complete.

CC              ifrdwt(1) - options/control data.

CC              ifrdwt(2) - receptor data.

CC              ifrdwt(3) - source data.

CC              ifrdwt(4) - meteorological data.

CC              ifrdwt(5) - solution/graphics data file.

CC              ifrdwt(6) - graphics plots.

CC     ifrsav - flag used to read output save data file to restore input

CC              data units and values.

CC              0 - no, do not read.

CC              1 - yes, read output save data file.

CC     ifsave - output data save file has been written.

CC              0 - no

CC              1 - yes

CC     ifsean - print seasonal or quarterly tables.

CC              0 - none.

CC              1 - seasonal.

CC              2 - quarterly.

CC     ifstch - solution direct access file flag.

CC              0 - file name specified.

CC              1 - scratch file used.

CC     ifstco - save input data file scratch flag.

CC              0 - file name specified.

CC              1 - scratch file used.

CC     ifspol - rectangular versus polar source coordinate flag used

CC              in plotting.

CC              0 - rectangular

CC              1 - polar

CC              2 - mixed

CC     ifswxy - hard copy plot orientation.

CC              0 - x = horizontal, y = vertical (portrait).

CC              1 - y = horizontal, x = vertical (landscape).

CC     iftpol - rectangular versus polar terrain grid system (yteran)

CC              flag.

CC              0 - rectangular

CC              1 - polar

CC     iftran - terrain input data flag.

CC              0 - no terrain data.

CC              1 - yes terrain data in file mtrnam or in zcoord.

CC     ifunit - mass units of total fuel/explosive.

CC     ifutms - = 1, if grid system is in UTM coords, else = 0          110198

CC     ifwrtt - Array of flags used to determine whether or not         110198

CC              concentration data should be read to accumlate results  110198

CC              for source groups.                                      110198

CC     igangu - grid orientation angle units.

CC              0 - degrees

CC              1 - radians

CC     igrect - grid system type flag.

CC              0 - rectangular system, 0 degrees north is positive

CC                  y axis, 90 degrees east is positive x axis.

CC              1 - polar system, x is range, y is angle measured

CC                  clockwise from 0 degrees north.

CC     igtfil - logical unit of the input save data file.

CC     igunit - grid system x,y and discrete point x,y units.  unless

CC              a polar system is used, then y is degrees.

CC              0 - meters.

CC              1 - kilometers.

CC              2 - feet.

CC              3 - yards.

CC              4 - miles.

CC     izunit - grid system z and discrete point z height units.

CC              0 - meters.

CC              1 - kilometers.

CC              2 - feet.

CC              3 - yards.

CC              4 - miles.

CC     ihmunt - units index of the mixing layer height.

CC     ihrfmt - format of input hourly meteorological data.

CC              0 - user specified ASCII free field format.

CC              1 - user specified ASCII fixed field format (format in

CC                  mformt).

CC              2 - ISCST preprocessed (CRSTER) binary unformatted

CC                  24 hour data.

CC              3 - ISCST hourly ASCII free field format (ISCST3 "FREE"

CC                  format.

CC              4 - ISCST hourly ASCII fixed field format (ISCST3 "CARD"

CC                  format

CC              5 - ISCST hourly ASCII fixed field format (ISCST3

CC                  "ASCII" default format)

CC              6 - ISCST preprocessed (CRSTER) ASCII fixed field format

CC                  24 hour data (ISCST "SPECIL" format).

CC     ihsfmt - format of input hourly source data.

CC              0 - free field format.

CC              1 - fixed field format (format in mforms).

CC     ihunit - units of the met. data measurement elevation,

CC              length units.

CC     ilbstr - array of indices giving the start of print labels -

CC              ilbstr(1) - index in array mscrch at which the main

CC                          calculation label begins.

CC              ilbstr(2) - index in array mscrch at which the units of

CC                          the x coordinate begins.

CC              ilbstr(3) - index in array mscrch at which the units of

CC                          the y coordinate begins.

CC              ilbstr(4) - index in array mscrch at which the name of

CC                          the quantity being calculated begins.

CC                          dosage concentration or deposition.

CC              ilbstr(5) - index in array mscrch at which the calc-

CC                          ulation height for the grid coordinate axes

CC                          label begins.

CC     ilinem - array giving the line numbers for lines in mscrnm that

CC              contain data fields. if ilinem(k) > 0, then the line

CC              starting at mscrnm(1,ilinem(k)) contains data fields,

CC              where k = 1 to 20 (maximum number of lines that can

CC              contain data fields = 20).

CC     ilstxs - last PCIT x point plotted.

CC     ilstys - last PCIT y point plotted.

CC     imodes - PC text/graphics mode index.

CC     impfil - logical unit number of input digitized map data.

CC     imsech - number of microseconds high for determination of

CC              number of mouse clicks.

CC     imsecl - number of microseconds low for determination of

CC              number of mouse clicks.

CC     imxfil - scratch file logical unit number used to compile

CC              dispersion calculation reports.

CC     imxpix - maximum number of pixels in horizontal crt screen.

CC     imxscl - size of mouse in pixels in horizontal direction.

CC     imypix - maximum number of pixels in vertical crt screen.

CC     imyscl - size of mouse in pixels in vertical direction.

CC     inhfil - logical unit number of input hourly met. data file.

CC     inhrec - hourly input data record counter.

CC     inidys - day of desired met. data (01 - 31).

CC     inihrs - hour of desired met. data (0000-2300) or (0100-2400).

CC     inimin - minute of desired met. data (00 - 59).

CC     inimon - month of desired met. data (01 - 12).

CC     iniyrs - year of desired met. data (19yy or yy).

CC     injdys - julian day of desired met. data (001 - 366).

CC     insrec - hourly source data input data record counter.

CC     ioffst - offset indices to plot matrix arrays.

CC              ioffst(1) - offset index to surface 1 data.

CC              ioffst(2) - offset index to surface 2 data.

CC     iofhrs - offset to use hour as index.

CC              0 - hourly range is 0100-2400.

CC              1 - hourly range is 0000-2300.

CC     iogfil - logical unit number of input/output solution/graphics

CC              data file.

CC     iogrec - record counter for concentration, dosage, etc. arrays

CC              in random access solution/graphics file.

CC     iogpnt - pointer to start of concentration, dosage, etc. arrays

CC              in random access solution/graphics file data record.

CC     iotfil - data print logical unit number.

CC     ipages - print page number.

CC     iplfil - graphics output file number.

CC     ipmode - PCIT plot mode -

CC              0 - no graphics

CC              5 - 320 by 200 monochromatic

CC              4 - 320 by 200 color

CC              6 - 640 by 200 monochromatic

CC             16 - EGA color card

CC             18 - VGA color card

CC     ipoptn - array containing plot option flags for the type of plot

CC              desired. a single plot is generated from the flags set.

CC              1 - plot y as a function of x.

CC              2 - plot line of maximum surface depth, height,

CC                  magnitude, et.) within x,y axes.

CC                  2 - surface 1

CC                 -2 - surface 2

CC              3 - plot contours of surface depth, height, magnitude

CC                  using actual values.

CC                  3 - surface 1

CC                 -3 - surface 2

CC              4 - plot contours of surface depth, height, magnitude

CC                  using log of values. this option is selected,

CC                  if plots produced under 3 above have stange wavy

CC                  lines or discontinuities, due to very flat or very

CC                  steep gradients.

CC                  4 - surface 1

CC                 -4 - surface 2

CC              5 - plot special symbols.

CC     iprunt - air pressure units index.

CC     ipsunt - particle diameter units.

CC     iptbuf - buffer used to read and write direct access solution

CC              pointer records.

CC     iptrec - record counter for random access solution file, used for

CC              counting hour/source pointers.

CC     iptpnt - counter used for placing hour/source pointers in consec.

CC              random access records.

CC     ipvunt - length units of particle settling velocity.

CC     iquart - quarter of the year index.

CC     iqunit - array containing a) the index of the source time units

CC              in the 1's position when emission is a rate. b) the

CC              index of the source mass units in the 10's position.

CC              c) the index of the source time or calculation time

CC              given by taucld in the 100's position.

CC     ircbuf - buffer used to read and write direction access dispersion

CC              calculation solution records.

CC     irdcol - current column number to read from or store into the

CC              the menu array mscrnm.

CC     irdrow - current row number to read from or store into the menu

CC              array mscrnm.

CC     irndys - current day of run.

CC     irnhrs - current hour of run.

CC     irnjdy - current julian day of run

CC     irnmon - current month of run.

CC     irnyrs - current year of run.

CC     irount - roughness length units index.

CC     irowmx - maximum number of rows on crt screen.

CC     irxndx - x axis ascending, descending flag.

CC              0 - ascending.

CC              1 - descending.

CC     iryndx - y axis ascending, descending flag.

CC              0 - ascending.

CC              1 - descending.

CC     iscdct - ISC input data day counter.

CC     iscdys - array specifying which days in the year are to be

CC              processed.

CC              0 - do not process.

CC              1 - process.

CC     iscfil - logical unit number of scratch file used to generate

CC              iplfil from.

CC     ischmd - ISC rural or urban mixing layer depths.

CC              0 - rural.

CC              1 - urban.

CC     ischrs - ISC input data 24 hour counter.

CC     iscyrs - year of ISC met. data.

CC     iseasn - season of the year index.

CC     isgrec - record counter for SigmaPlot output file.

CC     ishfil - logical unit number of hourly input source data.

CC     ismunt - array of length units for the source parameters xsmlry,

CC              xsmlrz, xlrgry and xlrgrz. a) xsmlry units index in 1's

CC              position. b) xsmlrz units index in 10's position.

CC              c) xlrgry units index in 100's position. d) xlrgrz

CC              units index in 1000's position.

CC     isofrq - array indicating whether or not a source is burned or

CC              or detonated for each hour of the day.

CC     isogrp - array containing the source sequence numbers used to

CC              describe each source print group.

CC              >0 - include specific source.

CC              <0 - include all sources up to and including the

CC                   absolute value or up to the previous positive

CC                   value in the same group.

CC     isotyp - array containing a) the index of the source type

CC              in the 1's position (1-volume/point,2-line).

CC              b) the index of the source emission type in the 10's

CC              position (1-instantaneous,2-quasi-continuous square

CC              wave).

CC     ispacm - array giving the number of columns in each data field

CC              in the current menu.  the l'th data field ends at

CC              mscrnm(itabsm(k,l)+ispacm(k,l)-1,ilinem(k)).

CC     ispage - saved current display page number.

CC     ispclc - array containing the type of special symbol to plot.

CC              1 - box with line from center to upper right

CC                  corner.

CC              2 - box with corners cut off with line from

CC                  center to top center.

CC              3 - triangle with line from center to top

CC                  center peak.

CC              4 - plus sign.

CC              5 - x symbol.

CC              6 - diamond with line from center to top center

CC                  peak.

CC              7 - upward pointing arrow.

CC              8 - x symbol with line jointing the upper ends

CC                  of lines.

CC              9 - z symbol with small line drawn mid-way down

CC                  horizontally.

CC             10 - y symbol.

CC             11 - box with diagonal lines drawn out from each

CC                  corner and with a line from the center of

CC                  the box to the upper right corner.

CC             12 - asterisk symbol.

CC             13 - hourglass symbol.

CC     ispclr - array containing the color index for each special

CC              symbol to plot

CC     isppol - array containing a rectangular/polar flag for each

CC              special symbol to plot

CC              0 - rectangular

CC              1 - polar

CC     ispunt - reference wind speed length units or knots.

CC     isrect - array containing a) the source x,y units in the 1's

CC              position. b) the source coord system index in the 10's

CC              position. c) the source height units in the 100's

CC              position.

CC     issunt - wind speed shear length units or knots.

CC     istabl - Pasquill stability category index.

CC     istrec - graphics/solution file record counter.

CC     istunt - time units index of the measurement time of the std.

CC              dev. of the wind direction angle.

CC     isunit - mass units of pollutant species.

CC     isvfil - logical unit number of the output save data file.

CC     isvbrd - PC video card index. Type of text/graphics board.

CC              see iboard.

CC     isvmod - saved current mode of crt screen, same as lmodes.

CC     iswopt - general program options.

CC              iswopt(1)  - dosage calculation option.

CC                           0 - not calculated.

CC                           1 - dosage calculated.

CC              iswopt(2)  - concentration calculation option.

CC                           0 - not calculated.

CC                           1 - calculated.

CC              iswopt(3)  - time-average concentration calculation

CC                           option.

CC                           0 - not calculated.

CC                           1 - calculated.

CC              iswopt(4)  - gravitational deposition calculation

CC                           option.

CC                           0 - not calculated.

CC                           1 - calculated.

CC              iswopt(5)  - produce output file that can be processed

CC                           by Sigma Plot routines.

CC                           0 - no.

CC                           1 - yes.

CC              iswopt(6)  - terrain elevation flag for complex terrain

CC                           dispersion calculations.

CC                           0 - no terrain, no flag poles.

CC                           1 - no terrain, but have flag poles. One   120702

CC                               Grid flag pole zcoord(1) and one or    120702

CC                               more discrete flag poles zdscrt        120702

CC                           2 - have complex terrain.

CC                           3 - have grid system of flag poles         120702

CC                               plus discrete flag poles               120702

CC              iswopt(7)  - packed date time of hours met. data.

CC              iswopt(8)  - final or gradual plume/cloud rise option.

CC                           1 - final.

CC                           2 - gradual depending on distance.

CC              iswopt(9)  - either Pasquill stability was entered or

CC                           net radiation index.

CC                           0 - net radiation index.

CC                           1 - Pasquill stability category.

CC              iswopt(10) - calms,

CC                           0 - flag calms as no dispersion calc.,

CC                               con, dos, dep are all zero for the

CC                               calm hour.

CC                           1 - set calms = 1.0 mps and wind direction

CC                               = to previous hour.

CC              iswopt(11) - missing met. data

CC                           0 - flag as no dispersion calc., con, dos,

CC                               dep are all zero for the missing data

CC                               hour.

CC                           1 - default according to stability and wind

CC                               speed or from previous hour.

CC              iswopt(12) - option to print table of highest and

CC                           second highest at each receptor.

CC                           0 - no.

CC                           1 - yes.

CC              iswopt(13) - dosage, concentration, deposition output

CC                           mass units index.

CC                           0 - grams         g.

CC                           1 - kilograms    kg.

CC                           2 - pounds       lb.

CC                           3 - ounces       oz.

CC                           4 - tons         tn.

CC                           5 - milligrams   mg.

CC                           6 - micrograms   ug.

CC                           7 - particles    pa.

CC              iswopt(14) - print time units option.

CC                           0 - print units are seconds.

CC                           1 - print units are minutes.

CC                           2 - print units are hours.

CC              iswopt(15) - print output length units.

CC                           0 - meters

CC                           1 - kilometers

CC                           2 - feet

CC                           3 - yards

CC                           4 - miles

CC                           5 - centimeters

CC                           6 - millimeters

CC                           7 - micrometers

CC                           8 - inches

CC              iswopt(16) - print output area units.

CC                           0 - square meters

CC                           1 - square kilometers

CC                           2 - square foot

CC                           3 - square yards

CC                           4 - square miles

CC                           5 - square centimeters

CC                           6 - square millimeters

CC                           7 - square micrometers

CC                           8 - square inches

CC                           9 - acres

CC                          10 - hectares

CC              iswopt(17) - print output volume units.

CC                           0 - cubic meters

CC                           1 - cubic kilometers

CC                           2 - cubic foot

CC                           3 - cubic yards

CC                           4 - cubic miles

CC                           5 - cubic centimeters

CC                           6 - cubic millimeters

CC                           7 - cubic micrometers

CC                           8 - cubic inches

CC                           9 - liters

CC                          10 - parts per million

CC                          11 - parts per billion

CC                          12 - parts per trillion

CC              iswopt(18) - option to print maximum 50 receptors.

CC                           0 - no.

CC                           1 - yes.

CC              iswopt(19) - flag used to indicate more than one year

CC                           of data under the summary mode.

CC                           0 - no.

CC                           1 - yes.

CC              iswopt(20) - print individual sources and/or sum of

CC                           sources.

CC                           0 - print sum only.

CC                           1 - print source groups.

CC              iswopt(21) - print source maximum plume rise heights.

CC                           0 - no.

CC                           1 - yes.

CC              iswopt(22) - print line width flag.

CC                           0 - 132 print columns.

CC                           1 -  80 print columns.

CC              iswopt(23) - rectangular source coordinates are added to

CC                           xorgin,yorgin.

CC                           0 - no

CC                           1 - yes

CC              iswopt(24) - print hourly input met. data option.

CC                           0 - no

CC                           1 - yes

CC              iswopt(25) - option used to either use both stable

CC                           and adiabatic plume rise for instantaneous

CC                           sources or only use stable rise.

CC                           0 - use both stable and adiabatic rise.

CC                               maximum stable rise is limited to

CC                               maximum adiabatic rise.

CC                           1 - use only stable rise. maximum stable

CC                               rise is limited only by restricting

CC                               dphidz to be >= 3.344e-4 (deg k/m).

CC              iswopt(26) - print hourly source data.

CC                           0 - no.

CC                           1 - yes.

CC              iswopt(27) - restrict dispersion calculations to daytime

CC                           hours (sunrise+1hour to sunset-1hour)

CC                           0 - no.

CC                           1 - yes.

CC              iswopt(28) - print each n-hour averaging period

CC                           0 - no.

CC                           1 - yes.

CC              iswopt(29) - if less then 75% of data is valid for short

CC                           term averages, use 75% of average interval or

CC                           use actual valid count to calculate average.

CC                           0 - 75% minimum

CC                           1 - Actual count

CC              iswopt(30) - used to refresh a menu display, but not allow

CC                           user interaction.

CC                           0 - allow interaction.

CC                           1 - do not allow interaction, display only.

CC              iswopt(31) - order of terrain elevation coordinates in

CC                           complex terrain input data file.

CC                           0 - all x elevations (1 to nxpnts) are read

CC                               for each y (1 to nypnts).

CC                           1 - all y elevations (1 to nypnts) are read

CC                               for each x (1 to nxpnts).

CC              iswopt(32) - Flag to turn off/on menu functions at the  071196

CC                           top of the CRT screen.                     071196

CC              iswopt(33) - Print calm wind warning messages.          090500

CC                           0 - no                                     090500

CC                           1 - yes                                    090500

CC              iswopt(34) - Wind direction is From or Towards          051503

CC                           0 - From                                   051503

CC                           1 - Towards                                051503

CC              iswopt(35) - Print met. data error/warning messages     051503

CC                           0 = no                                     051503

CC                           1 - yes                                    051503

CC              iswopt(36) - n/a

CC              iswopt(37) - n/a

CC              iswopt(38) - n/a

CC     iswplt - graphics options.

CC              iswplt(1)  - flag indicating coordinates adjusted for

CC                           origin.

CC                           0 - no.

CC                           1 - yes.

CC              iswplt(2)  - axes are linear, log-linear, log-log.

CC                           0 - linear

CC                           1 - log x and linear y

CC                           2 - linear x and log y

CC                           3 - log-log

CC              iswplt(3)  - draw curve point to point or smooth curve

CC                           using spline function. used for y as a

CC                           function of x curves and line of maximum

CC                           curves.

CC                           0 - spline

CC                           1 - point to point

CC              iswplt(4)  - plot axes & grid coordinates, plot only

CC                           grid coordinates or neither.

CC                           0 - plot axes & grid coordinates

CC                           1 - plot only grid coordinates

CC                           2 - plot neither axes or grid coordinates

CC                           if iswplt(4) = 0 or 1 the inputs xlmrgn,

CC                           xrmrgn,ytmrgn and ybmrgn must be input >

CC                           zero.

CC              iswplt(5)  - plot x and y as horizontal and vertical

CC                           axes, respectively or vice-versa.

CC                           0 - x and y are horizontal and vertical

CC                               axes.

CC                           1 - x and y are vertical and horizontal

CC                               axes.

CC              iswplt(6)  - draw solid, dashed, dotted, or dash-dot

CC                           line for y as a function of x or line of

CC                           maximum curve for surface 1. see ifdash.

CC              iswplt(7)  - center legend according to the length of

CC                           the longest line and left justify lines or

CC                           center each line independently.

CC                           0 - center independently

CC                           1 - left justify

CC              iswplt(8)  - plot grid coordinate positions.

CC                            0 - no

CC                           >0 - special symbol to draw at grid coord-

CC                                inates. see subroutine SPCLQ for

CC                                symbols.

CC              iswplt(9)  - plot background color index. see icolor for

CC                           color indices.

CC              iswplt(10) - option to plot only tick marks on axes to

CC                           indicate coordinates or to plot entire axis

CC                           to axis lines.

CC                           0 - tick marks on axes

CC                           1 - axis to axis lines

CC              iswplt(11) - option to draw the actual value to label

CC                           contour levels or use alpha or numeric

CC                           indices for surface 1.  if negative,

CC                           legend is also drawn.

CC                              0 - label contours with actual value.

CC                           +- 1 - label contours with alphabetic

CC                                  characters (a,b,c,...).

CC                           +- 2 - label contours with index numbers

CC                                  (1,2,3,...).

CC                              3 - no labels

CC              iswplt(12) - color of line of maximum curve for surface 1

CC                           or y as a function of x curve. see icolor.

CC              iswplt(13) - color of screen 1 for VT125 and PCIT

CC              iswplt(14) - color of screen 2 for VT125 and PCIT

CC              iswplt(15) - color of screen 3 for VT125 and PCIT

CC              iswplt(16) - color index of plot axes and labels (VT125

CC                           screen 1).

CC              iswplt(17) - boundary code - program restricts plotting

CC                           to inside of specified area.

CC                           1 - xlft = 0.0, xrit = xnchpl, ybot = 0.0,

CC                               ytop = ynchpl

CC                           2 - xlft = xlmrgn, xrit = xrmrpg,

CC                               ybot = ybmrgn, ytop = ytmrpg

CC              iswplt(18) - not used.

CC              iswplt(19) - not used.

CC              iswplt(20) - where to place legend giving actual value

CC                           of characters or indices used to label the

CC                           contour curves for surface 1.

CC                           0 - upper right corner inside of plot axes.

CC                           1 - upper left corner inside of plot axes.

CC                           2 - lower left corner inside of plot axes.

CC                           3 - lower right corner inside of plot axes.

CC                           4 - top right margin.

CC                           5 - top left margin.

CC                           6 - bottom left margin.

CC                           7 - bottom right margin.

CC              iswplt(21) - not used.

CC              iswplt(22) - draw solid, dashed, dotted, or dash-dot

CC                           line for surface 2 line of maximum. see

CC                           ifdash.

CC              iswplt(23) - color of line of maximum curve for surface

CC                           2 see icolor.

CC              iswplt(24) - option to draw the actual value to label

CC                           isopleth levels or use alpha or numeric

CC                           indices for surface 2.  if negative,

CC                           legend is also drawn.

CC                              0 - label contours with actual value.

CC                           +- 1 - label contours with alphabetic

CC                                  characters (a,b,c,...).

CC                           +- 2 - label contours with index numbers

CC                                  (1,2,3,...).

CC                              3 - no labels

CC              iswplt(25) - where to place legend giving actual value

CC                           of characters or indices used to label the

CC                           isopleth curves for surface 2.

CC                           0 - upper right corner inside of plot axes.

CC                           1 - upper left corner inside of plot axes.

CC                           2 - lower left corner inside of plot axes.

CC                           3 - lower right corner inside of plot axes.

CC                           4 - top right margin.

CC                           5 - top left margin.

CC                           6 - bottom left margin.

CC                           7 - bottom right margin.

CC              iswplt(26) - used to control whether or not menus are

CC                           skipped under the F8 key.

CC                           0 - not skipped.

CC                           1 - skipped.

CC              iswplt(27) - plot arrow indicating north flag.

CC                           0 - no.

CC                           1 - yes.

CC              iswplt(28) - plot background map.

CC                           0 - no.

CC                           1 - yes, black/white.

CC                           2 - yes, color.

CC              iswplt(29) - plotting on/off flag.

CC                           0 - off or no.

CC                           1 - on or yes.

CC              iswplt(30) - not used

CC     isxunt - units index of the standard deviation of the longitud-

CC              inal wind angle.

CC     isystm - computer system type.

CC              1 - Hewlett Packard HP9000/800 series.

CC              2 - PCIT (IBM compatible PC).

CC     isyunt - units index of the standard deviation of the wind

CC              direction angle.

CC     iszunt - units index of the standard deviation of the wind

CC              elevation angle.

CC     itable - print table number.

CC     itabsm - array giving the start columns of each data field on the

CC              line given by ilinem(k). the data field starts at

CC              mscrnm(itabsm(k,l),ilinem(k)) for the l'th field on the

CC              line, where l = 1 to 10 (maximum number of data fields

CC              per line = 10).

CC     itmunt - time units of time-mean concentration time timavg.

CC     itpunt - air temperature units index.

CC              0 - Celsius

CC              1 - Kelvin

CC              2 - Fahrenheit

CC              3 - Rankine

CC     itrfil - logical unit number of the terrain data input file.

CC     ivarmt - array specifying the location of values for met.

CC              parameter variables in an input record. 0 = parameter

CC              not in data record, hold constant.  >0 = index of value

CC              in input record.  <0 = parameter not in input record,

CC              use default value for parameter.

CC              parameter variables -

CC               (1) - year iniyrs.

CC               (2) - month inimon.

CC               (3) - day inidys.

CC               (4) - hour inihrs.

CC               (5) - julian day injdys.

CC               (6) - wind speed refspd.

CC               (7) - wind speed power law coefficient pwrlaw.

CC               (8) - wind direction wnddir.

CC               (9) - lateral turbulence intensity sigmap.

CC              (10) - vertical turbulence intensity sigmep.

CC              (11) - longitudinal turbulence intensity trbixr.

CC              (12) - air temperature airtmp.

CC              (13) - air pressure airprs.

CC              (14) - vert. pot. temp. gradient dphidz.

CC              (15) - mixing layer depth hmdpth.

CC              (16) - wind speed shear spdshr.

CC              (17) - wind direction shear dirshr.

CC              (18) - Pasquill stability category stblty.

CC              (19) - net radiation index radinx.

CC              (20) - measurement time for lateral turbulence intensity

CC                     tauosa.

CC              (21) - roughness length roughl.

CC              (22) - humidity airhum.

CC              (23) - clearing index clrinx.

CC              (24) - cloud cover cldcov.

CC              (25) - cloud ceiling cldhgt.

CC     ivarso - array specifying the location of values for source

CC              parameter variables in an input record. 0 = parameter

CC              not in data record, >0 = index of value in input record

CC              corresponding to the parameter variables -

CC               (1) - source number (required).

CC               (2) - total mass of material released.

CC               (3) - material heat content hetcnt.

CC               (4) - burn rate brnrat.

CC     ivgunt - temperature units index of the vertical gradient of

CC              potential temperature.

CC              0 - Celsius

CC              1 - Kelvin

CC              2 - Fahrenheit

CC              3 - Rankine

CC     ivrmod - Subversion number for most recent update to the program.090498

CC              See moddat.  ivrmod is added to the current version     090498

CC              number.  If the current version is xx.x, then the update090498

CC              version is xx.x.iiii, where iiii = ivrmod.              090498

CC     iwkfil - logical unit number of screen menu data work file.

CC     ixlpnt - array containing the first and last x point of each

CC              contour segment for the placement of contour labels.

CC     ixlpts - array containing the x locations where contour labels

CC              have been drawn to prevent over drawing of labels.

CC     iylpnt - array containing the first and last y points of each

CC              contour segment for the placement of contour labels.

CC     iylpts - array containing the y locations where contour labels

CC              have been drawn to prevent over drawing of labels.

CC     jdsunt - wind direction shear length (height) units.

CC     jdubrn - default time units of fuel burn rate.

CC     jduhet - default mass units of fuel heat content.

CC     jdunit - volume units of pollutant/species density.

CC     jfutms - = 2, if origin specified and is UTM, = 1, if origin     110198

CC              specified and not UTM, = 0 if origin not specified.     110198

CC     jjjoff - array containing ofset indices for the calculation of

CC              dosage, concentration, time-average concentration,

CC              and gravitational deposition.

CC     joffs1 - offset index to start of surface 1 data or surface 2

CC              data in array calcsa.

CC     jpoptn - counter used to move through the plot option array

CC              ipoptn.

CC     jpvunt - time units of particle settling velocity.

CC     jqunit - array containing the fuel heat content units.  a) heat

CC              units index in the 1's position. b) mass units index

CC              in the 10's position.

CC     jrxndx - prevent irxndx flag.

CC              0 - do not prevent.

CC              1 - prevent.

CC     jryndx - prevent iryndx flag.

CC              0 - do not prevent.

CC              1 - prevent.

CC     jspunt - reference wind speed time units if not knots.

CC     jssunt - wind speed shear time units if not knots.

CC     jvgunt - length (height) units index of the vertical gradient of

CC              potential temperature.

CC     kfutms - = 1, if discrete points are UTM, else = 0               110198

CC     kqunit - array containing the fuel burn rate units index.

CC              a) burn rate mass units in 1's position.  b) burn rate

CC              time units in 10's position.

CC     kssndx - source index counter.

CC     lcolor - last line color plotted.

CC     lfutms - = 1, if jfutms > 0 and source is in UTM, else = 0       110198

CC     linetl - total number of lines per print page.

CC     llines - print line counter.

CC     lmodes - PC text/graphics mode when OBODM is initialized, same as

CC              isvmod.

CC     locxms - current mouse location in the horizontal direction.

CC     locyms - current mouse location in the vertical direction.

CC     lswxax - x axis flag. 0 = linear, 1 = log.

CC     lswyax - y axis flag. 0 = linear, 1 = log.

CC     malpha - alphabet used for check of input commands (a-z),(a-z).

CC     mbcnam - file name of batch processing file.

CC     mblnk1 - one blank character.

CC     mcolor - array of color names for each color index.  see icolor

CC              for the respective color index.

CC               1 - 'black ',

CC               2 - 'blue  ',

CC               3 - 'green ',

CC               4 - 'cyan  ',

CC               5 - 'red   ',

CC               6 - 'magent',

CC               7 - 'yellow',

CC               8 - 'dkwhit',

CC               9 - 'gray  ',

CC              10 - 'ltblue',

CC              11 - 'ltgren',

CC              12 - 'ltcyan',

CC              13 - 'ltred ',

CC              14 - 'ltmage',

CC              15 - 'ltyell',

CC              16 - 'ltwhit'

CC     mcmlab - array containing command bar labels

CC     mdbnam - file name of database screen menu template file

CC              (OBODMENU.OBD).

CC     mdirec - directory path name.

CC     mfdnam - file name of input fuels/explosives default data file

CC              (OBODFUEL.OBD).

CC     mforms - array containing the format for user specified fixed

CC              field format source data (151).

CC     mformt - array containing the format for user specified fixed

CC              field format meteorological data (151).

CC     moddat - date of most recent update to current version of        090498

CC              program.  See ivrmod.                                   090498

CC     mxfrom - format for x axis data read from file mxynam.

CC     myform - format for y axis data read from file mxynam.

CC     mzform - format for z elevation data read from file mxynam.

CC     mgtnam - name of input save data file.

CC     mhrlab - plotting label for date/time.

CC     mlablx - x axis label plotted in x axis margin.

CC     mlably - y axis label plotted in y axis margin.

CC     mlngnd - plot legend or caption. the end of each line is given

CC              by a dollar sign. two consecutive dollar signs means

CC              end of legend.

CC     mapnam - file name of digitized map data input file.

CC     mernam - file name of error data file OBODMERR.OBD.

CC     mnhnam - file name of input hourly met. data file.

CC     mnpbuf - general character buffer, primarily input buffer.

CC     modelt - names of different quantities that can be calculated.

CC     mognam - file name of input/output solution/graphics data file.

CC     motnam - name of print output file.

CC     mplnam - name of plot output file.

CC     mpolnt - name of fuel/explosive being modeled.

CC     mscrch - scratch label space.

CC     mscrnm - nmxcol by nmxrow character array containing the screen

CC              menu. the first dimension specifies screen columns

CC              left to right, 1 to nmxcol.  the second dimension

CC              specifies screen line number top to bottom, 1 to nmxrow.

CC     msgnam - file name of Sigma Plot output file.

CC     mshnam - file name of hourly input source data file.

CC     msoids - source id name for each source.

CC     mspeci - name of pollutant or species being processed.

CC     msvnam - name of the output save data file.

CC     mtitle - run title.

CC     mtrnam - file name of terrain data input file.

CC     munita - area unit names.

CC     munitg - angle unit names.

CC     munitl - length unit names.

CC     munitm - mass unit names.

CC     munitt - time unit names.

CC     munitv - volume unit names.

CC     mwknam - name of screen menu data work file (OBODWORK.OBD).

CC     mxynam - name of data file containing x,y axes and z elevations.

CC     navhrs - n-hour averaging times or total times for deposition (6).

CC     nbtchc - number of current batch file case.

CC     nbxpss - number of special labels or legends drawn on the

CC              plot. used to prevent other labels or legends from

CC              writing over those plotted. see xbxpsl, xbxpel, ybxpsl,

CC              and ybxpel.

CC     nchlcr - number of total characters in plot legend.

CC     nchtcr - number of characters in plot title.

CC     nchtrs - number of characters in title in mtitle.

CC     nchxcr - number of total characters in x axis label.

CC     nchycr - number of total characters in y axis label.

CC     ncolmn - number of columns on crt terminal.

CC     ncontr - array containing the number of contour levels to plot

CC              for surface 1 and surface 2 (maximum = nmxctr).

CC     nctlbl - number of contour labels that have been drawn on a plot

CC     ncurvs - the number of contour curv segments used to determine

CC              where to label each contour line.

CC     ndgxes - number of digits to right of decimal for numbers

CC              labeling grid coordinates on x axis.

CC     ndgyes - number of digits to right of decimal for numbers

CC              grid coordinates on y axis.

CC     ngrdpt - number of receptors in the grid axis system (nxpnts*

CC              nypnts).

CC     ngroup - total number of different source groupings to print

CC              calculations for.

CC     ngrpct - number of sources in plot group.

CC     ngrplt - plot sum of sources or group source flag.

CC               0 - sum

CC              >0 - group

CC     nforms - number of FORTRAN edit codes using e, f or g in the

CC              format array mforms.

CC     nformt - number of FORTRAN edit codes using e, f or g in the

CC              format array mformt.

CC     nhravg - number of hours in averaging time being plotted.

CC     nhrlab - number of characters in label mhrlab.

CC     nincxs - number of minor tick marks to major tick marks at which

CC              axis coordinate is labeled.

CC     nincys - number of minor tick marks to major tick marks at which

CC              axis coordinate is labeled.

CC     nkeybd - array containing the extension number and character

CC              number for each of the control keys - enter (return),

CC              f1, f2, f3, f4, f5, f6, f7, f8, f9, f10, pgup, pgdn,

CC              up arrow, dn arrow, left tab, right tab, left arrow,

CC              right arrow, delete, ctrl k or k, backspace, home, end,

CC              esc, ctrl home, ctrl end.

CC     nlbsch - array giving the number characters for each respective

CC              label in mscrch. see ilbstr.

CC     nlincs - counter over the number of sources in current line      110198

CC              source simulation.                                      110198

CC     nlsorc - number of volume sources added for the simulation of

CC              line sources.

CC     nmxcol - maximum number of columns in a screen menu.

CC     nmxcor - maximum calculation space available for all dispersion

CC              and deposition calculations. dimension of array calcs

CC              in common 'OCDSPM.INC' in dispersion related routines.

CC     nmxctr - maximum dimension for number of contours, arrays contrs,

CC              icndsh, icnclr.

CC     nmxdrp - maximum number of particle size categories.

CC              arrays that depend on nmxdrp are - drpupr, drplwr,

CC              pctmat, vspart and gammad.

CC     nmxdys - maximum number of days in hourly input data year.

CC     nmxfld - maximum number of data fields in a screen menu line.

CC     nmxfnc - maximum number of pairs of points in y plotted as a

CC              function of x plots, arrays functx, functy.

CC     nmxgrp - maximum number of source groups that can be printed.

CC              arrays that depend on nmxgrp are - nsogrp and isogrp

CC     nmxhrs - maximum number of hours of input data to process.

CC     nmxlin - maximum number of lines of data in a screen menu.

CC     nmxrow - maximum number of rows in screen menu.

CC     nmxscn - maximum number of screen menu templates.

CC     nmxsor - maximum number of sources possible. one less than source

CC              arrays dimension.

CC              arrays that depend of the value of nmxsor are -

CC                isrect,idrect,iqunit,jqunit,kqunit,ismunt,isotyp,

CC                qemiss,taucld,relhgt,clddxs,clddys,cldlng,cldsht,

CC                cldang,clddpt,xsmlry,xsmlrz,xlrgry,xlrgrz,hetcnt,

CC                brnrat,clddxe,clddye,relhte,cldmax,gammat,alphas,

CC                betass,cldmxd,isofrq,clddzs,clddze,calhgt.            012599

CC     nmxspl - maximum dimension for number of special symbols,

CC              arrays spclxs, spclys, spclht, spclan, ispclc, ispclr.

CC              number to be plotted is in nspcls.

CC     nmxxyz - maximum number of receptor coordinates possible.

CC              arrays that depend on nmxxyz are - xcoord, ycoord,

CC              xdscrt, ydscrt, zdscrt, xteran and yteran.

CC     nochar - value returned by RDSTA to indicate no characters are   020797

CC              waiting in input buffer.                                020797

CC     nplots - plot number.

CC     npoptn - number of plot options selected in array ipoptn.

CC     nquart - array counter for number of hours in each quarter.

CC     nrecda - array of start record numbers for each screen menu

CC              data set in file mwknam.

CC     nrecdt - array giving the number of data records in file mwknam

CC              for each menu.

CC     nrecsc - array of start record numbers for each screen menu

CC              template found in file mdbnam.

CC     nseasn - array counter for number of hours in each season.

CC     nsogrp - array containing the number of source ident numbers

CC              used to describe each source print group.

CC     nsourc - number of sources to model (nsourc + nlsorc <= nmxsor).

CC     nspcls - number of special symbols to be plotted (maximum =

CC              nmxspl).

CC     nsubsc - array containing the number of data sets found in

CC              file mwknam for each screen menu template found in

CC              file mdbnam.

CC     ntotal - grand total number of calculation points required

CC              (ntotal <= nmxcor).

CC     ntothr - hourly met. input data hour or observation counter.

CC     ntotpt - total number of receptor points (nxpnts*nypnts+nxypnt).

CC     numbtn - number of mouse buttons.

CC     numsce - array of end screen numbers for the various menu

CC              categories shown under numscs below.

CC     numscs - array of start screen numbers for the various menu

CC              categories (receptor data, source data, etc.).

CC              numscs(1) - start menu number for default drive and

CC                          directory sub. RINPA.

CC              numscs(2) - start menu number for default data files,

CC                          sub. RINPB.

CC              numscs(3) - start menu number for main control menu,

CC                          sub. RINPC.

CC              numscs(4) - start menu number for receptor related

CC                          data, sub. RINPD.

CC              numscs(5) - start menu number for source related data,

CC                          sub. RINPE.

CC              numscs(6) - start menu number for meteorological

CC                          related data, sub. RINPK.

CC              numscs(7) - start menu number for control/options

CC                          related data, sub. RINPP.

CC              numscs(8) - start menu number for graphics related

CC                          data, sub. RINPQ.

CC              numscs(9) - start menu number for help related info.,

CC                          sub. IHLPS.

CC              numscs(10)- start menu number for units related info.,

CC                          sub. IHLPS.

CC              numscs(11)- start menu number for default fuel/

CC                          explosives and default pollutant/species

CC                          info., sub. IFULS.

CC              numscs(12)- start menu number for current disc files,

CC                          sub. IDISC.

CC     nvlprt - particle category index counter.

CC     nvsprt - number of particle categories.

CC     nxpnts - number of x (west to east) grid coordinates in the

CC              receptor grid system (nxpnts+nxypnt <= 100).

CC     nxtran - number of x (west to east) terrain coordinates.

CC     nxyfnc - number of x and y points entered for plot of y as a

CC              function of x (maximum = nmxfnc).

CC     nxypnt - number of discrete receptor points (nxpnts+nxypnt <= 100

CC              and nypnts+nxypnt <= 100).

CC     nypnts - number of y (south to north) grid coordinates in the

CC              receptor grid system (nypnts+nxypnt <= 100).

CC     nytran - number of y (south to north) terrain coordinates.

CC     paperl - length (vertical) in inches of hard copy paper.

CC     paperw - width (horizontal) in inches of hard copy paper.

CC     pconav - calculated time-average concentration (g/m**3).

CC     pctmat - fraction of total volume of material in each

CC              particle category.

CC     pdepgr - calculated gravitational deposition (g/m**2).

CC     pekcon - calculated concentration (g/m**3).

CC     pekdos - calculated dosage (g.s/m**3).

CC     pixelx - number of pixels in horizontal x axis of crt screen.

CC     pixely - number of pixels in vertical y axis of crt screen.

CC     pwrlaw - wind speed power law exponent coefficient.

CC     pxmaxs - maximum value of x axis in x axis units.

CC     pxmins - minimum value of x axis in x axis units.

CC     pymaxs - maximum value of y axis in y axis units.

CC     pymins - minimum value of y axis in y axis units.

CC     qemiss - array of source emission strength in total mass for

CC              instantaneous or quasi-continuous sources.

CC     qfract - ratio of mass of source species to total fuel.

CC     qkemss - scaled source strength (grams).

CC     qtfuel - mass of total fuel/explosive.

CC     radinx - net radiation index.

CC     radtod - conversion from radians to degrees (degrees/radian).

CC     rastrh - number of rasters per inch in the horizontal screen.

CC     rastrv - number of rasters per inch in the vertical screen.

CC     refhgt - reference height for wind speed power law (m).

CC     refspd - wind speed at height refhgt or average wind speed (m/s).

CC     relhgt - array containing the effective release height for

CC              each source or the start effective release height for

CC              line sources.

CC     relhte - array containing the end effective release height for

CC              line sources. also used to save the source number

CC              after sources have been generated for the line source

CC              simulation.

CC     rgtmrg - right margin of hard copy plot (inch).

CC     rmissc - data to be computed as program runs, set to -1.0e21

CC     rmissd - missing data indicator, set to -1.0e20.

CC     roughl - roughness length (cm).

CC     sclxes - scale of the x axis in inches per x unit.

CC     sclxys - vector resultant scale of sclxes and sclyes.

CC     sclyes - scale of the y axis in inches per y unit.

CC     sigapr - std. dev. of the azimuth wind direction angle after

CC              modification for crossing trajectories (radians),

CC              taken from sigatr.

CC     sigatr - std. dev. of the azimuth wind direction angle after

CC              modification for cloud rise time (source function, or

CC              averaging time).  taken from sigmaq.

CC     sigepr - std. dev. of the wind elevation angle after modification

CC              for crossing trajectories (radians).

CC     sigmap - initial standard deviation of the azimuth wind direction

CC              (radians).

CC     sigmaq - standard deviation of the azimuth wind direction

CC              (radians) after adjustment for measurement time and

CC              roughness, passed from RMDLA to RMDLB.

CC     sigmep - initial standard deviation of the wind elevation angle

CC              (radians).

CC     sigmeq - initial standard deviation of the wind elevation angle

CC              (radians) passed to dispersion routines.

CC     sigxax - alongwind std. dev. of the source material distribution

CC              at the receptor distance (m).

CC     sigxls - alongwind initial std. dev. of the source material

CC              distribution (m), adjusted for the effects of plume

CC              rise.

CC     sigxop - alongwind std. dev. of the source material distribution

CC              (m).

CC     sigyax - crosswind std. dev. of the source material distribution

CC              at the receptor distance (m).

CC     sigyls - crosswind initial std. dev. of the source material

CC              distribution (m), adjusted for the effects of plume

CC              rise.

CC     sigyop - crosswind std. dev. of the source material distribution

CC              (m).

CC     sigyta - crosswind std. dev. of the source material distribution

CC              for the averaging time used in time-average con (m).

CC     sigzax - vertical std. dev. of the source material distribution

CC              at the receptor distance (m).

CC     sigzls - vertical initial std. dev. of the source material

CC              distribution (m), adjusted for the effects of plume

CC              rise.

CC     sigzop - vertical initial std. dev. of the source material

CC              distribution (m).

CC     spbotm - bottom-most plot position used in the y dimension.

CC     spclan - array containing the angle at which each special

CC              symbol is to be plotted. angle clockwise from the pos.

CC              y axis.

CC     spclht - array containing the height in inches of each special

CC              symbol to plot

CC     spclxs - array containing the x coordinate of the center of

CC              each special symbol to plot

CC     spclys - array containing the y coordinate of the center of

CC              each special symbol to plot

CC     spdshr - wind speed shear (m/s).

CC     spfrct - fraction of character height used to calculate space

CC              between label lines.

CC     spheat - specific heat of air (0.24 cal/g.deg k).

CC     splftm - left-most plot position used in the x dimension.

CC     spmass - mass of pollutant species.

CC     spmins - minimum space between plotted label lines for

CC              resolution.

CC     spshrc - calculated wind speed shear (m/s), if spdshr < 0.0.

CC     sq2pii - inverse of the square root of 2*pi.

CC     sqr2pi - square root of 2*pi.

CC     sqrt2i - inverse of the square root of 2.

CC     stblty - Pasquill stability category index (same as istabl).

CC     svdpts - Saved depth of current line source.                     110198

CC     svdxss - Saved x coordinate of start of current line source.     110198

CC     svdyss - Saved y coordinate of start of current line source.     110198

CC     svdzss - Saved z coordinate of start of current line source.     110198

CC     svhgts - Saved height of start of current line source.           110198

CC     svhtes - Saved height of end of current line source.             110198

CC     svisss - Saved emission of current line source.                  110198

CC     svlngs - Saved length current line source.                       110198

CC     svshts - Saved width of current line source.                     110198

CC     taucld - source emission time for instantaneous and quasi-

CC              continuous sources.

CC     tauosa - measurement time for sigmap (sec).

CC     tautmo - reference time base for std. dev. azimuth wind

CC              direction angle (600.0 s).

CC     tcosth - cosine of the wind direction angle adjusted for the

CC              grid system orientation angle and wind direction shear

CC              if particle categories are being used.

CC     timinv - integration time increment for quasi-cont. dosage (s).

CC     timavg - time for the calculation of time-averaged concentration

CC              (s).

CC     timris - time to maximum cloud rise (sec).

CC     timtas - adjusted source emission time, depending on time to

CC              the receptor (s) for use in adjustments to crosswind

CC              turbulence.  same as timtau, depending on plume rise.

CC     timtau - adjusted source emission time, depending on time to

CC              the receptor (s).

CC     timtox - time to receptor distance (s).

CC     topmrg - top margin of hard copy plot (inch).

CC     trbixp - longitudinal turbulence intensity after modification

CC              for crossing trajectories (radians).

CC     trbixq - initial longitudinal turbulence intensity (radians)

CC              passed to dispersion routines.

CC     trbixr - initial longitudinal turbulence intensity (radians).

CC     trbixt - longitudinal turbulence intensity prior to modification

CC              for crossing trajectories, but after modification for

CC              calculation time or source emission time (radians).

CC     tsinth - sin of the wind direction angle adjusted for the

CC              grid system orientation angle and wind direction shear

CC              if particle categories are being used.

CC     twopii - inverse of 2*pi.

CC     versod - OBODM program current version number.

CC     verson - OBODM database file OBODMENU.OBD version number.

CC     vertcl - vertical term of the concentration equation.

CC     vertgv - vertical term of the gravitational deposition equation.

CC     vllimt - array of lower limits for the meteorological parameters

CC              given under ivarmt.

CC     vspart - settling velocity (m/s) for each particle size category.

CC     vulimt - array of upper limits for the meteorological parameters

CC              given under ivarmt.

CC     whtmol - molecular weight of the pollutant/species (g/mol).

CC     wnddir - average wind direction (degrees).

CC     wndspd - transport wind speed (m/s).

CC     wspmin - minimum wind speed (m/s).

CC     xbndxs - x plot point of curve and plot boundary intersection,

CC              if ifbndx = 2 or 3.

CC     xbxpel - array of x coordinates giving the end position of a

CC              label or legend already drawn.

CC     xbxpsl - array of x coordinates giving the start position of a

CC              label or legend already drawn.

CC     xcoord - x coordinates defining the x axis of the receptor grid

CC              system.

CC     xcord1 - x axis coordinate of the receptor grid system (m).

CC     xdscrt - x coordinates defining the discrete points.

CC     xftmrg - left margin of hard copy plot (inch).

CC     xincax - increment between minor tick marks on the x axis in

CC              input x units.

CC     xinchs - number of inches in x axis.

CC     xlmrgn - number of inches in left margin of CRT plot (pxmins).

CC              must be greater than zero if iswplt(4) is = 0 or 1.

CC     xlrgry - array of source lateral reference distances.

CC     xlrgrz - array of source vertical reference distances.

CC     xnchpl - number of inches in the x dimension of the plot area.

CC     xorgin - origin of the x axis of the grid system or x origin

CC              of the polar grid. if set non-zero all xcoord points,

CC              rectangular or polar are, relative to this point.  All

CC              rectangular grid system points are added to xorgin.

CC              source rectangular coordinates are added to xorgin,

CC              only if specified by the user.

CC     xpdist - alongwind distance from source to receptor (m).

CC     xplpnt - general x plot point.

CC     xrmrgn - number of inches in right margin of CRT plot (pxmaxs).

CC              must be greater than zero if iswplt(4) is = 0 or 1.

CC     xrmrpg - distance in inches from the left-most x dimension to

CC              the right plot margin.

CC     xsmins - minimum x plot axis value in x axis units

CC     xsmlry - array of distances from the virtual point source over

CC              which rectilinear expansion in the lateral occurs.

CC     xsmlrz - array of distances from the virtual point source over

CC              which rectilinear expansion in the vertical occurs.

CC     xtemp1 - x coordinate system distance between source and receptor

CC              (m).

CC     xteran - array of x coordinates of terrain data.

CC     xxdist - alongwind virtual point source distance (m).

CC     xydist - crosswind virtual point source distance (m).

CC     xymins - minimum distance between x or y coordinates.

CC     xzdist - vertical virtual point source distance (m).

CC     ybmrgn - number of inches in bottom margin of CRT plot (pymins).

CC              must be greater than zero if iswplt(4) is = 0 or 1.

CC     ybndxs - y plot point of curve and plot boundary intersection,

CC              if ifbndx = 2 or 3.

CC     ybxpel - array of y coordinates giving the end position of a

CC              label or legend already drawn.

CC     ybxpsl - array of y coordinates giving the start position of a

CC              label or legend already drawn.

CC     ycoord - y coordinates defining the y axis of the receptor grid

CC              system.

CC     ycord1 - y axis coordinate of the recpetor grid system (m).

CC     ydscrt - y coordinates of the discrete points.

CC     yincax - increment between minor tick marks on the y axis in

CC              input y units.

CC     yinchs - number of inches in y axis.

CC     ynchpl - number of inches in the y dimension of the plot area.

CC     yorgin - origin of the y axis of the grid system or y origin

CC              of the polar grid. if set non-zero all ycoord points,

CC              rectangular or polar are, relative to this point.  all

CC              rectangular grid points are added to yorgin. source

CC              rectangular coordinates are added to yorgin, only if

CC              specified by the user.

CC     ypdist - crosswind distance from cloud centerline to receptor (m)

CC     yplpnt - general y plot point.

CC     ysmins - minimum y plot axis value in y axis units.

CC     ytemp1 - y coordinate system distance between source and receptor

CC              (m).

CC     yteran - array of y coordinates of terrain data.

CC     ytmrgn - number of inches in top margin of CRT plot (pymaxs).

CC              must be greater than zero if iswplt(4) is = 0 or 1.

CC     ytmrpg - distance from the bottom-most y dimension to the top

CC              plot margin.

CC     zbotly - height at the top of the transport layer (m).

CC     zcoord - array containing complex terrain elevation or height

CC              data for the x,y grid system.

CC     zdscrt - dosage, concentration calculation heights for the

CC              discrete receptor points.

CC     zmincs - array containing minimum surface value used in plotting

CC              contours for surface 1 and surface 2 using a log scale

CC     zminds - array containing the shift factor used in plotting

CC              surface 1 and surface 2 contours using a log scale.

CC     zondif - time difference in hours between greenwich england and

CC              geolon, geolat. positive for west longitude geolon.

CC     zpdist - calculation height (m).

CC     zpsave - saved calculation height of the grid system, used for

CC              labeling purposes.

CC     ztoply - height at the bottom of the transport layer (m).

CC

CC**********************************************************************

CC

CC                    - OBODM Dimensional units -

CC

CC             Dimension          Index    Units           Abbreviation

CC             ---------          ----- ------------------ -----------

CC             length               0   meters                  m

CC                                  1   kilometers             km

CC                                  2 - feet                   ft

CC                                  3 - yards                  yd

CC                                  4 - miles                  mi

CC                                  5 - centimeters            cm

CC                                  6 - millimeters            mm

CC                                  7 - micrometers            um

CC                                  8 - inches                 in

CC             direction            0 - degrees                 d

CC                                  1 - radians                 r

CC             heat                 0 - calorie                ca

CC                                  1 - kilocalorie            kc

CC                                  2 - British thermal unit  btu

CC                                  3 - Joule                   j

CC                                  4 - erg                     e

CC             mass                 0 - grams                   g

CC                                  1 - kilograms              kg

CC                                  2 - pounds                 lb

CC                                  3 - ounces                 oz

CC                                  4 - tons                   tn

CC                                  5 - milligrams             mg

CC                                  6 - micrograms             ug

CC                                  7 - particles              pa

CC             pressure             0 - millibars              mb

CC                                  1 - pascals                 p

CC                                  2 - inches mercury       inhg

CC                                  3 - millimeters mercury  mmhg

CC             time                 0 - seconds                 s

CC                                  1 - minutes                mn

CC                                  2 - hours                   h

CC             temperature          0 - Celsius                 c

CC                                  1 - Kelvin                  k

CC                                  2 - Fahrenheit              f

CC                                  3 - Rankine                ra

CC             velocity        0 to 8 - length units

CC                                  9 - knots                 kts

CC                             0 to 2 - time units

CC             ratio                0 - percent                pr

CC                                  1 - fraction               fr

CC             area            0 to 8 - square length units

CC                                  9 - acres                  ac

CC                                 10 - hectares               he

CC             volume          0 to 8 - cubic length units

CC                                  9 - liters                 li

CC                                 10 - parts per million     ppm

CC                                 11 - parts per billion     ppb

CC                                 12 - parts per trillion    ppt

CC             mass/volume     0 to 7 - mass units

CC                             0 to 9 - volume units

CC             mass/time       0 to 7 - mass units

CC                             0 to 2 - time units

CC             heat/mass       0 to 4 - heat units

CC                             0 to 7 - mass units

CC             angle/length    0 to 1 - angle units

CC                             0 to 8 - length units

CC             temp./length    0 to 3 - temperature units

CC                             0 to 8 - length units

CC

CC**********************************************************************

CC

CC                     - OBODM Input Data -

CC

CC     The OBODM program can only be run in an interactive environment

CC     where input data are taken from various menus displayed on the

CC     users terminal screen.  The sequence of menus and data required

CC     is controlled by the user, but is checked by the OBODM program

CC     to insure solutions are not calculated with incomplete or

CC     erroneous data.  Input data can also be taken from saved data

CC     files written by the OBODM program or from the OBODM work file.

CC

CC**********************************************************************

CC

CC                     - OBODM Data Files -

CC

CC     1) OBODMENU.OBD - required direct access screen menu template

CC                       database file, created by program OBODMenu

CC                       and OBODMkey (mdbnam, idbfil=20) (RINPA).

CC     2) OBODWORK.OBD - required direct access work file created by

CC                       OBODM (mwknam,iwkfil=21) (RINPA).

CC     3) OBODFUEL.OBD - optional sequential fuels/explosives and

CC                       pollutant/species default data file (mfdnam,

CC                       ifdfil=22) (RINPA).

CC     4) OBODMERR.OBD - file containing OBODM error messages (mernam,

CC                       ierfil=17) (KERRS).

CC     5) user-specified - mgtnam,igtfil- optional input save data file,

CC                       created in a previous run of the OBODM program,

CC                       igtfil=23 (RINPB).

CC     6) user-specified - msvnam,isvfil- optional output save data

CC                       file, isvfil=24 (RINPB).  Default is

CC                       SCRATCH2.OBD.

CC     7) user-specified - motnam,iotfil- optional output solution print

CC                       file, iotfil=25 (RINPB).  Default OBPRINT.OUT.

CC     8) user-specified - mognam,iogfil- optional input/output

CC                       graphics/solution data save file, iogfil=26

CC                       (RINPB).  Default is SCRATCH1.OBD.

CC     9) user-specified - mnhnam,inhfil- optional input hourly

CC                       meteorological data file, inhfil=27 (RINPO).

CC    10) user-specified - mshnam,ishfil- optional input hourly source

CC                       data file, ishfil=28 (RINPJ).

CC    11) user-specified - mplnam,iplfil- optional output plot data

CC                       file, iplfil=29, (RINPR).

CC                       format and contents -

CC

CC              record 1    - first record in the file contains -

CC                            (xnchpl,ynchpl,nplots)

CC                   format - (' S',2f8.4,i4,4e12.6)

CC                            xnchpl - number of inches in horizontal

CC                                     plot dimension.

CC                            ynchpl - number of inches in vertical

CC                                     plot dimension.

CC                            nplots - plot number.

CC

CC              record 2    - second record contains -

CC                            (mtitle)

CC                   format - (80a1)

CC                            mtitle - run title label.

CC

CC              record n    - the third through the second to last

CC                            record contains plot x,y information -

CC                            (xp,yp,ipen,icolor)

CC                   format - (' P',2f8.4,2i2)

CC                            xp     - move pen to xp inches.

CC                            yp     - move pen to yp inches.

CC                            ipen   - pen up/down. 2=down, 3=up, neg.=

CC                                     establish new origin.

CC                            icolor - color index.

CC                                     0 - black

CC                                     1 - blue

CC                                     2 - green

CC                                     3 - cyan

CC                                     4 - red

CC                                     5 - magenta

CC                                     6 - yellow

CC                                     7 - dark white

CC                                     8 - gray

CC                                     9 - light blue

CC                                    10 - light green

CC                                    11 - light cyan

CC                                    12 - light red

CC                                    13 - light magenta

CC                                    14 - light yellow

CC                                    15 - light white

CC

CC              record m    - last record in the file indicates end of

CC                            plot data. if the file is tape (no file

CC                            name specified), an end of file is written

CC                            after this record.

CC                   format - (' END PLOT')

CC    12) SCRATCH3.OBD - iscfil - scratch sequential file, iscfil=30

CC                       (RINPR).

CC    13) user-specified - mtrnam,itrfil- optional input terrain data,

CC                       itrfil=31 (RINPR).

CC    14) user-specified - mapnam,impfil- optional input digitized map

CC                       data, impfil=32 (RINPR).

CC    15) OBD#####.PLT - optional graphics hard copy print/plot file,

CC                       ihdfl = 33 (PLTSQ).

CC    16) user-specified - mbcnam,ibcfil- batch processing file.  file

CC                       contains sets of four 22 character records,

CC                       where record one of each set contains the input

CC                       file name, record two contains the save file

CC                       name or blanks, record three contains the print

CC                       file name or blanks and record four contains

CC                       the solution file name or blanks. ibcfil=19

CC                       (RINPB).

CC    17) SCRATCH4.OBD - imxfil- file used to compile highest and second

CC                       highest concentration, dosage, etc. tables.

CC                       imxfil = 18 (ROUTA)

CC    18) user-specified - msgnam,isgfil- file used to save Sigma Plot

CC                       output information for subsequent plotting.

CC                       isgfil=34 (OUTPT).

CC    19) user-specified - mxynam, ixyfl- file used to input x,y

CC                       coordinates and z elevation data. ixyfl=31

CC                       (RINPD).

CC

CC**********************************************************************

CC**********************************************************************