- TRAFFIC MODULE

# 2.1	TRAFFIC MODEL DESCRIPTION

The traffic module in the HYROAD model is an enhanced version of the TRAF-NETSIM simulation model developed for the U.S. Federal Highway Administration by KLD Associates.  The traffic module provides a detailed representation of the dynamics of traffic flow over space and time using input data that are readily available to local traffic engineers.  Typical conditions of an suburban and urban environment are represented by the model, including such features as: double parked cars; unloading trucks; buses servicing passengers at stations; pedestrian interference with turning vehicles; and disruptions to traffic flow caused by vehicles pulling in and out of curb parking spaces.  Further, the module is capable of simulating individual vehicle trajectories through key activity states (acceleration, deceleration and idling) that are needed to calculate vehicle emissions on roadways.

The traffic module applies interval-based simulation to describe traffic operations.  This means that every vehicle in the traffic stream is represented as a distinct entity which is “moved” once each second accounting for the current traffic conditions. Vehicle trajectories are computed according to car-following logic which responds to the performance of neighboring vehicles, traffic control devices, and other conditions which influence driver behavior.  Furthermore, each vehicle is identified by category (auto, car pool, truck, bus) and by type.  Up to 16 different types of vehicles, with different operating and performance characteristics (speeds, acceleration capabilities, etc.) may be optionally specified, defining the 4 categories of the vehicle fleet.  Default values of maximum acceleration, maximum speed, headway factor, length of vehicle are included within the module.  In addition, a driver behavioral characteristic (passive, aggressive) is assigned to each vehicle.  Also its kinematic properties (speed, acceleration) are determined, as well as its status (queued, free flowing).  Turn movements are assigned stochiastically, as are its free flow speed, queue discharge headways and other behavioral attributes.  Consequently, each vehicle’s behavior may be simulated in a manner reflecting real world processes.

The traffic module generates important traffic information on vehicle dynamics that are used by the emission and dispersion modules to estimate emissions and their spatial and temporal allocation.  In particular, it provides Vehicle Miles of Travel (VMT) and distributions of vehicle speeds and accelerations by lane and roadway section information to the emission and dispersion modules.  Information on average speed and average queue lengths is also produced by the traffic module.  Vehicle speed estimates are used to produce non-uniform two-dimensional wind and turbulence inputs under light-wind conditions so that air flow and dispersion due to traffic can be simulated.  As such, the modified version of the TRAF-NETSIM traffic module (NETSIM) represents an important tool to predict the impacts of transportation improvements or traffic growth on air quality at the intersection level.

Table 2-1 summarizes the major features of the NETSIM traffic module.  A discussion of several specific capabilities of the model, including its handling of motorist behavior, transportation modes, and traffic control devices is provided here.  For a more detailed discussion of the model, users should consult the TRAF User’s Reference Guide (FHWA, 1995).

Table 2-1.  Major features of the TRAF-NETSIM traffic module.

| Characteristic | Features ( = feature is available) |
| --- | --- |
| Fleet Components | Buses, Carpools, Cars, Trucks |
| Vehicle Occupancy (# of passengers/vehicle) |  |
| Turn movement |  |
| Bus operations: |  |
| Paths |  |
| Flow volumes |  |
| Stations, dwell times and routes |  |
| Trucks |  |
| Carpools |  |
| HOV Lanes (Buses, carpools, or both) |  |
| Queue discharge distribution |  |
| Detailed approach geometry |  |
| Stop and yield signs |  |
| Pretimed signal control |  |
| Single ring actuated control |  |
| Dual ring actuated control |  |
| Number of lanes per approach |  - 7 |
| Incidents and temporary events |  |

## Motorist Behavior

Each simulated vehicle has its own performance capabilities and is “driven” by a simulated motorist exhibiting different behavioral characteristics.  A set of 10 distinct “drivers” is employed in the simulation representing the entire range of driver personalities from highly aggressive to very timid.  The driver’s personality is used in the simulation to make an extensive number of driver decisions including: if, where, and when to change lanes; whether or not to cooperate with a driver attempting a lane change in front; which gap in the oncoming traffic stream is acceptable to make an opposed left turn; whether or not to stop during a yellow signal; and others.  Default values are assigned for all of these values based on typical motorist behavior. The default values are documented in the TRAF User Reference Guide (FHWA, 1995).

## Transportation Modes

Each vehicle is also identified by category (car, high occupancy vehicle (HOV), bus, truck).  For example, buses and HOVs can be restricted to specific lanes.  An individual vehicle is further characterized by type of car, bus, etc. reflecting specific operational and performance characteristics.  A fleet of 16 different “types” of vehicles can be specified to the model.  Default values are provided for each category.

## Traffic Control Devices

Vehicle trajectories are computed according to car-following logic, which responds to the performance of neighboring vehicles, traffic control devices, and other conditions which influence driver behavior.  Actuated signal control and intersection between cars and buses are explicitly modeled.

Each time a vehicle is moved, its position (both lateral and longitudinal) on the link and its relationship to other vehicles nearby are recalculated. Vehicles are moved according to car following logic, response to traffic control devices and response to other demands.  For example, buses must service passengers at bus stops (stations); hence, their movements differ from those of private vehicles.  Congestion can result in queues extending throughout the length of a link and blocking the upstream intersection, thus impeding traffic flow there.  Pedestrian traffic can delay turning vehicles at intersections.

## 2.2	MODIFICATIONS TO TRAF-NETSIM FOR AIR QUALITY MODEL

The selection of microsimulation as the basis for the traffic module was made in part because it could provide additional vehicle behavior information useful for both emissions and dispersion calculations.  These inputs to the emission and dispersion modules are not standard outputs from TRAF-NETSIM, and required the extension of the code to track key variables at appropriate levels of temporal and spatial disaggregation.  Some of these measures are based upon travel within individual roadway segments or “blocks.”  To provide this information, the traffic model was extended to internally subdivide the intersection approach and departure roadways into blocks 10 meters in length and 1 lane in width.  These blocks are automatically constructed by the software from the center of the intersection, outward in each direction, for a distance of up to 1000 meters from the center of the intersection for each lane.  Information gathered for travel within each block is furnished to the emission and dispersion modules for each 15 minutes of simulated time.

Data for each block includes its operating mode distribution, which is a matrix of the speeds and acceleration rates observed for travel within the block.  Speeds were classified into 14 categories of 5 mph increments between 0 and 65 mph while acceleration/deceleration rates were classified into 9 categories of mph/s ranging from less than -7 mph/s to greater than +7 mph/s.  Each cell in this 14  9 matrix identifies the number of vehicle-seconds of travel within the block at a specific speed and acceleration range.  This information is further disaggregated for each signal phase and accumulated separately over each 15-minute period.

For an intersection with: four approach legs and four departing legs each 1000 m long; three lanes; and a traffic controller with four phases; the traffic module sends a data stream to the emission and dispersion modules which includes 1,209,600 values (1000/10 block lengths/approach  (4+4 legs)  (3 lanes)  (4 phases)  (14 speed classifications)  (9 acceleration/deceleration classifications)).  These values quantify the operating mode distribution in vehicle seconds of travel at each speed and acceleration range for every roadway block during each signal phase over every 15-minute period.

To produce this information for the emission and dispersion modules, it was recognized that the NETSIM module computes each vehicle’s position, speed and acceleration rate once each second.  New software was developed to use this information to determine how much time each vehicle spent in each roadway block as well as its average speed and acceleration/deceleration rate while traversing the block.  The simulation also “knows” which signal phase is active during the time that a vehicle traverses a block.  Together, this information is used to accumulate the simulated vehicle-seconds of travel by signal phase, travel speed, and acceleration rate through each block.

Thus, the underlying simulation logic, which computes vehicle movements and traffic signal indications, was not altered in any way. This enables the integrated air quality model to benefit from the extensive validation and application of TRAF-NETSIM over the years.

Other new information provided by NETSIM module for each 15-minute simulated period includes:

- VMT along each intersection approach stratified by speed range in mph: (0, 1-5, 6-10, 11-15, 16-20, 21-25, 26-30, 31-35, 36-40, 41-45, 46-50, 51-55, 56-60, 61-65);

- VMT along each intersection approach stratified by vehicle type: (autos, trucks, carpools/taxis, and buses);

- Average number of queued vehicles in each lane at the start of each signal phase; and

- Average number of vehicles discharging from each approach by movement during each signal phase.

Invariant geometric information input to the NETSIM module is also furnished to the other modules including: the number of approaches; number of lanes on each approach; width and length of each lane; and coordinates of the upstream intersection for each approach.  These coordinates are used to determine the angle of the approach to the intersection.

## Data Transfer from Traffic Module to Emissions and Dispersion Modules

Data produced from the NETSIM module which is needed by the emission and dispersion modules is written every 15 minutes of simulated time on a new file, named LU92.  This file is written by the SUBROUTINE WRTAIR.  At the conclusion of a Traffic simulation run, NETSIM calls the SUBROUTINE RDAIR to read the contents of file LU92 for all 15 minute periods and produce a easily readable table of much of the information contained in the file.  Appendix A in the User’s Guide contains an example of the output file from the NETSIM module simulation.

The data structure for the detailed output file used by the emission and dispersion modules, LU92 is shown in Table 2-2 and Table 2-3.

Table 2-2.  Structure of interface data file, LU92 from traffic module to emissions and dispersion modules.

File Header Record (Record Type 1)

For each 15 minute period:

Period Header Record (Record Type 2)

For each approach and departing link:

Link Header Record (Record Type 3)

For each 10 m section of roadway (starting from center of intersection):

Cell Data Record (Record Type 4)

End

Record of vehicle-miles of travel by speed and also by vehicle type (Record Type 5)

For approach links only and not departing links:

Record of queue by lane at start of each phase (averaged over 15 minutes), (Record Type 6)

Record of left turners discharging by phase (Record Type 7)				Record of through traffic discharging by phase (Record Type 8)				Record of right turners discharging by phase (Record Type 9)

Record of diagonal traffic discharging by phase (Record Type 10)

End

End

Delimiter Record for end of 15 minute section of data (Record Type 11)

End

Table 2-3.  Format of interface data file, LU92 from traffic module to emissions and dispersion modules.

Record Type	  Word	 		Definition

1	ITPCNT	Number of 15 minute periods of data written in file, LU92.

2	JN	Node number specified by user for intersection where air quality data is collected.

ICLOCK	Elapsed time in seconds since beginning of traffic simulation run.

ITOTA	Total number of approach + departure links specified for node, JN.

IMXFZ	Total number of signal phases defined for traffic control at node, JN.

3	IU	Upstream node number of approach/departure link.

ID	Downstream node number of approach/departure link.

INBLK	Length of link, (IU,ID) / 10 m.

IA	Approach/departure link number.

IMXLNE	Number of lanes on link, IA.

4	Array RCELL	Number of vehicle seconds of travel in each cell, IBL, lane, ILN, phase, IFZ, acceleration range, JC, and speed range JV.

(IBL,ILN,IFZ,

JC,JV)

5	RVMAIR(IA,K)	Vehicle miles of travel on approach, IA for each of 14 different speed ranges, K, in mph: (0, 1-5, 6-10, 11-15,16-20, 21-25, 26-30, 31-35, 36-40, 41-45, 46-50, 51-55, 56-60, 51-65).

RVMART(IA, K)	Vehicle miles of travel on approach, IA by vehicle type, K : (autos, trucks, carpools, buses).

6	Array RBGQUE	Average number of vehicles in queue on approach, IA, lane, ILN at start of phase, IFZ during this 15 minute period.

(IA,ILN,IFZ)

7	Array RCMLFT	Average number of left turning vehicles discharging from approach, IA during phase, IFZ for this 15 minute period.

(IA, IFZ)

8	Array RCMTHR	Average number of through vehicles discharging from approach, IA, during phase, IFZ for this 15 minute period.

(IA, IFZ)

9	Array RCMRIT	Average number of right turning vehicles discharging from approach, IA, during phase, IFZ for this 15 minute period.

(IA, IFZ)

10	Array RCMDIG	Average number of diagonal turning discharging vehicles from approach, IA, during phase, IFZ for this 15 minute period.

(IA, IFZ)

11		Five words all set to zero.

Note that all words are INTEGER except those beginning with an R, which are REAL.