Appendix B

GLOSSARY OF TERMS

Glossary of Terms

Accuracy.  A measure of how close a measurement is to the actual value.

ADT.  Average daily traffic volume.

Advection.  Transport of material (e.g. pollutants) by the wind.

Aerometric.  Referring to measurement in the atmosphere (e.g., of wind velocity, sunlight intensity, or pollutant concentration).

Air quality. Ambient pollutant concentrations and their temporal and spatial distributions.

Air quality model.  A mathematical relationship between pollutant emissions and pollutant concentrations.

Air quality monitor.  A device for measuring pollutant concentrations.

Ambient air quality.  The existing state of the atmospheric pollution in an outdoor area.

Ambient monitoring.  Systematic measurements of the characteristics (e.g., pollutant concentration and wind velocity) of the air at a fixed location.

Arterial.  A large, well-traveled street, usually with traffic signals and without limited access.

Atmospheric stability.  The resistance to or enhancement of vertical air movement caused by the vertical temperature profile.

Background concentration.  The pollutant concentration which exists in the vicinity of the highway project from areawide and nearby point sources.  Pollutant concentration in the absence of effects from local anthropogenic emissions.

Cold start.  Starting a motor vehicle with a cold engine.

Cold start emissions.  Emissions emanating from an internal combustion engine during the first 505 seconds of operation.

Dispersion.  The process by which atmospheric pollutants are transported and diluted due to wind and turbulent mixing.

Driving cycle.  A profile of velocity versus time, specified for determining vehicular emissions rates.

Emission factor.  The rate of pollutant emissions under specified conditions.  A number which describes the average emission of pollutants from a set of vehicles in various mixes of operating conditions.

Emissions rate.  The rate at which pollutants are emitted into the atmosphere by one source such as the exhaust system of a vehicle, or a combination of sources.

Eulerian.  Having a coordinate system fixed with respect to the earth’s surface.  Compare Lagrangian.

Federal Test Procedure (FTP).  A detailed procedure prescribed by EPA for certifying the performance of motor vehicles with regard to compliance with federal emission regulations.

Gaussian model.  A pollutant dispersion model based on the Gaussian dispersion equation.  A mathematical simulation of dispersion in which pollutant concentrations are normally distributed in the crosswind and vertical directions.

Inspection and Maintenance (I/M).  An emissions testing and inspection program implemented by states in nonattainment areas to ensure that in-use vehicles are properly maintained to meet emissions standards.

Lagrangian.  Having a coordinate system that moves with respect to the earth’s surface.  Compare Eulerian.

Level of service.  A measure of the level of traffic congestion on a roadway.  There are six levels of service, A through F, with A representing the best operating conditions and F the worst.

Link.  A portion of a road in a highway network.  Usually defined by nodes at each end-point.

Microscale.  Small scale, involving distances up of less than 1 km.  For highway projects, microscale usually refers to the area within 300m of the roadway and 1-5 kilometers up-wind.

Mixed layer.  Layer of air near the ground where turbulent mixing of pollutants occurs.  It is limited by an inversion base, if one is present.

Mixing depth.  The depth of the mixed layer.

Mixing height.  The upper limit of ground-based turbulence in the atmosphere.

Mobile source.  A moving vehicle that emits pollutants.  Such sources include automobiles, trucks, trains, ships, and farm equipment.

MOBILE5.  EPA’s earlier version of the emission factor model.

MOBILE6.  EPA’s most recently released (January 2002) emission factor model.

Modal emission factors.  Vehicular emission factor for individual modes of operation, e.g., accelerate, cruise, decelerate, idle.

Monitoring site.  The location of a measurement device in a monitoring network.

NETSIM.  Network Simulation Model – a traffic microscale simulation model developed for the U.S. Federal Highway Administration.  The model provides a detailed representation of the dynamics of traffic flow at the intersection.

Pasquill stability class (or Pasquill-Gifford Stability Classification).  A standard classification scheme for atmosphere stability.  An indicator of atmospheric conditions or vertical movement of air close to the surface.  Stability class is a function of time of day, insolation, amount of cloud cover, and surface wind speed.

Plume.  The spreading pollutants emitted by a fixed source, such as a smokestack.

ppb.  Parts per billion (109) by volume.

ppm.  A volume measure of air pollutant concentrations.  To convert to a weight measure (ug/m3) of carbon monoxide multiply ppm by

molec. wt.  106

24,500

For example, 50 ppm of carbon monoxide is equivalent to

50(28)(106)	=	5.71  104	g/m3

24,500

or 57.1 milligrams per cubic meter since 1 milligram = 103 micrograms.

ppmC.  Parts per million carbon; ratio of the volume of hydrocarbons to 1 million parts of air that would result if all carbon atoms were in the form of methane.

Receptor.  A designated point location where pollutant levels are examined.

Screening.  A relatively simple analysis technique to determine if a given source is likely to pose a threat to air quality.

Sensitivity analysis.  A test of a model using different input and parameters to discover what effects changes in their values have on model predictions.

Simulation model.  A mathematical description of real physical and/or chemical process.  The responses of the model to input variations are analogous to those of the real processes.

Stability.  A property of the atmosphere which determines the amount of vertical mixing.

Stable layer.  A layer of air in which very little mixing takes place.

Standard deviation.  The square root of the variance.  The standard deviation is a measure of the spread in a set of measurement of a single parameter.

Surface layer.  The layer of air near the ground, generally 1 to 100 meters deep, where surface features (e.g., trees, buildings) affect atmospheric turbulence and diffusion.

Surface roughness.  The characteristic height of obstructions in the path of the wind near the surface, such as the height of trees and buildings.

Tracer.  A substance used to follow the path of a physical, chemical, or biological process.

Transport.  Movement of pollutants or other material in the atmosphere from one location to another.