Appendix B

Particle Size Information for Selected HAPs

Appendix B

Particle Size Information for Selected HAPs

The fine particle mass is the percentage of the total mass for particles with diameters smaller than the cutoff value for each experiment.  The recommended values (shown in boldface) rely mostly on the observations made in urban or suburban areas.  The recommended values for mass median diameter (Dmm) are derived primarily with equation (28) rather than the observational values of Dmm reported.  These observations were made near sources or source regions but were not made immediately downwind of identifiable point sources.

| Particulate component | Fine particle mass (%) | Source of mass distribution information | Location, season | Dmm (m) | Source of Dmm information | Location, season (for Dmm information) |
| --- | --- | --- | --- | --- | --- | --- |
| Antimony, Sb, compounds | 50a 77b 60c 60 | Rizzio et al. 1999 Keeler et al. 1988 Paciga and Jervis 1976 | rural-residential Italy, winter rural Pennsylvania, summer Toronto, averaged | 1.2 0.7 1.0 | Paciga and Jervis 1976 Gatz 1975 | Toronto, averaged assumed for Lake Michigan |
| Arsenic, As, compounds | 80a 89d 69d 63c 75 | Rizzio et al. 1999 Pinto et al. 1998 Pinto et al. 1998 Paciga and Jervis,1976 | rural-residential Italy, winter Teplice, Czech R., winter Teplice, Czech R., summer Toronto, averaged | 1.5 0.7 0.5 | Paciga and Jervis,1976 Gatz 1975 | Toronto, averaged assumed for Lake Michigan |
| Asbestos | 75-95e 85 | OECD 1989 | urban areas, mostly in Canada | 0.3 |  |  |
| Beryllium, Be, compunds | (a) 80 (b) 0 | (a) assumed for combustion of coal and other fossil fuels (b) assumed for dust particles from mining |  | (a) 0.4 (b) 7.0 |  |  |
| Cadmium, Cd, compounds | 65a 72f 77g 65h 70 | Rizzio et al. 1999 Keeler et al. 1995 McDonald and Duncan 1979 Lee et al. 1972 | rural-residential Italy, winter Detroit, Michigan, spring Glasgow, U. K., summer St. Louis, MO, spring | 0.5 0.6 1.5 3.1 0.9 0.6 | Mészáros et al., 1997 McDonald and Duncan 1979 Lee et al. 1972 Lee et al.,1968 Gatz, 1975, from Harrison et al. 1971 | Hungary Glasgow, U. K., summer St. Louis, MO, spring Cincinnati, OH, summer Ann Arbor, MI |
| Calcium cyanamide, CCaN2 | 0 | (assumed to be in granular form for application to soil as herbicide) |  | 7.0 |  |  |

| Particulate component | Fine particle mass (%) | Source of mass distribution information | Location, season | Dmm (m) | Source of Dmm information | Location, season (for Dmm information) |
| --- | --- | --- | --- | --- | --- | --- |
| Chloride, Cl-, compounds | 72a 88d 55d 29h 55c 65h 60 | Rizzio et al. 1999 Pinto et al. 1998 Pinto et al. 1998 Orsini et al. 1977 Paciga and Jervis,1976 Lee and Patterson 1969 | rural-residential Italy, winter Teplice, Czech R., winter Teplice, Czech R., summer St. Louis, MO, winter Toronto, averaged three U.S. cities, summer | 0.6 0.85 2.5 1.0 | Paciga and Jervis,1976 Lee and Patterson, 1969 Gatz 1975 | Toronto, averaged three U.S. cities, summer assumed for Lake Michigan |
| Chromium, Cr, compounds | 50a 65d 68d 27a 56a 54g 68c 55 | Rizzio et al. 1999 Pinto et al. 1998 Pinto et al. 1998 Pirrone et al. 1995b Pirrone et al. 1995b McDonald and Duncan 1979 Paciga and Jervis,1976 | rural-residential Italy, winter Teplice, Czech R., winter Teplice, Czech R., summer Chicago, IL, summer Grand Haven, MI, summer Glasgow, U. K., summer Toronto, averaged | 0.5 1.4 1.3 1.5 1.0 1.2 | Mészáros et al., 1997 McDonald and Duncan ,1979 Paciga and Jervis 1976 Lee et al.,1968 Gatz 1975 | Hungary Glasgow, U. K., summer Toronto, averaged Cincinnati, OH, summer assumed for Lake Michigan |
| Cobalt, Co, compounds | 67a 81c 75 | Rizzio et al. 1999 Paciga and Jervis 1976 | rural-residential Italy, winter Toronto, averaged | 4.0 1.0 0.5 | Paciga and Jervis,1976 Gatz 1975 | Toronto, averaged assumed for Lake Michigan |
| Coke Oven Emissions | 80 | (assumed for combustion processes; emissions are actually a mixture of compounds and forms) |  | 0.4 |  |  |
| Cyanide, CN-, salts (does not consider HCN, which is present mostly as gas) | 0 | (assumed to be in granular form) |  | 7.0 |  |  |
| Hydrochloric acid, HCl | n/a | (a gas) |  |  |  |  |
| Hydrofluoric acid, HF | n/a | (a gas) |  |  |  |  |
| Glycol ethers | n/a | (primarily in gaseous form in the atmosphere) |  |  |  |  |

| Particulate component | Fine particle mass (%) | Source of mass distribution information | Location, season | Dmm (m) | Source of Dmm information | Location, season (for Dmm information) |
| --- | --- | --- | --- | --- | --- | --- |
| Lead, Pb, compounds | 69a 93d 88d 75b 57b 77b 86h 71g 81h 61h 60h 51c 59-90h 75 | Rizzio et al. 1999 Pinto et al. 1998 Pinto et al. 1998 Keeler et al. 1988 Pirrone et al. 1995b Pirrone et al. 1995b Caruso et al.,1981 McDonald and Duncan,1979 Orsini et al. 1977 Orsini et al. 1977 Orsini et al. 1977 Paciga and Jervis,1976 Lee et al. 1972 | rural-residential Italy, winter Teplice, Czech R., winter Teplice, Czech R., summer rural Pennsylvania, summer Chicago, IL, summer Grand Haven, MI, summer Milan, Italy, averaged Glasgow, U. K., summer St. Louis, MO, winter St. Louis, MO, summer Tallahassee, FL Toronto, averaged six U.S. cities, averaged | 0.5 0.7 0.36-1.43 0.47 0.5 | McDonald and Duncan,1979 Paciga and Jervis,1976 Lee et al. 1972 Gatz 1975 | Glasgow, U. K., summer Toronto, averaged six U.S. cities, averaged assumed for Lake Michigan |
| Manganese, Mn, compounds | 48a 54d 45d 28b 28b 53g 65h 33-66h 45 | Rizzio et al. 1999 Pinto et al. 1998 Pinto et al. 1998 Pirrone et al. 1995b Pirrone et al. 1995b McDonald and Duncan,1979 Orsini et al. 1977 Lee et al. 1972 | rural-residential Italy, winter Teplice, Czech R., winter Teplice, Czech R., summer Chicago, IL, summer Grand Haven, MI, summer Glasgow, U. K., summer St. Louis, MO, winter six U.S. cities, averaged | 1.4 1.4-2.43 1.3 1.8 | McDonald and Duncan,1979 Lee et al. 1972 Gatz 1975 | Glasgow, U. K., summer six U.S. cities, averaged assumed for Lake Michigan |
| Mercury, Hg, compounds (does not consider gaseous Hg, which is predominant form) | 42a 70b 70b 88f 80 | Rizzio et al. 1999 Ames et al. 1998 Pirrone et al., 1996 Keeler et al. 1995 | rural-residential Italy, winter upstate New York Chicago, IL, summer Detroit, MI, spring | 0.8 0.4 | Keeler et al. 1995 | Detroit, MI |
| Fine mineral fibers | 100 | (all particles assumed to be less than 1.0 _m in diameter) |  | 0.1 |  |  |

| Particulate component | Fine particle mass (%) | Source of mass distribution information | Location, season | Dmm (m) | Source of Dmm information | Location, season (for Dmm information) |
| --- | --- | --- | --- | --- | --- | --- |
| Nickel, Ni, compounds | 67a 35b 39b 88g 55c 59-82h 60 | Rizzio et al. 1999 Pirrone et al. 1995b Pirrone et al. 1995b McDonald and Duncan 1979 Paciga and Jervis 1976 Lee et al. 1972 | rural-residential Italy, winter Chicago, IL, summer Grand Haven, MI, summer Glasgow, U. K., summer Toronto, averaged six U.S. cities, averaged | 0.6 1.2 0.83-1.67 1.0 | McDonald and Duncan 1979 Paciga and Jervis 1976 Lee et al. 1972 | Glasgow, U. K., summer Toronto, averaged six U.S. cities, averaged |
| Phosphorus, P (assumed to be phosphates) | 40-65i 35h 40 | Delumyea and Petel 1979 Lee and Patterson 1969 | southern Lake Huron two 2 U.S. cities, spring | 3.8 2.2 | Lee and Patterson, 1969 | two U.S. cities, spring |
| Polycyclic organic matter (POM) | 80 | (assumed to result primarily from combustion processes; actually a mixture of compounds and forms) |  | 0.4 |  |  |
| Polycyclic organic compound example: PCCD/Fs (polychorinated dibenzo-p-dioxins and dibenzofurans) | 89-92j 90 | Kaupp and McLachlan 1999 | Bavaria, Germany | 0.1 |  |  |
| Polycyclic organic compounds example: PAHs (polycyclic aromatic hydrocarbons) | 88-97j 93 | Kaupp and McLachlan 1999 | Bavaria, Germany | 0.1 |  |  |
| Radionuclides (including radon) | 80 | (assumed to result from combustion processes or originat as daughters of radioactive elements; actually a mixture of compounds and forms) |  | 0.4 |  |  |

| Particulate component | Fine particle mass (%) | Source of mass distribution information | Location, season | Dmm (m) | Source of Dmm information | Location, season (for Dmm information) |
| --- | --- | --- | --- | --- | --- | --- |
| Selenium, Se, compounds | 72a 99b 99b 85b 80 | Rizzio et al. 1999 Pirrone et al. 1995b Pirrone et al. 1995b Keeler et al. 1988 | rural-residential Italy, winter Chicago, IL, summer Grand Haven, MI, summer rural Pennsylvania, summer | 3.9 0.4 | Gatz 1975 | assumed for Lake Michigan |
| Titanium tetrachloride, TiCl4 | n/a | (primarily in gaseous form in the atmosphere) |  |  |  |  |

aPercentage of mass for particles with diameters less than 1.1 m, corresponding to the  alveolar respirable fraction, relative to the total mass of the substance in particles with diameters smaller than 9 m.  The measurements by Rizzio et al. (1999) were all taken about 70 km north of Milan, Italy, during the winter.  The fraction of the relatively fine particles would be expected to be larger during the summer.  Because manganese is found in crustal materials, it was felt that the fine fraction observed in Rizzio et al. (1999) was lower than that likely associated with anthropogenic sources.

bPercentage of mass for particles below a cutpoint diameter of 2.5 m measured with a dichotomous sampler, relative to the total mass of particles with diameters less than 10 m.

cPercentage  for particles with aerodynamic diameters smaller than 1.1 m, relative to the total mass as obtained with a high-volume sampler.

dPercentage of mass for particles below a cutpoint diameter of 2.5m measured with a modified dichotomous sampler, relative to the total mass of particles with diameters less than 8m.

eDefined as percentage of asbestos fibers with length less than 5 m, but aerodynamic properties affecting deposition are not necessarily those characteristic of more spherically shaped particles

fPercentage of mass for particles below a cutpoint diameter of 2.5 m, relative to the mass of total suspended particulate matter.

gPercentage of mass for particles with aerodynamic diameters less than about 0.7 m, relative to mass of particles with diameters less than 17 m, as measured with a four-stage casade impactor.

hPercentage of mass for particles with aerodynamic diameters less than about 2.0m,  relative to the total mass sampled.

iPercentage of mass for particles with aerodynamic diameters less than about 1.6m,  relative to the total mass sampled.

jPercentage of mass for particles with aerodynamic diameters less than about 1.35 m,  relative to the total mass sampled.