DRAFT

- SCIENTIFIC PEER-REVIEW OF THE REPORT

“DEPOSITION PARAMETERIZATION FOR THE

INDUSTRIAL SOURCE COMPLEX (ISC3)”

- Authors:

| Dr. Chis Walcek Dr. Leiming Zhang Dr. Jeremy Hales - Dr. William J. Massman | Dr. Gary Stensland Mr. Ho-Chin Huang Mr. Clyde Sweet - Mr. Allen Williams |
| --- | --- |
| - Mr. James Dicke, Report Writer |  |

August 27, 2001

This Report was Compiled and Submitted by:

Veronica A. Hanzel

The KEVRIC Company, Inc.

Durham, NC

Under USEPA Contract no. 68-D-98-092

Work Assignment No. 3-01, Dated April 5, 2001

PREFACE

The U.S. Environmental Protection Agency, through their contractor, commissioned a panel of eight reviewers to conduct a scientific peer review of the report “Deposition Parameterization for the Industrial source Complex (ISC3) Model”.  This report summarizes updates and improvements recommended for algorithms used to describe dry and wet deposition in the ISC3 model.   The reviewers were asked to determine the scientific validity of the assumptions, limitations, data and methods described in the report.    The team of reviewers included Dr. Chis Walcek, Dr. Leiming Zhang, Dr. Jeremy Hales, Dr. William J. Massman, Dr. Gary Stensland, Mr. Ho-Chun Huang, Mr. Clyde Sweet, and Mr. Allen Williams.

This report presents the primary conclusions and comments resulting from the peer review as summarized by the report writer, Mr. James Dicke.   The original “Charge to Peer Reviewers”, which outlines EPA’s intended focus for the review, is attached as Appendix A.

A detailed overview of the mechanics of this peer review process is presented as Appendix B.  The qualifications of each member of the peer review team are presented via copies of their respective resumes in Appendix H.

The KEVRIC Company Inc. provided the administrative management necessary to conduct this peer review.   KEVRIC’s efforts are outlined in the overview of the process included as Appendix B.

- TABLE OF CONTENTS

- Introduction	1

2.0  	Activities of the Peer Review Team	1

3.0	Consensus Responses to the USEPA “Charge to Reviewers”	2

3.1	Dry Deposition of Gases and Particulates	2

3.2	Wet Deposition of Gases and Particulates	3

4.0	Summary	4

5.0	Acknowledgements	5

Appendix A – Charge to Peer-Reviewers	A-1

Appendix B – Overview of the Peer Review Process	B-1

Appendix C –	Final Reviewer Comment Report – Dr. Jeremy Hales	C-1

Appendix D –	Final Reviewer Comment Report – ISWS Group - Dr. Gary Stensland,

Mr. Ho-Chun Huang, Mr. Clyde Sweet, and Mr. Allen Williams	D-1

Appendix E –	Final Reviewer Comment Report – Dr. William Massman	E-1

Appendix F –	Final Reviewer Comment Report – Dr. Chris Walcek	F-1

Appendix G –	Final Reviewer Comment Report – Dr. Leiming Zhang	G-1

Appendix H – Qualifications of the Peer Review Panel Members	H-1

Appendix I– Documents made available to Peer-Reviewers	I-1

1.0	INTRODUCTION

The U. S. Environmental Protection Agency (EPA) and the U. S. Department of Energy co-sponsored work by M. L. Wesely, P. V. Doskey and J. D. Shannon, members of the Environmental Research Division, Argonne National Laboratory (ANL), to develop updates and improvements to the algorithms used to describe dry and wet deposition of gases and particles in the Industrial Source Complex model, version 3, system (ISC3).  In May 2001 these researchers submitted a report to EPA entitled “Deposition Parameterizations for the Industrial Source Complex (ISC) Model” and slightly revised it in June; hereafter it is referred to as the ANL Report.  EPA contractors coded the ANL Report updates and improvements in the ISC3 computer program, and provided model outputs for test cases and sensitivity analyses to demonstrate the appropriateness of the ANL approaches.

Also in May, EPA began an effort to obtain peer review comments on the ANL Report and the results from the updated ISC3 model.  A Peer Review Team and Report Writer were assembled and given a Charge to review and submit comments on the Model Formulation, Implementation, and Documentation.   Resumes of the Peer Review Team Members and the Report Writer and EPA “Charge to Reviewers” are contained in Appendices D and A, respectively.  The reader should note that the team of four reviewers from the Illinois State Water Survey (ISWS) submitted one, collective report.  At the request of the EPA Project Officer, this Final Peer Review Report reflects a consensus response of the Peer Review Team.   In addition, an individual comment report was submitted by each of the peer reviewers.  These reports are included with this summary report as Appendices C-1 through C-5.

- ACTIVITIES OF THE PEER REVIEW TEAM

The Team held its first conference call on May 30, 2001 to review the purpose, scope and overall process of the peer review project, as well as to answer questions and discuss the schedule for accomplishing its tasks.  Members had received the ANL Report earlier, but also wanted to review the updated ISC3 code, together with results of example scenarios and sensitivity tests.  A copy of the code was provided a week later.  Results of the various model runs were provided by EPA and its contractors several times during the review period, often in response to coding changes/errors identified by Team Members.  In addition, some Team Members also made calculations based on the ANL approaches and shared them with the rest of the Team.

By early July each Team Member had submitted either a draft report review or substantial technical comments.  This led to the second conference call on July 17, 2001 during which progress, comments, remaining work and an adjusted schedule for completing the project were discussed.  Since there were many questions and concerns about the model results, and each member had not received copies of all the other members’ reports/comments, it appeared premature to draw conclusions, although most members expressed strong reservations about the output from the wet deposition scenarios.   More test runs were requested – and provided later.  In-depth exchange of comments between Team members was strongly encouraged throughout the review process.

The third conference call was held on August 6, 2001.  One objective was to canvass the Team regarding consensus on the appropriateness of the dry and wet deposition approaches, since all had received the latest ISC3 model runs, as well as each other’s comments.  Most members expressed some support for the dry deposition modeling approaches, but several concerns were also raised.  All members had significant reservations about the wet deposition approaches for both gases and particles, and some believed coding errors might still be present.  The Report Writer asked each Team Member to submit his final report within two weeks, containing the response to the EPA Charge and additional comments, so that the final report could be drafted, circulated for comment, revised as necessary, and submitted to the EPA Project Officer through the contractor for this project, The KEVRIC Company, Inc.

- CONSENSUS RESPONSE TO THE EPA CHARGE TO REVIEWERS

The Peer Review Team’s consensus response to the EPA Charge is presented in two sub-sections: a) the dry deposition of gases and particles, and b) the wet deposition of gases and particles.  The Team Members often found themselves having to reconcile the use of rather sophisticated, technical deposition approaches in the steady-state, Gaussian-based plume model, ISC.  The Members expressed concern that while ISC3 short-term estimates of many hazardous air pollutant (HAP) concentrations in air are not likely to be reduced by dry or wet deposition mechanisms, the estimates of material so deposited have large uncertainties associated with them.  These uncertainties are not very likely to be considered by model users or decision-makers, who are increasingly interested in HAP deposition, because the discussion in the ANL Report is quite limited in this regard.  A frequent comment was for more discussion of uncertainty in input values and the ISC3 model estimates.

As noted above, the individual Team Member reports are in the Appendices.

  - Dry Deposition of Gases and Particles

    - Model Formulation

The Team Members generally agreed that the dry deposition approaches were definite improvements over the current methods in ISC3, and probably represented the state-of-the-science techniques that are appropriate for use in ISC3.  Further improvements in some areas were suggested, but these would involve some development effort and should be weighed against the inherent limitations of ISC3.  All Team Members expressed the need to improve the text that describes the approaches, including clarifications, expanded definition of terms and list of symbols, consistency of usage, etc.  Many assumptions and limitations, e.g., uncertainty, as discussed above, of the approaches also need expanded coverage.

The Team Members discussed the physicochemical properties in the ANL Appendices at some length, and generally agreed that the values compiled by Sander on his Internet web site represent the state-of-the-science values.  One Team Member submitted a stomatal resistance model for consideration as a replacement for the scaling method/factor LAIr.  Another identified problems with the parameterization of kinematic viscosity, , and the coefficients of molecular diffusion, Dv , and Da ; recommended improvements/solutions were provided.

    - Implementation

The Team Members agreed that the new algorithms in ISC3 for dry deposition of gases and particles appear consistent with the ANL Report, with a few reservations.  Some suggestions were provided on how the ANL authors could verify the correct values for soil resistance for SO2 (RgS) and aerodynamic resistance (Ra), for example.  Several Team Members offered suggestions for “improving” the code.  Numerous coding errors were identified and corrected during the review process.  However, final responsibility for the code rests with the authors of the ANL Report and EPA.

    - Documentation

All the Team Members provided comments on the way that model features were presented in the ANL Report, both in the main text and the Appendices.  Numerous items were cited as unclear or in need of more discussion.  For example, references should be provided for Equations 17, 18, and 19.  Values for Gr should be provided for other surfaces as is done for z0.  Several suggestions for improving the documentation could be termed “regulatory guidance”, and thus will be considered by the EPA Project Officer.  Indeed, it was not clear how the ANL Report would be used (e.g. as a chapter in the ISC3 Users Guide or as a stand-alone document, etc).  Team Members urge that the authors of the ANL Report carefully address the comments in their reports.

  - Wet Deposition of Gases and Particles

    - Model Formulation

The Team Members did not agree that the ANL approaches for wet deposition of gases and particles represented appropriate technical methods for use in ISC3.  Specifically, the formulation for gaseous wet deposition for highly soluble HAPs was seen as seriously flawed by some Members.  Both spot-checking answers and the results of ISC3 test runs indicated the gas and particle fluxes are several orders of magnitude too high.  However, unidentified coding errors may be responsible for these large discrepancies.

As possible remedies to the shortcomings, one Team Member submitted an alternative approach for the treatment of wet deposition of gases, but this would require additional work to complete.  A second Team Member offered detailed suggestions that, if implemented, would markedly increase the theoretical rigor of the ANL approach, decrease the use of some highly approximate improvisations, and improve the model’s gaseous wet scavenging predictive capabilities.  For wet scavenging of particles, the same Team Member recommended specific steps for improving the treatment in the ANL Report.  A third Team Member offered a specific reference that might contain a better scheme for particle washout coefficients, Wp.

Since the ISC3 model outputs of HAP deposition are becoming increasingly important from a regulatory standpoint, it is essential that the Team Members’ comments on modeling wet deposition be addressed.

    - Implementation

Team Members are concerned that the ISC3 model outputs for wet deposition test scenarios do not match expected values.  Since such model results were not expected, coding errors may still be the problem.  However, in any case, implementing the detailed suggestions noted above would bring the ANL Report’s wet deposition approach more in line with the state-of-the-art.

    - Documentation

Examples of the many comments submitted include:  One Team Member provided very specific comments on improving/correcting the text concerning washout/scavenging ratios and coefficients on page 18 of the ANL Report.  Another Team Member also offered specific comments and recommendations for improving the text in Sections 4 and 5 of the ANL Report.  Finally, an annotated copy of many of the pages in the ANL Report, with a variety of suggestions, recommendations and corrections, was also submitted.

- SUMMARY

A Peer Review Team comprised of eight noted scientists accepted a Charge from EPA to review the ANL Report “Deposition Parameterizations for the Industrial Source Complex (ISC3) Model”.  They provided extensive comments on the model formulation, implementation and documentation presented in the ANL Report.  The ANL approaches were coded into the ISC3 computer program. The Team Members used ISC3 model output from test scenarios and sensitivity analyses provided by EPA in their reviews, as well.

A Report Writer, who was not a Peer Reviewer, was tasked to prepare a Final Report for the EPA Project Officer that represented the consensus of the Team Members.  The consensus reached is that the ANL recommendation for dry deposition of gases and particles represents a reasonable state-of-the-art approach, given the inherent limitations of the ISC3 plume model.  Team Members submitted numerous suggestions for improving the ANL approach.  In the case of the ANL recommendation for wet deposition of gases and particles, the consensus reached is that the ANL approach is flawed and does not provide credible answers.  This may be the result of unidentified coding errors.  Team Members submitted very specific suggestions and recommendations for either correcting the ANL wet deposition approach or developing a new approach.

The Final Reports submitted by the Peer Reviewers are contained in Appendices C.1 through C-5.

5.0	ACKNOWLEDGMENTS

Each of the eight Team Members and the Report Writer contributed significantly to the Peer Review and Final Report for this project.  It will be abundantly clear to all readers of the individual comment reports that many hours of thoughtful, dedicated effort went into their comprehensive, insightful comments, and that the reviewers are a true, peer group of technical experts on atmospheric deposition processes.  Their deep concern that this project should result in a real advance in the capability of the regulatory, and regulated community to adequately describe/model the environmental deposition of HAPs emissions is also quite evident in their responses to the EPA Charge.

- Appendix A

- Charge to Peer-Reviewers

- CHARGE TO PEER REVIEWERS

EPA is revising the Industrial Source Complex Version 3 model system to incorporate the recommendations being described in the document “Deposition Parameterization for the Industrial Source Complex (ISC3) Model.”  The purpose of this work is to seek a scientific peer review of the assumptions, limitations, data, and methods described in this study.

- Model Formulation

1.1	Do the methods described for estimating dry and wet deposition represent the state-of-the-science for air toxic pollutants, given the observational data typically available for ISC3 applicants?  Are assumptions and limitations adequately described?

1.2	Are the particle size information and the physiochemical properties and gas-half lifetime data (shown in Appendix B-F of the report) based on sound scientific principles?

1.3	If you disagree with the methods, assumptions or data input, please provide specific alternative approaches.

- Implementation

  - Are the algorithms implementing the model formulation in the ISC3 model consistent with the report?

- Documentation

  - Is the current organization of the report adequate?

  - Is the presentation of the model features clear?  Please note any sections that are unclear or confusing.

- Appendix B

- Overview of the Peer Review Process

- OVERVIEW OF THE PEER REVIEW PROCESS

for the

- Peer Review of the Report

- “Deposition Parameterization for the Industrial Source Complex (ISC3) Model”

The US Environmental Protection Agency, Office of Air Quality Planning & Standards, Air Quality Modeling Group, in an effort to provide for independent, credible peer reviews of air dispersion models and studies retained a contractor, The KEVRIC Company Inc. of Silver Spring MD/Durham, NC to manage and coordinate the peer review project.

The peer review was handled as a work assignment under a general contract with the KEVRIC Company.  Once the work assignment was issued, KEVRIC prepared a detailed work plan for the approval of the Work Assignment Manager (WAM).  For the ISC3 Report project, the WAM was Mr. Joe Touma.

The work plan provided a description of the tasks to be completed, the estimated time frame and estimated manhours/cost requirements.  The description below describes the process by which the peer review  was conducted.

The information provided to KEVRIC  under the Work Assignment Statement of Work included;

- a “Charge to Reviewers”, formulated by EPA, that outline the specific direction and technical scope of the task for the peer review team.

- a list of qualified candidates, as known to EPA

- a list of materials to be provided to the peer reviewers by KEVRIC/EPA

Mr. Joe Touma made the initial contact of the qualified peer reviewer candidates, described the project to them and requested their participation based on their interest and availability.  This list was turned over to KEVRIC and consequently, eight (8) peer review team members were retained (Dr. Chis Walcek, Dr. Leiming Zhang, Mr. Jeremy Hales, Dr. William J. Massman, Dr. Gary Stensland, Mr. Ho-Chin Huang, Mr. Clyde Sweet, and Mr. Allen William).  In addition, Mr. James Dicke consented to compile a report that would summarize all of the peer reviewers comments and opinions into one concise report.  This peer review team was approved by the acting WAM.

KEVRIC provided a sub-contract mechanism for the peer reviewers to be compensated for their time.  It was estimated that each peer reviewer would spend up to 40 hours on the review and Mr. Dicke would spend 40 hours compiling a report.

KEVRIC then arranged for the review materials to be distributed to each reviewer.  The materials provided to be reviewed were those provided to KEVRIC by USEPA as provided under the work assignment.  These materials were as follows:

- The report titled “Deposition Parameterization for the Industrial Source Complex (ISC3) Model”, dated May 2001, prepared by Argonnie National Laboratory, Environmental Research Division.

- Computer code generated and provided by Roger Brode of PES.

Once these materials were distributed, KEVRIC scheduled a teleconference call involving the team members, V. Hanzel of KEVRIC, and Joe Touma of EPA.  The conference call was conducted to discuss the charge to the reviewers and establish commonality in the peer reviewer’s efforts and to initiate the peer review.

The peer reviewers were instructed to perform their review according to the “Charge to Reviewers”.  It was agreed that contact amongst them was permissible and encouraged.  Several subsequent teleconferences were held among the reviewer team.  Several recommendations for changes to code were submitted and code was redistributed with test cases.  Draft comment reports were forwarded to V. Hanzel and J. Dicke and it was decided that these reports should be shared among the team.  KEVRIC arranged for copies of all comment reports/correspondences be distributed to the team.

After the reviewers provided final comment report, a draft report was compiled by Mr. Dicke that provided a summary of the opinions of the team and specific individual comment, as appropriate.  This report was distributed via email to all parties for their review.

A conference call was scheduled by KEVRIC to discuss the draft report and determine if any changes, modifications or clarifications were needed.  Mr. Dicke revised the draft report and redistributed the final version to the team members.

The final version of the report was compiled, reproduced and submitted to EPA by KEVRIC.  This compilation included addition of other documentation such as resumes of the reviewers and this overview of the process.

Appendix C

Individual Peer Reviewer Final Report

J. M. Hales

August, 2001

Review of ISC3 Deposition Parameterizations

J. M. Hales

June, 2001

# Introduction

This review is in response to a request by the EPA to comment on a draft report by Wesely, Doskey, and Shannon describing deposition parameterizations for ISC3, a multisource model based on Gaussian plumes.  Guidelines for the review were specified in a Charge to Reviewers.  The comments and suggestions given here are summarized immediately below in the context of specific questions of the Charge, and subsequently discussed in detail in the topical order of the report.

# The Charge

1.  Model Formulation

1.1	Do the methods described for estimating dry and wet deposition represent the state-of-the-science for toxic air pollutants, given the observational data typically available for ISC3 applicants?

This is generally true for dry-deposition parameterizations, which provide a good balance between the current state of understanding and the inherent limitations of ISC.  In my opinion this is not true for the wet-deposition parameterizations, which I recommend upgrading as discussed below.

Are Assumptions and limitations adequately described?

Generally, yes; although I have several minor suggestions for improvement in the general text.  The exception is the wet-deposition material, which I feel needs substantial clarification in addition to technical revision.

1.2	Are the particle size information and the physicochemical properties and the gas half-lifetime data based on sound scientific principles?

In general yes, although some of the material needs revision to better describe the assumptions and trade-offs that are necessitated by the inherent linearity of the ISC.  Related suggestions appear below

1.3 	Suggested alternative approaches

As noted above and below, I recommend alternative approaches in the wet-deposition sections.

2.  Implementation

2.1	Are the algorithms implementing the model formulation consistent with the report?

Something is drastically wrong with the wet-deposition algorithms, which produce fluxes for gases and particles that are several orders of magnitude too high - at least for the points that I've spot-checked.  Although I feel that the derivational basis for these calculations could be improved, the authors' approach should provide answers that are in the right ballpark most of the time.  This suggests that a coding error is responsible for these large discrepancies.

3.  Documentation

3.1	Is the current organization of the report adequate?

Yes.

# 3.2	Is the presentation of the model features clear?

# In general, yes; but please see following detailed comments

# Sections 1 - 3:  Introduction and Dry-Deposition of Gases and Particles

These sections are written by individuals who are world-recognized for their competence in parameterizing dry-deposition processes, and the product reflects this expertise.  Other than this the only general comment I have is that their approach is essentially unidirectional, in the sense that the deposition parameters and submodels described here tend to be exposure-independent and tacitly assume irreversible capture at the surface; at least they don't explicitly provide for re-emission of deposited materials to the atmosphere.  This is somewhat problematic in the case of HAPs in the sense that one could easily envision a situation where prolonged exposure to a particular gaseous or semi-volatile HAP could result in buildup on or within the receptor substrate, with a corresponding tendency to re-emit to the atmosphere.  This, plus the notorious "grasshopper effect" suggests certain limitations to the approach.

By mentioning this I am not suggesting that the authors revise their approach:  the one they have chosen certainly represents the appropriate state of knowledge that is applicable in the context of the ISC.  But I do suggest that the authors put in an introductory paragraph that addresses this point and cautions the user about related limitations in the model.

Other than that I have only a few specific suggestions to improve clarity of the presentation.  These are itemized immediately below.

1.  I'd suggest clarifying the discussion of the various heights following equation (2c) somewhat.  There are a lot of heights mentioned here and the text gets confusing.  In particular, z isn't particularly well defined.  To the uninitiated reader it's unclear whether this is a reference height or a continuous variable.

2.  Paragraph following equation (2):  What Rb should be used in non fibrous/even situations?

3.  Figure 1:  Suggest changing the resistance label from "Aerodynamic to ground" to "Within-canopy aerodynamic to ground."  Otherwise it tends to get confused with the above-canopy aerodynamic resistance.

4.  Page 1, first paragraph:  You really mean that you're scaling the conductances linearly with the LAI, don't you?

5.  Section 2.2.2.2, first paragraph, 4th line:  Suggest explicitly defining r at this point.

6.  Page 13, 3rd paragraph, Line 7:  A couple of typos here with the sulfur oxides.

7.  Equation (22):  Should Rb be Rbi?

# Sections 4 and 5:  Wet Deposition of Gases and Particles

In contrast to the dry-deposition description where my comments were mainly editorial, I have some more substantive suggestions for improvement in these wet-deposition sections.  First (and editorially speaking) the authors should check their terminology regarding washout/scavenging coefficients and ratios. In their text they tend to reverse standard usage. (I note that the authors for the scavenging section for the previous ISC document have reversed standard usage as well.  This is probably what induced the current authors into this situation.)  It's OK to use "washout" and "scavenging" interchangeably in this context; but the terms "coefficient" and "ratio" have distinct meanings in the wet-removal literature and should not be confused; i.e.,

(dimensionless),

while the washout/scavenging coefficient (units of reciprocal time) is defined in terms of the local removal rate and local concentration:

In essence this is a first-order rate coefficient, and Slinn refers to corresponding values simply as "rates" in his article.

Thus "coefficient" on line 2, par 1, p 18 should be changed to "ratio."  Similarly for lines 3, 5, and 7, par 2, p 18. Similarly for the definition of Wg on p 18.  Then just above equation ( 27) the scavenging coefficient is referred to as the scavenging ratio.  Similar reversals of terminology are made in Section 5 as well.

The last line in par 1, page 18 is incorrect, or at least misleading.  Scavenging coefficients (using the terminology suggested here) are used extensively in the literature to describe below-cloud scavenging of particulate matter, and scavenging/washout ratios have been applied in several analyses of in-cloud processes. The real reason that scavenging ratios tend to be applicable for gases in below-cloud situations is that falling raindrops tend toward solubility equilibrium with extended exposure times to the pollutant gas.

While there is nothing particularly incorrect about the authors' treatment of gas scavenging, it could be improved markedly to increase theoretical rigor, to decrease the use of some highly approximate improvisations (e.g. the application of equation (26)), and to improve the model's predictive capabilities.  The treatment suggested below should increase the execution time of ISC3 somewhat, but not substantially.

First, I'd recommend using the Wg approach described by the authors for a defined subset of washout circumstances only, and with the following changes:

1.  Use the surface concentration of pollutant in air, rather than equation (26) in the application of the washout ratio.

2.  Use the Wg approach only for conditions where solubility equilibrium is guaranteed.

Conditions for this "equilibrium-scavenging" condition can be determined by a dimensionless criterion, as follows:

In the particular case of a Gaussian plume this dimensionless group can be expressed as1

(1)

where

= overall mass-transfer coefficient				(2)

kx = liquid-phase mass-transfer coefficient

ky = gas-phase mass-transfer coefficient

H = the Henry's law coefficient

vz = raindrop velocity (negative downward)

a = raindrop radius

h = emission height

z = vertical plume-spread parameter

The mass-transfer coefficients can be estimated using established methods.1,2,3  As a practical matter it is appropriate to apply equation (1) for the largest drop size of interest in the rain spectrum being considered, and follow the equilibrium-scavenging approach whenever Nes>50 (this is somewhat conservative, and could be relaxed somewhat; but this point is relatively unimportant since the more rigorous procedure described immediately below results in equilibrium-scavenging values as Nes becomes large).

Taking phenol [moderately large solubility (low H)] as an example and choosing a upper-limit drop radius of 0.5 mm, this criterion suggests that surface releases must attain a plume spread of the order of one hundred meters, or at least several tens of meters, to approach equilibrium scavenging conditions.  Conditions for elevated releases will of course depend on emission height as well, as indicated in equation (1).

Second, for conditions where equilibrium scavenging conditions are not met, I'd suggest applying the established (linear) equations for gas scavenging through Gaussian plumes that exist in the literature.4  These take the form

(3)

where

c(a) = concentration of pollutant in raindrops of radius a at ground level.

F = plume depletion factor.

u = wind speed.

y = horizontal plume-spread parameter.

Equation (2) can be applied directly, and because of linearity its results are additive for overlapping plumes.  Under some conditions the error functions cause computational precision problems, but a simpler, asymptotic form of (3) is available for cases in which such conditions prevail.  For practical purposes, I'd recommend performing this calculation for a single representative drop size (the mass-mean drop size is appropriate).  More accurate representations could be obtained by distributing over the raindrop spectrum, but this probably isn't warranted in view of the additional complexity and computer time involved.

In summary of this discussion of gas scavenging, the approach suggested above should be computationally tractable, and is well-matched to Gaussian models such as ISC.  It certainly should result in enhanced predictive capability of ISC3.

With regard to particle scavenging (Section 5), note the earlier comments regarding terminology.  Also, the first paragraph in this section is misleading.  The rationale for application of scavenging ratios using the Slinn approach described here is totally different from that corresponding to gases.

Again, there is nothing particularly incorrect with the approach to scavenging calculations described here, but the treatment can be improved substantially.  In order to construct his parameterization in terms of washout ratios, Slinn has made simplifying assumptions that are not necessary (and not particularly appropriate) to Gaussian models such as ISC, resulting in more of a back-of-the-envelope estimate than a direct calculation.  I suggest that particle scavenging be calculated more directly on the basis of scavenging coefficients, as follows:

1.  Note that the cumulative washout effect of an ensemble of raindrops falling vertically through a Gaussian plume is simply

where

cb = concentration of pollutant in bulk precipitation collected at ground level.

 = the washout coefficient.

u = wind speed.

J = precipitation rate

2.  Use this equation in conjunction with appropriately estimated scavenging coefficients to calculate particle washout.  Slinn's approximations for the scavenging coefficient are probably fine for this situation; but the authors may wish to consider alternative forms which do not contain so many embedded assumptions.3,5

Section 6:  Physicochemical Properties

Equations (32) and (33) are appropriate for calculating diffusivities to the accuracy needed for ISC simulations.  I suggest indicating units for Da and Dw at this point in the text.

There are several blank entries in the physical property tables, and I recommend the authors explain, in the text, the reasons for this missing information.

In the context of Section 6.5 it is worth noting that wet-removal calculations for highly soluble substances such as HCl will not present any particular problem if the gas-scavenging approach recommended above is used; thus some of the discussion in this section can be eliminated.

Finally, the discussion of atmospheric half-life in Section 6.6 should point out that the half-lives discussed here are those associated with chemical reaction only, and that the actual atmospheric residence time of the pollutant depends on its removal by wet and dry processes as well.  Moreover it is probably worthwhile mentioning here that the half-life concept is strictly applicable only for linear systems and that equation (38) represents a rather gross linearization of some highly nonlinear reactions.  Of course this is necessary for linear models such as ISC; but it wouldn't hurt to remind the reader of all this at this point.

Conclusions

In summary, I think the authors have done a nice job in most areas of this effort, although the scavenging calculations definitely need attention.  The technical changes recommended above should be relatively easy to implement and will, I believe, add to the general computational proficiency of the ISC3.

References

1.  Atm. Envi. 6, 635-659 (1972)

2.  J. Atm. Chem. 1, 269-289 (1984)

3.  Seinfeld, J. H. and S. N. Pandis (1998)  Atmospheric Chemistry and Physics.  Wiley, New York

4.  AIChE Journal 19, 292-297 (1973)

5.  Atm. Envi. 10, 45-50 (1976)

Appendix D

Individual Peer Reviewer Final Report

ISWS Group

Dr. Gary Stensland

Mr. Ho-Chun Huang

Mr. Clyde Sweet

Mr. Alan Williams

The Review Report of ISWS Group (Stensland, Ho-Chun, Sweet and Williams) on the ISC3 Report by the Argonne Group

SUMMARY

Our specific suggestions for improvements are presented on the first four attached pages as well as on the approximately 20 scanned pages (see below) from the report, which have annotations (questions, comments, or suggestions).  The comments on the first four pages are grouped into the areas of Model Formulation, Documentation, and Implementation of the Code.  Our effort and comments focused most heavily on the Implementation and Documentation aspects of the report, and less on the Model Formulation.

Model Formulation---Our five comments under Model Formulation raise issues about the amount of uncertainty in certain situations or for certain kinds of HAPS.  Although it is difficult to discuss uncertainty in a way that becomes available and relevant to each ISC model user, we feel it should get more attention that it has in the current report.  Perhaps a specific section should be devoted to this issue.  In this section we would suggest that the authors discuss uncertainly related to some specific issues, perhaps including some of those that we have raised.  Our comment on uncertainly of H values for certain HAPS (comment (3)(i)) could explicitly be addressed by adding an additional column in Appendix D to flag those HAPS where the uncertainty is expected to be particularly large.

Implementation---Considerable effort was spent to check the algorithms in the model formulation (the Fortran code) against the equations and text in the report. Specific errors were noted.  We have not checked on whether these errors have all been fixed.

Documentation---We feel that the current organization of the report is probably adequate. It is somewhat unclear exactly how this report will be disseminated, that is, will it become an addition to the current ISC user documents or is it to be a standalone document and will another user document be prepared?  It seems that some text should tie this current report to the user documents currently available.

Is the presentation of the model features clear?  We think improvement is needed here and hope that our specific suggestions will assist the authors in making improvements.

## I. Comments on Model Formulation:

- EPA has recommended that the ISC code can be applied out to distances of 50 km.  In going from a source to a receptor at 50 km, the roughness length could change drastically.  The report should explain explicitly how that is handled in the ISC code, e.g. is some type of average Zo assigned?

- Discussion of potential bias because in-cloud scavenging (rainout) is not included in ISC:  For short distances, a few km, it is a good assumption to neglect in-cloud scavenging.  However when ISC is applied to large distances, near 50 km, it would seem that considerable in-cloud scavenging might occur and thus ISC wet deposition may be significantly underestimated.  This limitation should probably be discussed in the Argonne report.

- Other Uncertainty Discussion:  In three areas we think the uncertainties of key parameters in some of the equations may be much higher (a factor of 10 or more) than would be assumed by most users.  Such uncertainties ought to be flagged for certain HAPs in Tables in the ISC report, so that users are aware of them.			(i) Hoff (1994) published an “error budget” for atmospheric deposition of toxic chemicals to the Great Lakes.  He concluded that for PCB 28 the error in the estimate of deposition was controlled by high uncertainty (factor of 5) in the estimate of the Henry’s Law coefficient (H).  It seems likely that there are similar or greater uncertainties in the H values for some of the dioxins, dibenzofurans, Aroclors, and perhaps some of the pesticides and POMs.						(ii) We question the use of the ISC procedures with the Aroclors.  Aroclors are mixtures of dozens of individual PCB congeners with a very wide range of physical-chemical properties.  The research community has pretty much gone to analysis of individual congeners when calculating atmospheric deposition of PCBs.		(iii) There is considerably more uncertainty in the proper value to use for the dry particle deposition velocity than is indicated in the Argonne ISC report.  Much depends on the distribution between fine and coarse particles, which is very poorly known for many chemicals.  We are surprised to see no reference to the work of Holsen and others (for example: Franz et al., 1998) on sampling and deposition of large particles (>10 m).  It seems that these would be very important in near-source deposition of particulate pollutants.

References:

T.P. Franz, S.J. Eisenreich, T.M. Holsen, 1998.  Dry deposition of particulate polychlorinated biphenyls and polycyclic aromatic hydrocarbons to Lake Michigan.  Environ. Sci. & Technol. 32: 36181-3688.

R.M. Hoff, 1994.  An error budget for the determination of atmospheric mass loadings of toxic chemicals in the Great Lakes.  J. Great Lakes Res. 20:229-239.

## II. Comments on Documentation

- Appendix A:  It is useful to summarize the Equations.  To assist the reader in going back to the main text for more explanation it would be useful to have the equations in the Appendix also show their numbers as they appeared in the main text.  It would also be useful to repeat Fig. 1 at the beginning of Appendix A.

- Page 4,  The current Ra computation in the computer code is equation (2b) in section 2.1 without the third term in large parentheses.  It is different from the description on page 4 or page A-4 for Ra in unstable conditions.  We are also confused as to what is the correct equation, since it is not clearly stated in section 2.1.  However, from appendix A, the summary seems to suggest that equation (2c) is used in new ISC3, but it is inconsistent with the current code.  Please revise the text to clearly state the equation implemented in code.

- Page 4,  Calculating Ra:  The last paragraph on page 4 gives two checks that can be made on the Zo calculated by the preprocessor and if agreement is not within about a factor of two, the text indicates that u* should be recalculated.  One is a check against Table 2 and the other is a check against a rule of thumb. However this is not what the current ISC code does.  For the gas case the code uses Table 2 to get a value for Zo, calculates u*, and uses this value to calculate Ra.  For the particulate case the code uses the meteorological preprocessor value of Zo to compute u*.  The text (including Appendix. A) should be modified to reflect what is actually coded.

- Page 4,  We suggest that the values of Zr for different land use types be listed in the report.  Please also make it clear that whether Zr for gas and particle be different or not.  If Zr are different for both cases, please also list the values of Zr for particle.

- Page 9-10,  The definition of f2 is confusing.  First, in page 9, a simple linear relationship of f2=qr is recommended for ISC.  Then, in the first paragraph below equation (9) of page 10 “Then f2 is estimated from ….qr=Wh-1/200”.  The reader might assume f2 =Wh-1/200. However, we believe that f2(t)=qr(t)=W(t)/200. and W(t)=F(W(t-1), P(t-1), f2(t-1), T(t)).  We suggest that a time step index be applied on f2 in equation (9) as well as in the text, e.g., f2h and f2h-1.

- Page 12,  Equation (12) is not consistent with equation (15) on page A-6.  Equation 15 in appendix A has an additional temperature correction term Rx.

- Page 16,  Should Rbi be used in equation (22) instead of Rb?

- Page A-4,  What is the definition of “Rsm” below equation (8) and where does it apply?

- Page A-7,  We suggest that equation (22) on page 16 be included in the description of Method 1.

## III. Comments on implementation of code

Subroutines VDP, VDP_TOX, PDEP, PDEPC, and PDEPG have been examined for any discrepancy between the implementation of the computer code and procedures described in the ISC report.  Several errors in the current code and inconsistencies between report and computer code were found.  Because the report did not clearly state the usage of some equations or provide the values of needed data, the correct implementation of the code can’t be determined.  The following is the list of our findings.  They are reported by the subroutine name along with the reference line number of file “calc1.for”.

# Subroutine VDP

‘Non-toxic’ case is not reviewed because it does not cover the scope of the report.

Subroutine VDP_TOX

Line 3236:	‘0.06’ should be ‘0.6’ for Z0tab(5,3) as listed in Table 2.

Line 3246:	‘200’ should be ‘2000’ for Restab(6,2,1) as listed in Table 3.

Line 3352-3356:	Zrdep is computed as “Z-0.67h” for the aerodynamic resistance (Ra) computation. The “h” values for various land use categories are not listed in the report.  We recommend that the average height of vegetation (h) used for various land use categories be listed in the report.  According to the current code, the land use category 9 (non-forested wetlands) actually has the same “h” as that for ‘urban land’, i.e., 6m.  Is it reasonable?

Line 3364:	In the current code Zrdep was set as 1 m for all land use categories for the particle Ra computation.  We do not find an explanation in the text to justify this setting.  Why should the Zrdep of particles be different from that in gas case?

Line 3455-3456:	This is also related to comment (2) of section II (see above).  The current Ra computation is equation (2b) in section 2.1 without the third term in large parenthesis.  If the correct form is different from the current code, it should be modified accordingly.

Line 3485:	Equation (11) on page 10 uses 298.0, not 298.2.  It may be different because the ISC3 code always treats absolute temperature = 273.2 K. However, we can’t locate the original source of this formulation (Dickinson, R. E., Modeling evaporation for three dimensional global climate models, in Climate Processes and Climate Sensitivity, Geophys. Monogr., 29, 58-72, 1984) and thus can not verify that the number of 298.0 is obtained on the basis of the absolute temperature = 273.0 K.

Line 3503-3506:	When the surface is wet, Rgs should also be set as 50 sm-1 (equation 15, page 13).  Add line Rgs = 50. after the line “Rcox = 0.75*Rco”.

Line 3507-3510:	The current code resets Ra =1000 sm-1 if Ra < 1000 sm-1 and the surface is wetted by dew.  We do not find an explanation in the text to justify this resetting of Ra.

Line 3516-3517:	According to page14 of the report, Rcs, Rco, Rgs (wet or dry) and Rgo should be individually corrected with the addition of the temperature correction term 1.e03*exp(-(Ta-269.2)).  Then corrected values are used in Rm, Rcut, and Rg computations (equations 17-19).  In the current code Rcs and Rco (Rcox) additions are executed, however, the additions of Rgs and Rgo are missing.  Furthermore, the temperature correction term is added to Rg after the computation using equation (19), which is a wrong implementation. The computation of

Rgs = Rgs + 1.e03*exp(-(Ta-269.2)) and

Rgo = Rgo + 1.e03*exp(-(Ta-269.2))

should be added after the line “Rcox = Rcox+1.e03*exp(-(Ta-269.2))”.  Then remove line 3557, “Rg = Rg+1.e03*exp(-(Ta-269.2))”.

Line 3548:	This is also related to comment (4) of section II (see above).  The current code follows the form of equation (12) in section 2.2.3.3.  If equation (15) in appendix A is the correct form then the current code needs to be changed by adding the term Rx.

Line 3600:	Additional term, ‘vdphor’, was found in the computation of ‘vdep(i)’.  It is different from that described in equation (22) of section 3 (page16).  Please clarify this difference.

Subroutine PDEP

Line 3982:	The minimum depth for wet deposition of particles is 500 m as stated in section 5 (page 21).  In the current code it is 100 m.  (Note 100 m is the minimum depth set for wet deposition of gas)

Subroutine PDEPC

Line 4066:	The minimum depth for wet deposition of particles is 500 m as stated in section 5 (page 21).  In the current code it is 100 m.

Appendix E

Individual Peer Reviewer Final Report

Dr. William Massman

July, 2001

Scientific peer review comments on

Deposition parameterization for the Industrial Source Complex (ISC3) Model

authors:  ML Wesely, PV Doskey, and JD Shannon

for the USEPA (Jawad S. Touma) and the Kevric Company (Veronica Hanzel)

by

Bill Massman

July 2, 2001

I have reviewed both the model code and the supporting document authored by Wesely  et al. and I find that there are some improvements that can be made to the deposition parameterization of the ISC3 Model.  Below I list my thoughts and comments on how this can be done.  I also enclose 4 manuscripts that I use to support my recommended changes.  Part I of my comments refers to the report by Wesely  et al. and Part II to the deposition module of the ISC3 Model.

Part

Review comments on

Deposition parameterization for the Industrial Source Complex (ISC3) Model

by ML Wesely, PV Doskey, and JD Shannon

p.4 discussion about the reference height $z_r$} - In general $z_r = z - d$ where $d$ is the displacement height.  For natural vegetated areas it can be assumed to be $\frac{2}{3} h $, where $h$ is the height of the vegetation.  For urban areas it is taken to be 4 m.  For flat areas like water and deserts $d = 0$ m is probably appropriate.  But because the parameterization document is not specific enough on this point, the code is not necessarily consistent with it.  As implemented in the code $d$ is 4 m for the urban land use types 1, 5, 6, and the nonurban types 3 and 9.  It is assumed to be $\frac{2}{3}$ (1 m) for land use types 2 and 8 and $\frac{2}{3}$ (10 m) for land use type 4.  For land use type 7 it is $10z_{0m_{loc}}$.  I suspect that $d$ is incorrect or unreasonable for land use types 3, 7, 8, and 9.  I would suggest that the deposition module should include a look up table of default values for $d$ as it has for $z_0$.

parameterizations of kinematic viscosity, $\nu$, and coefficients of molecular diffusion, $D_v$ and $D_a$ (in air);

pp.5, 9, and, Sections  6.1 and  6.5, Tables 5-7, Appendices A, C and E - Much of the emphasis on the temperature and pressure dependency of $\nu$, $D_v$, and $D_a$ is unnecessary.  All that is important are the ratios: $\frac{\nu}{D_a}$ (Eqn 4) and $\frac{D_v}{D_a}$ (Eqn 7).  Near STP all temperature and pressure dependencies should cancel and the ratios should be constant for any given pair of gases.  Here I include two papers (Massman 1998 and 1999A) as templates and references for the evaluation these ratios.  I also note: (a) that the present formulations for $\nu$ [p.5 and A-3, Eqn (4b)] are not necessarily consistent with one another, nor is the value of 0.1505(10$^{-4}$) m$^2$ s$^{-1}$ correct at STP, (b) the value for $D_{Hg^0}$ [Table 7] is too low by almost a factor of 2, and (c) that the model of Fuller et al. (1966, 1969) consistently under predicts the true value of $D_a$ for CO, CO$_2$, N$_2$O, NH$_3$, H$_2$O and SO$_2$.  This under prediction, however, is potentially serious only for N$_2$O, but this consistent model bias does give cause for some concern about applying the model of Fuller et al. universally.  I recommend: (1) where possible follow Massman (1998) and determine $D_a$ from available data, (2) check Perry and Green (1984: Perry's Chemical Engineers' Handbook, pp. 3-285-286) for other information relevant to the model of Fuller et al., (3) check Yaws (1995: Handbook of Transport Property Data, for a detailed listing of the coefficients of molecular diffusion of gases in air and water for all organic compounds of concern to ISC3, and (4) considering using an approach like Makar et al. (1998: Journal of Geophysical Research, 103, 13,095-13,110 for evaluating the diffusion volumes, $\Sigma_v$, of gases. There is no particular reason why all these quantities, and $\frac{\nu}{D_a}$ and $\frac{D_v}{D_a}$ in particular, cannot be precomputed and provided directly as a look up table in VDP\_TOX or somewhere else in ISC3.

Resistance circuitry for VDP\_TOX, Figure 1, p.6 - The simplification of the old RADM deposition module to VDP\_TOX is a good idea and I certainly support the decision to do so.

\ Solar Irradiance function, $f_1$, Eqn. 8, p.9 - Here I am primarily confirming that it is intended that the stomata should remain open at night.  At $G = 0$, $f_1 = 0.01$.  I do not find fault with this because the stomata of many plant species do not close at night (e.g., Musselman and Minnick 2000: Atmospheric Environment, 719-733).  This last reference could help in selecting better values for the two numerical parameters (0.01 and 1) of $f_1$.

p.11, paragraph beginning after {\bf 2.2.3.1 Solubility in Water}, second to last line} - There appears to be a contradiction in this line, viz., `units of moles-liter per atmosphere (atm M$^{-1}$)'.   The units are the inverse of one another.

p.12, first complete paragraph, lines 4-6} - There appears to be a contradiction in these two sentences, viz., `the leaf could become saturated, driving $R_{cl}$ to considerably larger values than estimated with Equation (12).  To avoid highly unrealistic situations, a minimum value of $R_{cl}$ = 100 s m$^{-1}$ is assumed for all substances.'  If the goal is to avoid unrealistically large resistances why is a minimum value assigned?

p.12, Table 4} - Replace `Factor' in table heading with `S Factor'.

p.14, discussion of length of time canopy remains wet after rain} - It is assumed that the canopy remains wet for 2 hours after a rainfall event ceases.  How realistic is this assumption?  My experience (in Cascade Mountains of Oregon) suggests that some trees can remain wet and dripping for several hours or days after a rainfall.  It is possible to improve on this parameterization, but to do so requires a sophisticated surface physics package for the meteorological model.  I have very little information on the meteorological preprocessor, MPRM.

Some general thoughts on parameterizing the bulk boundary layer resistance, $R_b$, p.5, - I think the current formulation of $R_b$ can be improved upon, but it would require some development and investigation before hand.  There has been a significant amount of work on parameterizations of $R_b$ for heat transfer especially by the remote sensing and large scale modeling communities.  The object of much of this effort has been to improve the ability to infer and predict the land surface energy balance using remotely sensed data.  Here I enclose 2 more papers (Massman 1999B and Su et al. 2001, In Press) that are potentially relevant to introducing some of the most recent developments in this area to the dry deposition community.  Specifically I would like to point out Eqn (24) of the first paper and Eqn (12) of the second as improvements for $R_b = 2.2(\nu/D_a)^{2/3}(ku_*)^{-1}$.  (Note: the relation ship between $kB^{-1}$ and $R_b$ is $R_b = kB^{-1}(ku_*)^{-1}$.)  For application to ISC3 it may be necessary to further simplify these newer models, which is why I think some development effort may be required before implementing a new model for $R_b$.  In fact, the simplest improvement might be to assign a value of $kB^{-1}$ to each land use type within VDP\_TOX.  This should be possible because $R_b$ (or more appropriately $kB^{-1}$) varies with foliage type, percent foliage cover, leaf size, wind speed, LAI, and other parameters.  Conceptually there is no reason why some of this variation in $R_b$ could not be included in ISC3 or VDP\_TOX.  This improved parameterization is most important for gases and meteorological situations for which stomatal uptake is relatively unimportant or is not occuring.

Part II}

Review comments on

Subroutine VDP\_TOX

by RW Brode

Comment line above Stab data statement - I think the variable being initialized is solubility not stability.

lines converting Pressure and Temperature to correct units- I think the documentation should indicate that pressure is being converted from mb to kPa.

comment lines beginning with `Assign surface roughness, stability,...'- The word `stability' should read `solubility'.  See {\bf 10.} above.

line defining rLAI - Would it not be better coding to use `rLAI = sqrt(F)' rather than `rLAI = F**0.5'?

lines defining Zrdep - I think the adjustments to Zrdep for the displacement height are potentially incorrect.  See {\bf 1.} above.

line defining Ustarr - To calculate the natural logarithm of Zref/Z0m\_loc requires the Fortran function `alog' rather than `log' does it not?

The calculation of Net Radiation (RNET) - This portion of the code could be improved.  I suggest defining a variable, CLDCV, for cloud cover amount.  At present the code is hard wired for 0.5.  Also define the Bowen ratio as a variable, BOWEN.  I think the line defining the heat flux, HFLUX, should read `HFLUX = BOWEN*RNET/(1.0+BOWEN)'.

lines defining f1 using if test on Ri - I think the coding would be improved if this line read `if (Ri .gt. 0.9e07) f1 = 0.1' rather than the present test for equality between two real numbers.

line: if ((ustarr.lt.(a/dq).and.....ppp.gt.0.) - Does the Fortran complier make a distinction between `Ustarr' and `ustarr'?  Is it more appropriate to compare the MPRM estimate of $u_*$ with $a/dq$ rather that the locally adjusted value of $u_*$? Why does the time of day (`(ihour.lt.8).or.(ihour.gt.19)') matter in this and other statements?

the first line of code defining Rg - I believe the current formulation has a factor of `HENRY(ISRC)' missing.  This line should read

`Rg = 1./((1.e-3/HENRY(ISRC)*Rgs))+((fo+(0.1*fo*fo)/HENRY(ISRC))/Rgo))'.

See Eqn (16) page A6 of the model documentation by Wesely, Doskey and Shannon.

variable names not defined in the code} - SFCP (surface pressure), Rco (ozone cuticular resistance), Z0m, vairms, tstop(i), st, vgrav(i), and vdphor.  There may be others that I did not catch.

Appendix F

Individual Peer Reviewer Final Report

Dr. Chris Walcek

July, 2001

ATMOSPHERIC SCIENCES RESEARCH CENTER

State University of New York

# 251 Fuller Rd.  •  Albany, NY 12203 • Telephone: (518)437-8720  Fax: (518)437-8758

E-mail:  walcek@asrc.cestm.albany.edu

Jawed (Joe) S. Touma	August 21, 2001

Emissions, Monitoring and Analysis Division (MD-14)

Office of Air Quality Planning and Standards

U. S. Environmental Protection Agency

Research Triangle Park, NC 27711

Dear Joe,

Enclosed you will find my final review of the report: “Deposition Parameterizations for the Industrial Source Complex (ISC3) Model…” by Wesely et al.

The report contains some ambiguous portions where additional information should be presented, and there are several probable areas where technically significant errors are made. However, it is highly unlikely that for many applications airborne concentrations will be appreciably modulated by the details of the dry deposition process, which I assume is the primary output of the ISC3. If ISC3 is being used to quantitatively assess dry deposition rates from particular sources, then these errors could be appreciable.

I thank you for the opportunity to review this work. Please feel free to contact me with additional questions or comments on this review.

Thank you,

Chris Walcek

Enclosures:	4 pages of detailed comments

CC: Veronica Hanzel

Peer Review for the report
“Deposition Parameterizations for the Industrial Source Complex (ISC3) Model…” by Wesely et al.

Summary: The authors of this report have done a good job of upgrading and expanding the treatment of dry deposition within ISC. On first reading, the report seems somewhat carelessly prepared, but this doesn’t adversely affect the technical content. The following list of negative comments must be viewed in a positive light, as I think dry deposition effects on plume concentration calculations must be extremely small most of the time, so even gross errors or simplifications in the treatment of dry deposition should have a minor impact on calculated concentrations.

The report contains some ambiguous portions where additional information should be presented, and there are several probable areas where technically significant errors are made. However, it is highly unlikely that for many applications airborne concentrations will be appreciably modulated by the details of the dry deposition process, which I assume is the primary output of the ISC3. If ISC3 is being used to quantitatively assess dry deposition rates from particular sources, then these errors could be appreciable.

1. Model formulation. Do the methods described for estimating dry and wet deposition represent stat-of-the-science for toxic air pollutants?

In terms of MODEL FORMULATION, I find that the treatment of gas and particle dry deposition, as well as particle wet deposition represents a reasonable description of the state of the science, given the limitations of the ISC3 modeling input and formulation basis. However, the formulation of gaseous wet deposition is grossly flawed for highly soluble HAPs where the current formulation probably severely overestimates wet fluxes. Suggestions are presented below for more appropriate and general treatments for wet deposition.

1.1 Model formulation. Are assumptions and limitations adequately described?

Generally the assumptions are described, although in many cases somewhat vaguely. This could be corrected with minor descriptive adjustments described below

1.2 Are the particle size information and the physicochemical properties and the gas half-lifetime data based on sound scientific principles?

The only area where I find some discrepancies with are the specification of Henry’s Law solubilities. The current use of Chemical handbooks for solubility and vapor pressure appears conceptually flawed, and should only be used as last resort if “real” Henry’s law solubilities have not been measured. A comprehensive review of Henry’s law measurements are noted below.

2. Implementation. Are the algorithms implementing the model formulation consistent with the report?

For a couple of HAPs crudely “hand checked” from the figures of model output provided, I find no evidence that there are gross discrepancies between formulations and coded implementations. However, these cursory “quality checks” are limited in their ability to detect minor discrepancies and inconsistencies.

3. Documentation. Is the current organization of the report adequate? Is the presentation of the model features clear?

Generally, the answer to both of these questions is true. However, please note the following suggestions for improvements to the various sections of the report. The following enumerated comments are my listing of ambiguous areas in need of further explanation, or provide suggestions for improved presentation.

1) Ambiguous area never explained: Just how are gaseous dry deposition velocities used when calculating concentrations and deposition in ISC3? I recall from use of ISC nearly 20 years ago that deposition velocities were used as “settling” rates where the plume centerline was allowed to drift downward towards the surface. The “downward displacement” of the plume centerline was simply vdx/u where x was a downwind distance from the source. I thought deposition was the accumulated product of the deposition velocity times the surface concentration. Maybe I have an incorrect interpretation of how deposition velocities were used in Gaussian plume models.

There needs to be a section showing how deposition velocities are used for calculating concentrations and deposition accumulations in the underlying Gaussian plume equation. Maybe this is already presented elsewhere in ISC documentation not provided to reviewers of this report?

2) pg 4, calculation of aerodynamic resistance: How is u* calculated for use in ISC3?

3) pg 4, calculation of Ra: This description of aerodynamic resistance applies to turbulent transfer of momentum. It is well recognized that the aerodynamic resistance for passive tracers (pollutants, heat, water vapor) can under many conditions be appreciably different than the momentum resistance. I was expecting to see a factor of “0.74” somewhere in the definition of Ra.

4) pg 4, zr: This reference height is alluded to throughout the technical description, but never defined. Just what value is used for zr. Deposition velocities as well as dry fluxes are probably sensitive to the choice of zr? What height are deposition velocities calculated and why?

5) pg 6, 3rd line from the bottom is confusing: Does Rac = Raci ÷ (0.3 u*) or does Rac = 0.3 Raci ÷ u*  ?

6) Minor point about Table 3: There are a lot of “9999” listings, and in the table heading there is an instruction to “use 107”. For clarity, why not just list “>107” wherever there is a 9999 in the table?

7) pg 8, specification of “F”. It appears that LAIr is a function of land surface type and season. The description says to do one thing in autumn and spring (SEASONAL CATEGORIES), and also tells what to do for wooded areas (SURFACE CATEGORIES 4-6) and “other vegetative surfaces (Which categories are these “other”?). How is LAIr specified for other surface categories (e. g. range, urban) and seasons (summer, winter)?

8) pg 9, bottom: specification of evaporation: The report describes a clever prognostic parameterization of soil moisture, accounting for precipitation and evaporation. I would expect evaporation from a surface to be proportional to RELATIVE HUMIDITY *AND* temperature, with fluctuations in relative humidity producing huge variations in evaporation. Why was the Rh effect ignored? It seems like if you are going through such an elaborate algorithm, that the consideration of Rh would be appropriate. Apparently Rh data is available to ISC3 as moisture deficit q is used in Eq. (16) for assessing dew.

9) pg 9-10, description of f2. The wording here is unclear. It looks like f2 = r = wh-1/200, but this is not obvious from the wording. If this is the case, this should be stated rather than making the vague statement “f2 is estimated from …”.

10) pg 11, 1st paragraph of “Reactivity” section, 4th line: What is a “half reduction-oxidation reaction”? My chemistry is a little weak, but I thought reactions changed oxidation states by integer oxidation numbers.

11) pg 18, description of scavenging: For gases that partition predominantly into the aqueous phase (e. g. HNO3), H is essentially zero, and therefore according to Eq. (25) fluxes become infinite, which is clearly wrong. The washout described in this section uses what has been referred to as an “open system” assumption. In other words, if there is 1 ppb of HNO3 it is assumed that water present with the 1ppb is at equilibrium with 1 ppb remaining in the gas phase. However, a more appropriate assumption is a “closed system” where HNO3 dissolving in the raindrops is DEPLETED from the 1 ppb, and equilibrium is established with respect to a much smaller gas-phase concentration, but the TOTAL HNO3  remains at 1 ppb (most/all of that 1ppb is in the aqueous phase for highly soluble gases)

Using simple mass balance (total=gas+aqueous), an “effective H” (H’) accounting for this “closed system” depletion of gases can be simply written as 

	H’ = H(1+LRT/H/104)

where L is the “water content” of the rain [according to Kessler microphysics = (r/18.35)8/9 ], R is the ideal gas constant (0.082 molar atm-1) and T is the temperature. This expression for H’ should be used wherever “H” is used in this report. For low solubility gases, (high H), H = H’, but for high solubility gases (H  0), H’ can become appreciably greater than H.

For highly soluble gases, there is a natural upper limitation to the rate of depletion.

12) Another factor in the discussion of solubility: Simple Henry’s law solubilities don’t tell the entire story of just how soluble many atmospheric species are. For example, HCHO partitions rapidly in the aqueous phase among two species: a simple dissolved HCHO and an aquated complex H2C(OH)2 and under many conditions the aquated form is the dominant product determining the total solubility. Other compounds dissociate into weak acids where the ionic forms of the dissolved pollutants are the dominant. There should be a statement describing this “effective” solubility being composed of Henry’s law PLUS additional dissociations occurring in the aqueous phase.

13) pg 20, Eq. (29): hw used in this expression SHOULD be conceptually identical to the zt used for describing gas scavenging. In deriving Eq (29), hw  is DEFINED as the depth of the layer that is experiencing scavenging. It is inappropriate to use a fixed height here.

14) pg 21, definition of Wp: the use of a constant Wp = 104 is inappropriate since there is no consideration of the depth of the atmosphere experiencing scavenging. (zt or hw). Obviously if rain is falling through twice as thick a layer, there should be twice the amount of scavenged material. It would be better to use some assumed “efficiencies” the produce “reasonable” Wp  and retain the use of Eq. (29). Even here, I don’t believe the “measurements” of below-cloud scavenging are hardly adequate.

15) pg 21, Eq. 31: According to Eq. 31, Dmm is negative for all fine particle fractions x<1?? Is this a typo?

16) pg 21, 3rd paragraph from bottom beginning with “Implementation of equation…”: If Eq. (29) (using zt rather than hw- see note 11 above) is used to quantify wet removal (as it should be, although see next point), it doesn’t make any difference what the vertical limits to the integration are for defining the concentration. Each falling raindrop “sees” and scavenges the vertical integral of the concentration.

17) pg 21, 2nd paragraph from bottom beginning with “Near emissions sources…”: It is stated that Eq. (29) produces excessive wet deposition fluxes. First of all, in line 5 of pg 21 it is stated that Wp=104, and therefore apparently Eq. (29) is not used (if Eq. 29 were used with reasonable Eo, there would be no “excessively large” wet deposition!). Most importantly, there is no scientific justification for arbitrarily lowering the scavenging rate “near” the plume source. The justification that “relatively little time would exist” for particle removal is complete gibberish. The time it takes a particle to impact with a falling drop is the time it takes the particle to run into the drop. The mechanism for particle uptake by falling rain (impaction scavenging) is physically different from the diffusive uptake of gases where the “uptake times” can be appreciable.

18) The last paragraph of an addendum to pg 21 beginning with “Washout can…”. Here there is just sloppy presentation. It is stated that dry deposition calculations are not performed if the plume hasn’t extended downward to the height were dry deposition velocities are calculated. As noted above in point (4), this height is never defined, and just where is the “edge” of a smoothly varying Gaussian plume? Why is this technical detail describing DRY DEPOSITION in the section on WET DEPOSITION OF PARTICLES? This detail needs to be in a NEW section describing technical details of how ALL of both the dry and wet deposition parameters are implemented into ISC.

19) pg. 22, Eq. (32). Please define Da and specify its units following the equation.

20) Last paragraph on pg 22: The definitions of Henry’s law constants are TOTALLY INCORRECT. Henry’s law constants are NOT the chemical’s vapor pressure (partial pressure over a PURE SOLUTION of a liquid or even solid HAP) divided by its solubility (equilibrium aqueous-phase concentration of a liquid or solid HAP in water (no gas-phase concentration involved in the definition of solubility). Henry’s law constants are DEFINED as the equilibrium partial pressure of a HAP divided by the aqueous phase concentration of the same HAP dissolved in water. Solubility as recorded in standard textbooks has physically NOTHING to do with Henry’s law constants.

Just as an example, I looked up the Henry’s law constant for Acrolein in Snider and Dawson (1985 JGR, vol p3797) who report a value of about Kh=10 mol per liter per atmosphere, which in the units used in this study corresponds to H= 101325 Pa per atm ÷ 1000 liters per m3 ÷ Kh = 10 Pa m3 mol-1, which is about a factor of 10 lower than the vapor pressure for acrolein divided by its aqueous solubility (98.4). It appears that for many compounds, the ratio of solubility produces a reasonably approximation of the true Henry’s Law solubility. However, primary references should be made to actual measurements of H, and not solubility and vapor pressure. The methods presented here are a good “fall back” approximation.

As a suggestion, numerous Henry’s law constants are listed in an excellent compilation:

http://www.mpch-mainz.mpg.de/~sander/res/henry.html

Unfortunately, there are probably numerous HAP where Henry’s law constants have never been measured. For these compounds, I would think that Raoult’s Law would apply, where the vapor pressure reported (over pure HAP solutions WITHOUT WATER) listed in the vapor pressure column of Appendix D could be divided by the mole fraction of water in a water solution containing a dissolved HAP. I don’t think the measurement techniques for the “solubilities” reported in Appendix D have anything to do with H.

21) pg 25, top paragraph: It seems like assuming 50 µg m-3 for regimes where actual measurements are lacking is a serious OVERESTIMATE of PM10 which would lead to a systematic bias in the gas/particle partitioning.

22) pg 27, properties of mercury: The Henry’s law solubility for Hg(0) is about a factor of 7 LOWER than that reported in the Henry’s law compilation noted in (18) above. This would make Hg much more susceptible to washout.

23) pg 21, bottom lines: It seems like daytime dry deposition velocities for Hg(0) on the order of
0.1 cm s-1 are much too high. Using this value for vd results in a PBL residence for Hg(0) time of about 11 days. Based on measurements of Hg(0) in the atmosphere, its emission rate, and budget studies, I thought the residence time for Hg(0) was actually about a year or more. Clearly there are some gross discrepancies between longer-term measurements of concentrations, emissions, and dry deposition velocities this large. This could be an area of legitimate scientific debate.

24) pg 28, Eq(38): Why are the NO3 and photolysis terms included in this equation if they are subsequently ignored? This is purely a style formality. I would prefer a text statement that these terms are ignored, and would keep the equations as close as possible to what is actually implemented in the code.

### end of review ###

Appendix G

Individual Peer Reviewer Final Report

Dr. Leiming Zhang

August, 2001

# Review comments on:

# “The Enhanced Algorithm for ISC3 model report and software”

Authors: ML Wesely, PV Doskey and JD Shannon

Reviewed by: Leiming Zhang

e-mail: Leiming.Zhang@ec.gc.ca	Tel: 1-(416) 739-5734

The current scheme has most updated information, especially on dry deposition. It is a great and nice effort to find all physicochemical properties for so many different HAPs.  The main improvements of the current scheme for dry deposition compared to earlier versions are: (a) the inclusion of cuticle uptake of gaseous species by lipid solubility, (b) revision on surface resistances for two base species (SO2 and O3) under wet conditions, and (c) improved estimation of particle deposition under unstable conditions. Overall, the dry deposition velocities for both gaseous and particle species calculated from the current scheme are within a reasonable range compared to available measurements. The model formulation is generally adequately. The code in ISC3 model is consistent with the algorithms described in the report, although there are some places that has omitted several second order (or even smaller) terms. The documentation is generally clear and adequate. Details of comments and suggestions on the scheme, the report and the code are listed below.

1.  Model formulation

  - Table 3:

In Table 3, soil resistance for SO2 (RgS) are given larger than for O3 (RgO) for most land types. Cuticle resistance for SO2 (RcS) are also given larger than for O3 (RcO). However, from published measurements over the past two decades, I found the dry deposition velocity (Vd) of SO2 were usually higher than for O3 over bail soil or during dry nighttime over canopies. Some data were obtained simultaneously for both SO2 and O3 at the same sites (Finkelstein et al., 2000; Meyers et al., 1998). This implies that the non-stomatal uptake, which are controlled by cuticle and soil uptake, for SO2 should be faster than for O3.

From the current report, it is difficult to know if Vd(SO2) is bigger than Vd(O3) during dry nighttime due to the reason that a new term, Rcl, is added in the current new dry deposition scheme. Rcl for SO2 and O3 are not described in this report and thus there is no way to compare its contribution to the Vd of SO2 and O3. Thus, I need to make some assumptions to check the model’s performance.

Assumption 1: Rcl for SO2 is not too small. If the contribution of Rcl to Vd(SO2) is smaller than the total contribution of all other non-stomatal (cuticle and soil) uptake, than Vd(SO2) will probably lager than Vd(O3). This is inconsistent with most available measurements.

Assumption 2: Rcl for SO2 is small. rcl for Formaldehyde is 49.5. Since Formaldehyde is similar to SO2 to some extent, I would assume SO2 also having a small rcl. Considering the scaling by different land types (equation 12), Rcl for SO2 could be as small as 100 sm-1. By using this value, Rcl would be the dominate term, not only for dry nighttime, but also for dry daytime. Thus, even the daytime Vd will be decided by Rcl since it is smaller than stomatal resistance. Then the diurnal variation of Vd will be controlled by the aerodynamic resistance, Ra. As is known, the diurnal cycle of SO2 under relatively dry conditions are controlled by stomatal uptake, only Ra will not give such a big diurnal variation of Vd as observed. A too small Rcl for So2 and similar species may not be reasonable.

To further demonstrate this conclusion, figure 1 shows a sensitivity test for Formaldehyde, a species considered in ISP3, for land type of forest. The only available dry deposition measurement for Formaldehyde I found is from Wahner and Krinke (2000), who showed that nighttime Vd values were around 0.2 cm s-1 for O3 and 0.5 cm s-1 for HCHO, and around midday Vd values were 1.1 cm s-1 for O3 and 1.3 cm s-1 for HCHO.  The diurnal cycles for O3 and HCHO are very similar under dry conditions, but different under wet conditions. However, surface resistance shown in figure 1 will not give a reasonable diurnal cycle, especially considering Ra should be small most of the time due to the big roughness length.

Assumption 3: The last possibility is that rcl for SO2 is not too big , nor is it too small. If the contribution of Rcl to Vd is still much smaller than stomatal uptake, but larger than the total of cuticle (excluding Rcl term) and soil uptake, then the model could still give a very reasonable estimation for SO2.

Suggestions:

- rcl and other information (i.e., H, f0, Da) that are needed for dry deposition calculation should be provided for both SO2 and O3. I noticed that some discussions on these information are provided for some species that are not considered in ISP3. Since SO2 and O3 are two base species, it is better to have all these information for SO2 and O3.

- Depending on the value of rcl for SO2 and O3, RgS and/or RcS may need to be adjusted based on a review of published dry deposition measurements. Values of rcl, RgS and RcS should give a reasonable diurnal cycle, and a reasonable dry nighttime Vd(SO2) compared to measurements.

  - Page 5, Equation (3):

Should Equation (3) be changed to : u* = {kumu*m/[ln(zrm/zo)- ]}1/2 ? The stability function  should be available since equation (2) also needs it.

  - Forest land type and Scaling with leaf area index

There is only one land use type (LUT) for forests. Considering that there are so many different types of forests (i.e., evergreen forest, deciduous forest, broadleaf forest, needleleaf forest, mixed forest), the stomatal resistance for different forest LUTs would be much different. Although for seasonal category 2 and 5, the value of F can be adjusted by the user, for all other seasonal categories, F is always set to 1. Since the dry deposition velocity is very sensitive to the real leaf area index, the simple scaling method introduced in this report can not satisfy the diversity of LUT for forests. As shown in Figure 1, Rc for Formaldehyde for season 3 is lower than for season 2. In reality, Rc for season 3 maybe much higher than for season 2. This is caused by the unreasonable scaling factor LAIr.  Also, USEPA may not be satisfied with the sudden big change of surface resistance Rc from season to season.

Suggestions:

While I still believe the current scaling method and the related formula are OK compared to so many other uncertainties in ISP3 model, I do have suggestions which can solve the above mentioned problems. Final decisions are leave to the model developer and USEPA.

- First choice: rather than use 5 different minimum stotal resistance (Ri)  for 5 different seasons, just use one Ri for all 5 seasons, then use LAI to sacle Ri (may not be a liniar scaling). Then choose LAI as an input parameter. Some guidedance for choosing LAI may be needed, such as some reference values for LAI.

- Second choice: as requested by USEPA, an existing stomatal resistance model is provided (see Appendix 1). Two terms in equation (7) Ri/f1 can be replaced by 1/Gst(PAR) calculated in Appendix 1 and all the rest terms in equation (7) can still be used. This approach only needs one value of minimum stomatal resistance for all 5 season. An extra input parameters, brs, is also needed, however, only one value for all 5 seasons.

  - Wet Deposition of particles:

The particle-phase washout coefficient Wp varies strongly from one type of aerosol to another, and also depends on precipitation type, intensity. More sophisticated scheme on washout coefficient are available (e.g., Mihaela and Stefan, 2000).

Suggestion:

Check the above reference to see if it is better than the one used in ISP3.

- Implementation

The algorithms implemented in the ISC3 model are mostly consistent with the report.

2.1 Ra under dew conditions

Under dew conditions during nighttime, the aerodynamic resistance Ra is adjusted to 1000 m s-1 if it is smaller than 1000 m s-1. This implies that Vd for any species are smaller than 0.1 cm s-1. Observed data show that the averaged Vd under dew conditions during nighttime for SO2 can be much higher than 0.1 cm s-1 (Finkelstein et al., 2000).  A lower limit of 1000 m s-1 for Ra under dew conditions during night seems too big.

2.2 Some unnecessary lines

Line: [ if (de.lt.0.) de = 0.  ] is not needed since “de” will never be less than 0. Above this line, RH is already checked and set within 5 to 100.

2.3 Some omitted terms

Several terms that appeared in the formula have been omitted. Most of these terms are second-order or smaller terms.

2.4 Zr for particle Vd

Model sensitivity tests show that Vd for particles is very sensitive to the reference height Zrdep. Theoretically, Vd should not be so sensitive to Zrdep. This is because changing Zrdep should only change Ra, not u* and thus not Rb (or Rp in equation 23a, b). However, in the present model local u* is adjusted using equation (3). This equation should be applied only when the reference height Zrdep is the same as the height used in pre-processor where u*m is calculated. As stated in the report, this height is usually at 10m and Zrdep is chosen as 1m. Thus u* is increased by applying equation (3).

Suggestion: Zrdep should not be chosen as 1 m, this height may be too low for land type with canopies. It can be changed with land use type as was done for gaseous species, but not lower than a few meters.

- Documentation.

The organization of the report is adequate and model features are clear. Some places may need more descriptions.

  - Henry's Law values.

The value of effective Henry's Law constants could be several orders different from their corresponding physical Henry’s law constants for those species that dissociates in the water. For those species that has no dissociation, the effective henry’s law constants equal to their physical Henry’s law constant. Different authors may report different physical Henry’s law constants as shown in Sander (2001). Since this parameter is important for both dry and wet deposition, it is better to show all reported Henry’s law constant in the Appendix C besides the value used in this report.

- 3.2 Description of Rcl in Page A-5

It is better to use consistent name for Rcl through the report. I believe Rcl is the bulk canopy cuticle resistance by lipid solubility. It is not bulk canopy stomatal resistance as stated in Page A-5.

References:

Finkelstein P.L., Ellestad T.G., Clarke J.F., Meyers T.P., Schwede D., Hebert E.O. and Neal J.F., 2000.  Ozone and sulfur dioxide dry deposition to forests: observations and model evaluation.  Journal of Geophysical Research, 105, 15365-15377.

Meyers T.P., Finkelstein P.L., Clarke J., Ellestad T.G. and Sims P.F. (1999)  A multi-layer model for inferring dry deposition using standard meteorological measurements. Journal of Geophysical Research 103 (D17), 22645-22661.

Mihaela M and Stefan S. (2000) A theoretical study of the microphysical parameterization og the scavenging coefficient as a function of precipitation type and rate. Atmospheric Environment 32, 2931-2938.

Sander R., 2001.  Compilation of Henry’s Law constants for inorganic and organic species of potential importance in environmental chemistry (version 3).  http://www.mpch-mainz. mpg.de/~sander/res/henry.html.

Wahner and Krinke (2000) Formaldehyde and ozone deposition velocities determined above a deciduous forest during summer.  Presented in the 6th Air-surface exchange conference July 3-7, Edinburgh, UK.

Zhang L., Moran M.D. and Brook J.R. (2001) A comparison of models to estimate in-canopy phtosynthetically active radiation and their influence on canopy stomatal resistance.  Atmospheric Environment 35, 4463-4470.

Figure 1: Surface resistance for Formaldehyde over a forest

Appendix 1

The stomatal resistance calculated in Zhang et al. (2001) is as follows:

Rst=1[Gst(PAR)f(T)f(D)f(ψ)DiDv]

where  Gst(PAR)  is the unstressed canopy stomatal conductance, a function of photosynthetically active radiation (PAR).  The dimensionless functions f(T), f(D), and f() represent the conductance-reducing effects of air temperature T, water vapour pressure deficit D, and water stress (leaf water potential) respectively, on leaf stomatal conductance.  Dv and Di are the molecular diffusivities for water vapour and the pollutant gas, respectively.

To fit this model into ISP3, only the term Gst(PAR) is needed. This term is equivalent to the reverse of Ri/f1 in equation (7), that is Ri/f1=1/Gst(PAR). Gst(PAR) is calculated as

Gst(PAR)=Fsunrst(PARsun)+Fshaderst(PARshade)			rst(PAR)=rstmin(1+brsPAR)

where  Fsun  and  Fshade are the total sunlit and shaded leaf area indexes (LAIs), respectively,  PARsun  and PARshade are PAR received by sunlit and shaded leaves, respectively, rst is the unstressed leaf stomatal resistance,  rstmin  is the minimum leaf stomatal resistance, and  brs is an empirical constant.

The expressions for Fsun and Fshade are:

Fsun=2cosθ[1−e(−0.5LAI/cosθ)]					Fshade=LAI−Fsun

The expressions for PARsun and PARshade are:

PARshade=Rdiffe(−0.5LAI0.7)+0.07Rdir(1.1−0.1LAI)e−cosθFor LAI < 2.5 or solar radiation < 200 Wm-2:

.

For all other conditions:

PARshade=Rdiffe(−0.5LAI0.8)+0.07Rdir(1.1−0.1LAI)e−cosθ

PARsun=Rdir0.8cosα/cosθ+PARshade

where LAI is the leaf area index of the canopy,  q  is the solar zenith angle, and  a  is the angle between the leaf and the sun and has a value of 60o for a canopy assumed to have a spherical leaf angle distribution.  Rdiff  and  Rdir  are the downward visible radiation fluxes above the canopy from diffuse and direct-beam radiation, respectively.

The following is the code for calculating Gst(PAR):

C        KEY   VARIABLES

* brs       Constant for stomatal resistance(W/m2)

* coszen  Cosine of solar zenith angle

* fsun     fraction of sunlit leaves (0.0-LAI)

* lai       Leaf area index  (no unit)

* pardir    visible beam radiation (W/m2)

* pardif    diffuse visible radiation (W/m2)

* rsmin    | minimum stomatal resistance (s/m)

* srad     | Solar irradiance (w/m2)

*-----------------------------------------------------------------------

C

C --  Calculate direct and diffuse PAR from solar radiation and solar zenith angle

RDU=600.*EXP(-0.185/COSZEN)*COSZEN

RDV=0.4*(600.-RDU)*COSZEN

WW=-ALOG(COSZEN)/2.302585

WW=-1.195+0.4459*WW-0.0345*WW**2

WW=1320*10**WW

RDM=(720.*EXP(-0.06/COSZEN)-WW)*COSZEN

RDN=0.6*(720-RDM-WW)*COSZEN

RV=AMAX1(0.1,RDU+RDV)

RN=AMAX1(0.01,RDM+RDN)

RATIO=AMIN1(0.9,SRAD(IL)/(RV+RN))

SV=RATIO*RV                            ! Total PAR

FV=AMIN1(0.99, (0.9-RATIO)/0.7)

FV=AMAX1(0.01,RDU/RV*(1.0-FV**0.6667))  !fraction of PAR in the direct beam

PARDIR=FV*SV                           ! PAR from direct radiation

PARDIF=SV-PARDIR                       ! PAR from diffuse radiation

C

C -- Calculate sunlit and shaded leaf area, PAR for sunlit and shaded leaves

C

C Note that the dimension I represent land type, the dimension ISEA represent season.

PSHAD=PARDIF*EXP(-0.5*LAI(I,ISEA)**0.7)

& +0.07*PARDIR*(1.1-0.1*LAI(I,ISEA))*EXP(-COSZEN)

PSUN=PARDIR*.5/COSZEN+PSHAD

IF (LAI(I,ISEA).GT.2.5.AND.SRAD(IL).GT.200.) THEN

PSHAD=PARDIF*EXP(-0.5*LAI(I,ISEA)**0.8)

& +0.07*PARDIR*(1.1-0.1*LAI(I,ISEA))*EXP(-COSZEN)

PSUN=PARDIR**0.8*.5/COSZEN+PSHAD

END IF

RSHAD=RSMIN(I)+BRS(I)*RSMIN(I)/PSHAD

RSUN=RSMIN(I)+BRS(I)*RSMIN(I)/PSUN

GSHAD=1./RSHAD

GSUN=1./RSUN

FSUN=2*COSZEN*(1.-EXP(-0.5*LAI(I,ISEA)/COSZEN))  ! Sunlit leaf area

FSHAD=LAI(I,ISEA)-FSUN                    ! Shaded leaf area

C

C – Unstressed stomatal conductance

C

GSPAR=FSUN*GSUN+FSHAD*GSHAD

Appendix H

Qualifications of Peer Review

Team Members

Appendix I

Documents Provided to

Peer Review Team Members

? The authors do touch on this subject briefly in their discussion of water and lipids in Section 2.2.3, and their discussion of mercury in Section 6.