## WQX / WQX Web 3.0 production release on April 13, 2020.

Webinar files: ftp://newftp.epa.gov/storet/xfer/How/2020-04-23/

WQX 3.0 streamlines the biological data model to make it simpler, while also providing critical new data elements to make the biological data more meaningful and more useful.  It also adds some new rules around data elements that were identified as critical for data reuse per the best practices documents that we've put out over the past couple years.  Lastly, WQX 3.0 puts WQX completely in-line with the data elements available in the Water Quality Portal.  USGS had added a number of elements to the Portal that weren't in WQX.  We have now added those to WQX.

WQX Version 2.2 is 100% backwards compatible with v2.1 and provides expanded data element lengths for various key description text type elements.  See the schema change log v2.2 and Data Exchange Template v2.2 for more details. Version 3.0 adopts new elements, some of which are not backwards compatible with version 2.1. See the schema change log v3.0 and Data Exchange Template v3.0 for more details.

WQX 3.0 addresses quality assurance by applying supported best practices and business rules. Good QA/QC ensures that your data can be reused by another user for analysis and safeguards against misuse. New version 3.0 supported QA/QC business rules are outlined via both the WQX 3.0 Flow Configuration Document and guidance documents at the WQX website (www.epa.gov/waterdata/water-quality-data-wqx):

Best Practices for Sharing Nutrients Data.

Best Practices for Sharing Metals Data.

Best Practices for Sharing Macroinvertebrates Data.

WQX Navigation Page: https://cdx.epa.gov/WQXWeb/StaticPages/GlossaryWqx.htm

- WQX resources

- WQX Web resources

- Helpdesk and Data assistance

- Video library (2-5 min) – Coming soon…

- Below are the best practice documents: The Guides are designed to be used with the 2.0 version of WQX. Some of these practices will become required elements in WQX 3.0 (scheduled for release in 2018 or 2020), so we encourage data submitters to address them now.

Introduction to QA Workgroup’s Best Practices for WQX / Water Quality Portal.

The Water Quality eXchange (WQX) Nutrient & Metals Best Practices Guide was created to guide organizations in submitting nutrient data to WQX and to address other metadata issues with data to support the submission of nutrient information. This guide was also created to make submitting nutrient datasets more user-friendly, promote consistency when submitting data and thus remove confusion and ambiguity for secondary data users. It was produced through both a WQX Nutrient QA Workgroup and Metals QA Workgroup respectively and comprised of representatives from EPA, USGS and several States which were also members of the STORET/WQX Users Group. The QA Workgroup addressed six areas of concern with respect to documenting nutrient data to improve its value for secondary users. For metals, this guidance does not require any new metadata to be submitted to WQX but does advise data submitters to make adjustments to characteristic names with new characteristic name proposals. The QA Workgroup included (1) duplicate monitoring locations, (2) addressing duplicate or ambiguous nutrient characteristic names (synonyms) in WQX (3), how to correctly use the WQX elements “method speciation” and “sample fraction”, (4) correctly submitting values above or below detection limits, (5) National analytical methods, and (6) how to submit a complete and unambiguous nutrient result.  This Guide is designed to be used with the 2.0 version of WQX. Some of these practices will become required elements in WQX 3.0 (scheduled for release in 2018), so we encourage data submitters to address them now.

Nutrient Conclusions: A complete nutrient result contains a Characteristic name, a result (or result detection condition) value and unit, a speciation, a sample fraction, and method. If reported as above or below a quantitation limit by the laboratory, it will need to contain the detection limit type, value, and unit instead of a result value and unit.

Metals Conclusions:  Documenting Method Speciation, Ions and Sample Fraction for Metals Characteristics Speciation Metals are only ever reported as a single speciation, thus the workgroup determined that method speciation is a nonessential metadata element and should not be a required data element. For example: Aluminum is always reported “as Al”, thus there is no need to require data submitters to report metadata that can be reasonably assumed.

- Aligning domain values within Water Quality Portal data sources

- Reference Document:    ftp://newftp.epa.gov/storet/xfer/today/WQPs_public_srsnames+speciation_February_2020.xlsx

- WQP Web services guide: Characteristic names identify different types of environmental measurements. The names are derived from the USEPA Substance Registry System (SRS). USGS uses parameter codes for the same purpose and has associated most parameters to SRS names.

## NEW Features:

- Import Configurations, User Supplied element names

- New Elements: NationalAquiferCode, LocalAquiferCode, LocalAquiferContext, GroupSummaryCount, GroupSummaryWeightMeasure, ComparableAnalyticalMethod, LaboratorySampleSplitRatio, ResultSamplingPointName, ResultSamplingPointType,  ResultSamplingPointPlaceInSeries (SamplingComponentPlaceInSeries)

- WQP Elements: HydrologicCondition, HydrologicEvent,

- Key Elements: ActivityIdentifierUserSupplied, HabitatSelectionMethod, CollectionArea, TargetCount, ProportionSampleProcessedNumeric , SampleContainerLabelName, CharacteristicNameUserSupplied, SubjectTaxonomicNameUserSupplied,  SubjectTaxonomicNameUserSuppliedReferenceText,

NEW Rules:

- Rule #32	Either Result Measure Value and/or Result Detection Condition Text must be reported

- Rule #35	ResultMeasureQualifierCode will support new WQX 3.0 measure qualifiers as independent child elements <MeasureQualifierCode> in parent <ResultMeasure> with a maximum number of 6 codes.

- Rule #41	Removed (v3.0) Sample Collection Method is required when Activity Type Code contains the word "Sample"

- Rule #46	ResultAnalyticalMethod may be required depending on the value provided for CharacteristicName. See the domain value list for CharacteristicName for more information

- Rule #47	MethodSpeciation may be required depending on the value provided for CharacteristicName. See the domain value list for CharacteristicName for more information

Frequently Asked Questions:

- Resubmitting Data. Will it cause a problem?

  - Projects perform an INSERT new or UPDATE existing

  - Monitoring Locations perform an INSERT new or UPDATE existing

    - For each Primary Entity you must submit the complete record. Partial submission will NULL existing values.

  - Activity and Results perform a DELETE and INSERT (for both new or existing)

  - Caution ACTIVITY ID are unique like SSN#.

    - For each Primary Entity you must submit the complete record. Partial submission will REMOVE existing results.

- Measurement Unit = "None" is assigned for all General Text characteristic names.

- Method Speciation = "None" can be assigned for all assumed characteristic names (ie Metals)

- Activity Elements - This Activity ID matches another one (from 1st sheet, row ####). So all other Activity elements must match as well. However, an Activity ELEMENT is different on this row.

Addressing quality assurance in your data 
Good QAQC ensures that your data can be reused by another user for analysis, as well as safeguarding against misuse. The documents below will help you improve the quality of your data.

- Best Practices for Sharing Nutrients Data (10p, 285K)

- Best Practices for Sharing Metals Data (6p, 151K)

Exchange Network - Water Quality Exchange Visit this site for information on WQX XML schema, data exchange template and Node information.

| STEP# | ## 2 | ## Data Dictionary and WQX Business Rules |
| --- | --- | --- |
|  | URL: | https://www.exchangenetwork.net/data-exchange/wqx/ |
|  |  | Download the Data Exchange Template (excel file) - data dictionary and element definition. |
|  |  | Download the Flow Configuration Document (acrobat file) - data validation rules, services, and schema. The principle document which captures the detailed data exchange processing rules governing WQX using narrative text, diagrams and examples. |
|  |  | WQX Domain Values tables provide data elements to assist data owners and users in conforming to a consistent nomenclature and can be queried to determine the EPA-supplied values. |

Frequent Questions about WQX

- Web Client (no software installed)

- Virtual Node Client

Frequently Asked Questions (FAQs) for Submitting Season eBeaches Notification and Monitoring Data

- Node / Node Client

Validation rules specify permissible attribute configurations and general relationships on an element.

| STEP# | ## 1 | ## WQX Business Rule 3.0 |
| --- | --- | --- |
|  | Rule #1 | When ElectronicAddressText or ElectronicAddressTypeName is reported, both must be reported. |
|  | Rule #2 | When TelephoneNumberText or TelephoneNumberTypeName is reported, both must be reported. |
|  | Rule #3 | When AddressText or AddressTypeName is reported, both must be reported. |
|  | Rule #4 | When HorizonitalCollectionMethodName is “Interpolation-Map”, SourceMapScale must be reported. |
|  | Rule #5 | When VerticalMeasure's MeasureValue is reported, the following also must be reported: VerticalMeasure's MeasureUnitCode, VerticalCollectionMethodName, VerticalCoordinateReferenceSystemDatumName. |
|  | Rule #6 | Either ProjectDescriptionText or Project's AttachedBinaryObject must be reported. |
|  | Rule #7 | Activity Depth/Height can be reported in only one of the following two ways (but not both): a. Specific depth using ActivityDepthHeightMeasure's MeasureValue. b. Depth Range using ActivityTopDepthHeightMeasure's MeasureValue and ActivityBottomDepthHeightMeasure's MeasureValue. i. This method must be used when ActivityTypeCode is “Sample-Integrated Vertical Profile”. |
|  | Rule #8 | When ActivityTypeCode contains the word 'Logger', DataLoggerLineName must be reported. |
|  | Rule #9 | When ActivityMediaName is "Tissue" then BiologicalIntentName must also be "Tissue" (and visa-versa) |
|  | Rule #10 | When ActivityMediaName (or BiologicalIntentName) is "Tissue", then SampleTissueAnatomyName must be reported. |
|  | Rule #11 | When ActivityMediaName is "Biological" then AssemblageSampledName must be reported |
|  | Rule #12 | When ResultDetectionConditionText is 'Not Detected', 'Present Above Quantification Limit' or 'Present Below Quantification Limit', then DetectionQuantitationLimitTypeName and DetectionQuantitationLimitMeasure must be reported. |
|  | Rule #13 | CharacteristicName and ResultStatusIdentifier must be reported. |
|  | Rule #14 | When DetectionQuantitationLimit's MeasureValue is reported, DetectionQuantitationLimit's MeasureUnitCode must be reported. |
|  | Rule #15 | ActivityDescription’s MonitoringLocationIdentifier may be required depending on the value provided for ActivityTypeCode. See the domain value list for ActivityTypeCode for more information. |
|  | Rule #16 | ResultAnalyticalMethod may be required depending on the value provided for ActivityTypeCode. See the domain value list for ActivityTypeCode for more information. a. However, ResultAnalyticalMethod is never required if BiologicalIntentName is "Individual", "Population Census", "Frequency Class", or "Group Summary" |
|  | Rule #17 | ResultSampleFractionText may be required depending on the value provided for CharacteristicName. See the domain value list for CharacteristicName for more information. |
|  | Rule #18 | ResultAnalyticalMethod’s MethodIdentifierContext must either match a value from the AnalyticalMethodContext domain list or it must be the same as the value for the OrganizationIdentifier provided in the submission file. a. If the MethodIdentifierContext matches a value from the domain list, then the MethodIdentifier must also match a value from the AnalyticalMethod domain list (for that Context). Furthermore, MethodName, MethodQualifierTypeName, and MethodDescriptionText are not required and will be ignored (since only the Identifier and IdentifierContext are needed to uniquely identify the Analytical Method). b. If the MethodIdentifierContext matches your OrganizationIdentifier (indicating your own method), then MethodIdentifier and MethodName are both required, but do not need to match a value from the domain list (since they are your own). Additionally, MethodQualifierTypeName and MethodDescriptionText can be provided, but are optional, to further describe the Analytical Method used. |
|  | Rule #19 | ProjectIdentifier, MonitoringLocationIdentifier, ActivityIdentifier, IndexIdentifier and ActivityGroupIdentifier must be unique within an Organization. The value for each of these identifiers may occur only once in a submission file. a. Unique identifiers are treated as case-insensitive by WQX. For example, the following three identifiers would be treated as identical: “Mx571”, “mx571”, “MX571”. |
|  | Rule #20 | ResultMeasure's ResultMeasureValue may be constrained to a list of domain values depending on the value provided for CharacteristicName. See the domain value list for CharacteristicName for more information. |
|  | Rule #21 | If a numeric value is reported for ResultMeasureValue, then ResultMeasure's MeasureUnitCode and ResultValueTypeName are required. a. The exception to this is when the ResultMeasureValue is a Characteristic Pick List Value. These do not have units. |
|  | Rule #22 | If a CountyCode is reported then a StateCode must also be reported. |
|  | Rule #23 | If NetTypeName = "Net/Horizontal Tow" then BoatSpeedMeasure is required. |
|  | Rule #24 | If NetTypeName is reported then the SampleCollectionEquipmentName must be one that relates to that type of equipment. |
|  | Rule #25 | ActivityMetric's MetricTypeIdentifierContext must either match a value from the MetricTypeContext domain list or it must be the same as the value for the OrganizationIdentifier provided in the submission file. a. If the MetricTypeIdentifierContext matches a value from the domain list, then the MetricTypeIdentifier must also match a value from the MetricType domain list (for that Context). Furthermore, MetricTypeName, MetricTypeCitation, MetricTypeScaleText, and FormulaDescriptionText are not required and will be ignored (since only the Identifier and IdentifierContext are needed to uniquely identify the MetricType). b. If the MetricTypeIdentifierContext matches your OrganizationIdentifier (indicating your own metric), then MetricTypeIdentifier and MetricTypeName are both required, but do not need to match a value from the domain list (since they are your own). Additionally, MetricTypeCitation, MetricTypeScaleText, and FormulaDescriptionText can be provided, but are optional, to further describe the Metric Type used. |
|  | Rule #26 | If BiologicalIntentName is "Group Summary" then GroupSummaryCount or GroupSummaryWeightMeasure must be reported |
|  | Rule #27 | If BiologicalIntentName is "Frequency Class" then Result's CharacteristicName must be "Count" |
|  | Rule #28 | If BiologicalIntentName is "Population Census" then Result's CharacteristicName must be "Count" or "Total Sample Weight" |
|  | Rule #29 | FrequencyClassDescriptorUnitCode may be required depending on the value provided for FrequencyClassDescriptorCode. See the domain value list for FrequencyClassType for more information. |
|  | Rule #30 | FrequencyClassInformation's LowerClassBoundValue and UpperClassBoundValue may be required depending on the value provided for FrequencyClassDescriptorCode. See the domain value list for FrequencyClassType for more information |
|  | Rule #31 | Biological Intent Name and Subject Taxonomic Name must be reported when Activity Media Name is "Biological" or "Tissue" |
|  | Rule #32 | Either Result Measure Value and/or Result Detection Condition Text must be reported |
|  | Rule #33 | Measure Unit is required when Measure Value is supplied |
|  | Rule #34 | Measure Value is required when Measurement Unit is supplied |
|  | Rule #35 | ResultMeasureQualifierCode will support new WQX 3.0 measure qualifiers as independent child elements <MeasureQualifierCode> in parent <ResultMeasure> with a maximum number of 6 codes. |
|  | Rule #36 | Habitat Selection Method is required when Activity Assemblage is "Benthic Macroinvertebrates" |
|  | Rule #37 | Target Count is required when the Activity Assemblage is "Benthic Macroinvertebrates" |
|  | Rule #38 | Proportion Sample Processed Numeric is required when the Activity Assemblage is "Benthic Macroinvertebrates" |
|  | Rule #39 | Proportion Sample Processed Numeric will be set to NULL when value is equal to 0.000000001 and the Activity Assemblage is "Benthic Macroinvertebrates" |
|  | Rule #40 | Proportion Sample Processed Numeric must be a positive number between 0 and 1" |
|  | Rule #41 | Removed (v3.0) Sample Collection Method is required when Activity Type Code contains the word "Sample" |
|  | Rule #42 | Statistical N-Value Numeric must be a positive whole number |
|  | Rule #43 | Analytical Method is required when submitting a Comparable Analytical Method |
|  | Rule #44 | Comparable Analytical Method Identifier Context must match a national value from the domain list |
|  | Rule #45 | If ResultDetectionQuantitationLimit is reported, then DetectionQuantitationLimitTypeName and DetectionQuantitationLimitMeasure are required |
|  | Rule #46 | ResultAnalyticalMethod may be required depending on the value provided for CharacteristicName. See the domain value list for CharacteristicName for more information |
|  | Rule #47 | MethodSpeciation may be required depending on the value provided for CharacteristicName. See the domain value list for CharacteristicName for more information |
|  | Rule #48 | Lab Sample Preparation Method is optional. |
|  | Rule #49 | Sample Preparation Method, Sample Container Type Name, Sample Container Color Name, and Sample Transport Storage Description are all optional. |

| Parm Code | Description | Characterstic Name | Measure Unit | Method Speciation |
| --- | --- | --- | --- | --- |
| 52886 | Inorganic monomeric aluminum, water, unfiltered, calculated, micrograms per liter as aluminum | Inorganic monomeric aluminum | ug/l | as Al |
| 49287 | Monomeric aluminum, water, unfiltered, micrograms per liter as aluminum | Monomeric aluminum | ug/l | as Al |
| 49288 | Organic monomeric aluminum, water, unfiltered, micrograms per liter as aluminum | Organic monomeric aluminum | ug/l | as Al |
| 63718 | 3-Nitro-4-hydroxyphenylarsonic acid, water, filtered, micrograms per liter as arsenic | 3-Nitro-4-hydroxyphenylarsonic acid | ug/L | as As |
| 52532 | Arsenate, water, unfiltered, micrograms per liter as arsenic | Arsenate | ug/L | as As |
| 99034 | Inorganic arsenic(III), water, filtered (0.45 micron filter), micrograms per liter as arsenic | Arsenic ion (3+) | ug/l | as As |
| 00997 | Inorganic arsenic, water, unfiltered, micrograms per liter as arsenic | Arsenic, Inorganic | ug/l | as As |
| 52531 | Arsenite, water, unfiltered, micrograms per liter as arsenic | Arsenite | ug/L | as As |
| 52534 | Dimethylarsinate ((CH3)2HAsO2), water, unfiltered, recoverable, micrograms per liter as arsenic | Cacodylic acid | ug/L | as As |
| 52533 | Monomethylarsonate ((CH3)HAsO3-), water, unfiltered, recoverable, micrograms per liter as arsenic | Monomethylarsonate | ug/L | as As |
| 34756 | Triazines, water, filtered, enzyme link immuno sorbent assay (ELISA) screen, recoverable, micrograms per liter as atrazine | Triazines mixture, unspecified | ug/L | as Atrazine |
| 99134 | Dissolved organic carbon (DOC), water, in situ, concentration computed by regression of sensor data with field sample lab results, milligrams per liter as carbon | Organic carbon | mg/l | as C |
| 85556 | Organic acids, water, unfiltered, recoverable, milligrams per liter as propionic acid | Organic acids | mg/L | as C3H6O2 |
| 00435 | Acidity, water, unfiltered, milligrams per liter as calcium carbonate | Acidity, hydrogen ion (H+) | mg/l | as CaCO3 |
| 00436 | Mineral acidity, water, unfiltered, methyl orange endpoint (pH 3.1-4.4) titration, milligrams per liter as calcium carbonate | Acidity, mineral methyl orange (as CaCO3) | mg/l | as CaCO3 |
| 00437 | Carbon dioxide acidity, water, unfiltered, phenolphthalein endpoint (pH 8.5-9.0) titration, milligrams per liter as calcium carbonate | Acidity, total, phenolphthalein (as CaCO3) | mg/l | as CaCO3 |
| 00410 | Acid neutralizing capacity, water, unfiltered, fixed endpoint (pH 4.5) titration, field, milligrams per liter as calcium carbonate | Alkalinity | mg/l | as CaCO3 |
| 00425 | Bicarbonate, water, unfiltered, milligrams per liter as calcium carbonate | Bicarbonate | mg/l | as CaCO3 |
| 00910 | Calcium, water, unfiltered, milligrams per liter as calcium carbonate | Calcium | mg/l | as CaCO3 |
| 00430 | Carbonate, water, unfiltered, milligrams per liter as calcium carbonate | Carbonate | mg/l | as CaCO3 |
| 00900 | Hardness, water, milligrams per liter as calcium carbonate | Hardness, Ca, Mg | mg/l | as CaCO3 |
| 00898 | Carbonate hardness, water, filtered, milligrams per liter as calcium carbonate | Hardness, carbonate | mg/l | as CaCO3 |
| 00902 | Noncarbonate hardness, water, unfiltered, field, milligrams per liter as calcium carbonate | Hardness, non-carbonate | mg/l | as CaCO3 |
| 00920 | Magnesium, water, unfiltered, milligrams per liter as calcium carbonate | Magnesium | mg/l | as CaCO3 |
| 71860 | Sodium carbonate, residual, water, unfiltered, milligrams per liter as calcium carbonate | Sodium carbonate | mg/l | as CaCO3 |
| 32334 | Chlorophyll, water, in situ, uncorrected for degradation products, computed by regression of sensor data with sample lab results, micrograms per liter as chlorophyll | Chlorophyll | ug/l | as Chlorophyll |
| 32316 | Chlorophyll fluorescence (fChl), water, in situ, concentration estimated from reference material, micrograms per liter as chlorophyll | Chlorophyll a | ug/l | as Chlorophyll |
| 32277 | Chlorophylls greater than 35 microns, calculated as chlorophylls (P32217) less chlorophylls <35 um (P32271), micrograms per liter | Chlorophyll | ug/l | as Chlorophyll a |
| 49268 | Organic halides, extractable, bed sediment smaller than 2 millimeters, wet sieved (native water), recoverable, dry weight, milligrams per kilogram as chloride | Organic halides, total -- SWDA NPDWR | mg/kg | as Cl |
| 01032 | Chromium(VI), water, filtered, micrograms per liter as Cr | Chromium(VI) | ug/l | as Cr |
| 51738 | N-Nitrosodiphenylamine, soil, recoverable, wet weight, milligrams per kilogram as diphenylamine | N-Nitrosodiphenylamine | mg/kg | as DPA |
| 71825 | Acidity, water, unfiltered, heated, milligrams per liter as hydrogen ion | Acidity, hydrogen ion (H+) | mg/l | as H |
| 00639 | Albuminoid nitrogen, water, unfiltered, milligrams per liter as nitrogen | Albuminoid nitrogen | mg/l | as N |
| 00619 | Ammonia (un-ionized), water, unfiltered, calculated, milligrams per liter as nitrogen | Ammonia | mg/l | as N |
| 00604 | Ammonia (NH3 + NH4+), suspended sediment, total, milligrams per liter as nitrogen | Ammonia and ammonium | mg/l | as N |
| 53573 | Inorganic nitrogen, water, unfiltered, calculated as NH3+NO2+NO3, milligrams per liter as nitrogen | Inorganic nitrogen | mg/l | as N |
| 51186 | Inorganic nitrogen, water, dissolved, calculated as NH3+NO2+NO3, milligrams per liter as nitrogen | Inorganic nitrogen (nitrate and nitrite and ammoni | mg/l | as N |
| 00628 | Nitrate plus nitrite, suspended sediment, total, milligrams per liter as nitrogen | Inorganic nitrogen (nitrate and nitrite) | mg/l | as N |
| 00623 | Ammonia plus organic nitrogen, water, filtered, milligrams per liter as nitrogen | Kjeldahl nitrogen | mg/l | as N |
| 00618 | Nitrate, water, filtered, milligrams per liter as nitrogen | Nitrate | mg/l | as N |
| 00613 | Nitrite, water, filtered, milligrams per liter as nitrogen | Nitrite | mg/l | as N |
| 01465 | Total nitrogen [nitrate + nitrite + ammonia + organic-N], biota, tissue, total, milligrams per liter as nitrogen | Nitrogen, mixed forms (NH3), (NH4), organic, (NO2) and (NO3) | mg/l | as N |
| 00605 | Organic nitrogen, water, unfiltered, milligrams per liter as nitrogen | Organic Nitrogen | mg/l | as N |
| 00933 | Sodium plus potassium, water, filtered, milligrams per liter as sodium | Sodium plus potassium | mg/l | as Na |
| 52155 | Ammonia (NH3 + NH4+), bed sediment, total, dry weight, milligrams per kilogram as NH3 | Ammonia and ammonium | mg/kg | as NH3 |
| 71845 | Ammonia (NH3 + NH4+), water, unfiltered, milligrams per liter as NH4 | Ammonia and ammonium | mg/l | as NH4 |
| 52153 | Nitrite, bed sediment, total, dry weight, milligrams per kilogram as nitrite | Nitrite | mg/kg | as NO2 |
| 51290 | Nitrate, water, in situ, milligrams per liter as nitrate | Nitrate | mg/l | as NO3 |
| 00669 | Hydrolyzable phosphorus, water, unfiltered, milligrams per liter as phosphorus | Hydrolyzable phosphorus | mg/l | as P |
| 76010 | Hydrolyzable phosphorus plus orthophosphate, suspended sediment, total, milligrams per liter as phosphorus | Hydrolyzable phosphorus plus orthophosphate | mg/l | as P |
| 00670 | Organic phosphorus, water, unfiltered, milligrams per liter as phosphorus | Organic phosphorus | mg/l | as P |
| 00671 | Orthophosphate, water, filtered, milligrams per liter as phosphorus | Orthophosphate | mg/l | as P |
| 00662 | Phosphorus, water, unfiltered, recoverable, micrograms per liter as phosphorus | Phosphorus | ug/l | as P |
| 32279 | Pheophytin a greater than 35 microns, calculated as pheophytin a (P32213) less pheophytin a <35 um (P32273), micrograms per liter | Pheophytin a | ug/l | as Pheophytin a |
| 32319 | Phycocyanin fluorescence (fPC), water, in situ, concentration estimated from reference material, micrograms per liter as phycocyanin | Phycocyanins | ug/l | as Phycocyanin |
| 32336 | Phycoerythrin fluorescence (fPE), water, in situ, concentration estimated from reference material, micrograms per liter as phycoerythrin | Phycoerythrin fluorescence (fPE) | ug/l | as Phycoerythrin |
| 52314 | Phosphates (orthophosphate plus condensed and polyphosphates), water, unfiltered, milligrams per liter as PO4 | Phosphates (orthophosphate plus condensed and polyphosphates) | mg/l | as PO4 |
| 00650 | Phosphate, water, unfiltered, milligrams per liter as PO4 | Phosphate-phosphorus | mg/l | as PO4 |
| 51131 | Orthophosphate, water, unfiltered, recoverable, milligrams per liter as phosphate | Orthophosphate | mg/l | as PO4 |
| 32341 | Optical brightener fluorescence (fOB), water, in situ, concentration estimated from reference material, micrograms per liter as 1,3,6,8-pyrene tetrasulfonic acid (PTSA) | Optical brightener fluorescence (fOB) | ug/L | as PTSA |
| 32295 | Dissolved organic matter fluorescence (fDOM), water, in situ, concentration estimated from reference material, micrograms per liter as quinine sulfate equivalents (QSE) | Colored dissolved organic matter (CDOM) | ug/l | as QSE |
| 83157 | Sulfate, wet atmospheric deposition, filtered, milligrams per liter as sulfur | Sulfate | mg/l | as S |
| 51284 | Dissolved organic selenium, water, filtered (0.45 micrometer filter), micrograms per liter as selenium | Organic selenium | ug/l | as Se |
| 00954 | Silica, water, unfiltered, recoverable, micrograms per liter as SiO2 | Silica | ug/l | as SiO2 |
| 00075 | Turbidity, water, unfiltered, Hellige turbidimeter, milligrams per liter as silicon dioxide | Turbidity | mg/l | as SiO2 |
| 22706 | Uranium, water, unfiltered, micrograms per liter as uranium(V,VI) oxide (U3O8) | Uranium | ug/L | as U3O8 |

Note: Contractor has code translations (mappings) for all LEGACY or DEPRECATED result measure qualifier.

| Result Measure Qualifier DEPRECATED | MSRQLF_CD1 | MSRQLF_CD2 | MSRQLF_CD3 | MSRQLF_CD4 | MSRQLF_CD5 | MSRQLF_CD6 | RMQLF_DESC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| " | LCS | + |  |  |  |  | LCS or LCSD is outside acceptance limits. |
| $ | $ |  |  |  |  |  | Incorrect sample container |
| * | * |  |  |  |  |  | Sample was warm when received |
| ^ | ^ |  |  |  |  |  | Yield outside of contractual acceptable range (USGS) |
| 4 | 4 | MSD | GR |  |  |  | MS, MSD: The analyte present in the original sample is greater than 4 times the matrix spike concentration; therefore, control limits are not applicable |
| A | A |  |  |  |  |  | Compound not analyzed |
| AL | AL |  |  |  |  |  | Aldol condensation present. Analyte may not be present. |
| ALK | ALK |  |  |  |  |  | Estimate, Alkylated PAH Sum |
| ALT | ALT |  |  |  |  |  | Alternate Method |
| B | B |  |  |  |  |  | Detection in blank |
| BAC | BAC |  |  |  |  |  | Correction Factor, background |
| BDJ | B | DI | J |  |  |  | B D J; Detection in blank. Dilution required. Estimated value |
| BH | B | H |  |  |  |  | Detection in blank. Holding time exceeded. |
| BJ | B | J |  |  |  |  | B J; Detected in blank. Estimated value |
| BJHQC | B | H | J | QC |  |  | Estimated value, compound also detected in LRB, holding time exceeded, QC problems |
| BJHT | B | H | J |  |  |  | Estimated value, compound also detected in LRB, holding time exceeded |
| BJQC | B | J | QC |  |  |  | Estimated value, compound also detected in LRB, QC problems |
| BL | B | L |  |  |  |  | Detection in blank. Lowest available reporting limit for the analytical method used. |
| BMDL | MDL | LT |  |  |  |  | Concentration is less than method detection limit (MDL) |
| BQB | B | BQL |  |  |  |  | Below Quantitation Limit. Detection in blank. |
| BQBJ | B | BQL | J |  |  |  | Below Quantitation Limit. Detection in blank. Estimated. |
| BQJ | BQL | J |  |  |  |  | Below Quantitation Limit. Estimated. |
| BQL | BQL |  |  |  |  |  | Below Quantitation Limit |
| BRL | BRL |  |  |  |  |  | Below Reporting Limit |
| BU | B | U |  |  |  |  | Detection in blank. Not Detected: The analyte was analyzed for, but was not detected at a level greater than or equal to the level of the adjusted Contract Required Quantitation Limit (CRQL) for sample and method. |
| BVER | BVER |  |  |  |  |  | Continuing calibration blank criteria was not met |
| C | C |  |  |  |  |  | Presence of compound may be due to contamination of sample during laboratory processing |
| C25 | C25 |  |  |  |  |  | Dual Column result difference >25% |
| CAJ | CAJ |  |  |  |  |  | Correction Factor, lab |
| CAN | CAN |  |  |  |  |  | No Result Reported, analysis canceled |
| CB | CC | B |  |  |  |  | C B; Co-eluting congener. Detection in blank. |
| CB+ | CB | HH |  |  |  |  | CCB out high. |
| CB+J | CB | HH | MDL | MQL | J |  | CCB out high. Estimated value between MDL and MQL. |
| CBC | CBC |  |  |  |  |  | No Result Reported, cannot be calculated |
| CBD | CC | DI | B |  |  |  | C B D; Co-eluting congener. Detection in blank. Dilution required |
| CBDJ | CC | DI | J | B |  |  | C B D J; Co-eluting congener. Detection in blank. Dilution required. Estimated |
| CBJ | CC | B | J |  |  |  | C B J; Co-eluting congener. Detection in blank. Estimated |
| CBL | CBL |  |  |  |  |  | Correction Factor, blank |
| CBQBJ | B | BQL | CC | J |  |  | Co-eluting congener. Below Quantitation Limit. Detection in blank. Estimated. |
| CC | CC |  |  |  |  |  | co-eluting congener |
| CCB | B | CC |  |  |  |  | Co-eluting congener. Detection in blank. |
| CCBJ | B | CC | J |  |  |  | Co-eluting congener. Detection in blank. Estimated. |
| CCBQ | BQL | CC |  |  |  |  | Co-eluting congener. Below Quantitation Limit. |
| CCBQJ | BQL | CC | J |  |  |  | Co-eluting congener. Below Quantitation Limit. Estimated. |
| CCJ | CC | J |  |  |  |  | Co-eluting congener. Estimated. |
| CCU | CC | U |  |  |  |  | Co-eluting congener. Not Detected. |
| CD | CC | B |  |  |  |  | C D; Co-eluting congener. Detection in blank. Dilution required |
| CDI | CDI |  |  |  |  |  | Correction Factor, dilution |
| CDJ | CC | DI | J |  |  |  | C D J; Co-eluting congener. Dilution required. Estimated |
| CEBDJ | CC | EMPC | B |  |  |  | C EMPC B D J; Co-eluting congener. Estimated maximum possible concentration. Detection in blank. Dilution required. Estimated. |
| CECBJ | CC | EMPC | B |  |  |  | C EMPC B J; Co-eluting congener. Estimated maximum possible concentration. Detection in blank. Estimated. |
| CECDJ | CC | EMPC | DI |  |  |  | C EMPC D J; Co-eluting congener. Estimated maximum possible concentration. Dilution required. Estimated. |
| CEMCB | CC | EMPC | B |  |  |  | C EMPC B; Co-eluting congener. Estimated maximum possible concentration. Detection in blank |
| CEMPC | CC | EMPC |  |  |  |  | C EMPC; Co-eluting congener. Estimated maximum possible concentration |
| CEPCD | CC | EMPC | DI |  |  |  | C EMPC D; Co-eluting congener. Estimated maximum possible concentration. Dilution required. |
| CEPCJ | CC | EMPC | J |  |  |  | C EMPC J; Co-eluting congener. Estimated maximum possible concentration. Estimated. |
| CJ | CC | J |  |  |  |  | Co-eluting congener. Estimated value. |
| CJQC | C | J | QC |  |  |  | Estimated value, possible contamination of sample during laboratory processing, QC problems |
| CLC | CLC |  |  |  |  |  | Correction Factor, other |
| CNT | CNT |  |  |  |  |  | Non-acceptable colony counts. |
| CON | CON |  |  |  |  |  | Value Confirmed |
| CU | CC | U |  |  |  |  | Co-eluting congener. Not detected. |
| CUD | CC | U | DI |  |  |  | C U D; Co-eluting congener. Not detected. Dilution required |
| D | D |  |  |  |  |  | Contract Required Quantitation Limit (CRQL) not met due to sample matrix interference, dilution required. |
| DB | B | D |  |  |  |  | Contract Required Quantitation Limit (CRQL) not met due to sample matrix interference, dilution required. Detection in blank. |
| DBH | B | D | H |  |  |  | Contract Required Quantitation Limit (CRQL) not met due to sample matrix interference, dilution required. Detection in blank. Holding time exceeded. |
| DE | DE |  |  |  |  |  | Serial dilution acceptance criteria not met. |
| DEC | DEC |  |  |  |  |  | Value Decensored |
| DH | D | H |  |  |  |  | Contract Required Quantitation Limit (CRQL) not met due to sample matrix interference, dilution required. Holding time exceeded |
| DHJ | D | H | J |  |  |  | Contract Required Quantitation Limit (CRQL) not met due to sample matrix interference, dilution required. Holding time exceeded. Estimated: The analyte was positively identified and the associated numerical value is the approximate concentration of the analyte in the sample |
| DJ | D | J |  |  |  |  | Contract Required Quantitation Limit (CRQL) not met due to sample matrix interference, dilution required. Estimated:The analyte was positively identified and the associated numerical value is the approximate concentration of the analyte in the sample. |
| DL | DL |  |  |  |  |  | Not Detected: The analyte was not detected at a level >= to the Method Detection Limit for the analysis. |
| DR | D | R |  |  |  |  | Contract Required Quantitation Limit (CRQL) not met due to sample matrix interference, dilution required. Rejected: The sample results are unusable due to the quality of the data generated because certain criteria were not met. The analyte may or may not be present in the sample. |
| DU | D | U |  |  |  |  | Contract Required Quantitation Limit (CRQL) ... Not Detected: The analyte was analyzed for, but was not detected at a level greater than or equal to the level of the adjusted Contract Required Quantitation Limit (CRQL) for sample and method. |
| E | E |  |  |  |  |  | Concentration of analyte being analyzed exceeded calibration range of instrument. |
| ECBDJ | EMPC | B | DI | J |  |  | EMPC B D J; Estimated maximum possible concentration. Detected in blank. Dilution required. Estimated value |
| ECI | ECI |  |  |  |  |  | Estimated Value, Coelution |
| EE | EE |  |  |  |  |  | Identifies compounds whose concentration exceed the calibration range addition of the instrument for that specific analysis. |
| EER | EER |  |  |  |  |  | No Result Reported, entry error |
| EFAI | EFAI |  |  |  |  |  | Equipment failure |
| EHT | H |  |  |  |  |  | Sample or extract held beyond acceptable holding time. |
| EMBDJ | EMPC | B | DI | J |  |  | EMPC BDJ; Estimated maximum possible concentration. Detected in blank. Dilution required. Estimated value |
| EMCBD | EMPC | B | DI | J |  |  | Estimated maximum possible concentration. Detected in blank. Dilution required. Estimated value. |
| EMCBJ | EMPC | J |  |  |  |  | Estimated maximum possible concentration. Estimated value. |
| EMCDJ | EMPC | DI | J |  |  |  | Estimated maximum possible concentration. Dilution required. Estimated value. |
| EMPC | EMPC |  |  |  |  |  | Estimated Maximum Possible Concentration |
| EMPCB | EMPC | B |  |  |  |  | EMPC B; Estimated maximum possible concentration. Detected in blank |
| EMPCJ | EMPC | J |  |  |  |  | EMPC J; Estimated maximum possible concentration. Estimated value |
| ENPNF | J | NPNF |  |  |  |  | Estimated; Normal protocol not followed |
| EPCBD | EMPC | B | DI |  |  |  | EMPC B D; Estimated maximum possible concentration. Detected in blank. Dilution required |
| EPCBJ | EMPC | B | J |  |  |  | EMPC B J; Estimated maximum possible concentration. Detected in blank. Estimated value |
| EPCD | EMPC | DI |  |  |  |  | EMPC D; Estimated maximum possible concentration. Detected in blank. Dilution required. |
| EPCDJ | EMPC | DI | J |  |  |  | EMPC D J; Estimated maximum possible concentration. Dilution required. Estimated value |
| EQF | EFAI |  |  |  |  |  | Equipment failed, sample not analyzed |
| ESD | ESD |  |  |  |  |  | Estimated Value, serial dilution difference |
| EST | EST |  |  |  |  |  | Estimated Value, outside limit of precision |
| EVA | EVA |  |  |  |  |  | Estimated Value, multiple Aroclors |
| EVAD | EVAD |  |  |  |  |  | Estimated Value, degradation |
| EVID | EVID |  |  |  |  |  | Estimated Value, tentatively identified compound |
| F | F |  |  |  |  |  | Estimated value: compound failed initial calibration check (CCC) or QC criteria |
| F1F2 | MSD | + |  |  |  |  | MS and/or MSD Recovery is outside acceptance limits and MS/MSD RPD exceeds control limits. |
| F1F2B | MSD | + | N | B | S |  | MS and/or MSD Recovery is outside acceptance limits, MS/MSD RPD exceeds control limits, and compound was found in the blank and sample. |
| FBK | B |  |  |  |  |  | Analyte found in blank. Sample contamination indicated. |
| FDB | FDB |  |  |  |  |  | Dry Blank, failed |
| FDC | FDC |  |  |  |  |  | Drift Check, failed |
| FDL | FDL |  |  |  |  |  | Lab Duplicate, failed |
| FEQ | FEQ |  |  |  |  |  | Field Equipment Questionable |
| FFB | FFB |  |  |  |  |  | Failed. Field blank not acceptable. |
| FFD | FFD |  |  |  |  |  | Field Duplicate, failed |
| FFS | FFS |  |  |  |  |  | Failed. Field spike not acceptable. |
| FFT | FFT |  |  |  |  |  | Failed. Trip blank not acceptable. |
| FIS | FIS |  |  |  |  |  | Internal Standard, failed |
| FLA | FLA |  |  |  |  |  | Field Lab Anomaly |
| FLD | FLD |  |  |  |  |  | Failed. Lab duplicate not acceptable. |
| FLS | FLS |  |  |  |  |  | Failed. Lab spike recovery not acceptable. |
| FMD | F | MSD |  |  |  |  | Failed. Matrix Spike Duplicate not acceptable. |
| FMS | FMS |  |  |  |  |  | Failed. Matrix spike recovery not acceptable. |
| FPC | FPC |  |  |  |  |  | Performance Check, failed |
| FPR | FPR |  |  |  |  |  | Ongoing Precision and Recovery, failed |
| FQC | FQC |  |  |  |  |  | Quality Control, failed |
| FQCU | F | FQC | U |  |  |  | FQC,U -Quality Control, failed, Not Detected |
| FRS | FRS |  |  |  |  |  | Lab Reference, failed |
| FSD | FSD |  |  |  |  |  | Lab Spike Duplicate, failed |
| FSL | FSL |  |  |  |  |  | Failed. Spiked lab blank recovery not acceptable. |
| FSP | FSP |  |  |  |  |  | Failed. Surrogate spike recovery not acceptable. |
| FUB | FUB |  |  |  |  |  | Field Tubing Blank, failed |
| G | G |  |  |  |  |  | lock mass interference present |
| GG | RR | MDL | PQL |  |  |  | Reported Value Is Between MDL and the Practical Quantitation Level (Or Reporting Limit) |
| GT | GT |  |  |  |  |  | The listed result is greater than the upper quantitation limit for either the analytical method or the meter used for the measurement. Equivalent to the Legacy STORET Remark Code of L: Actual Value is known to be greater than the value given. |
| GXB | GXB |  |  |  |  |  | Estimated Value, greater than 10x blank |
| H | H |  |  |  |  |  | Holding time exceeded: |
| HBJ | B | H | J |  |  |  | Holding time exceeded. Detection in blank. Estimated: The analyte was positively identified and the associated numerical value is the approximate concentration of the analyte in the sample. |
| HIB | HIB |  |  |  |  |  | Likely Biased High |
| HICC | HICC |  |  |  |  |  | Initial calibration criteria not met – high |
| HIM | HIM |  |  |  |  |  | High Moisture |
| HL | H | L |  |  |  |  | Holding time exceeded. Lowest available reporting limit for the analytical method used. |
| HLBL | HLBL |  |  |  |  |  | high labeled compound recovery in sample, estimated value, estimated value |
| HMSD | HMSD |  |  |  |  |  | Matrix spike duplicate acceptance criteria not met – high |
| HMSR | HMSR |  |  |  |  |  | high matrix spike recovery, potential high bias |
| HMSRJ | MSR | J | HH |  |  |  | High matrix spike recovery, potential high bias, Estimated: The analyte was positively identified and the associated numerical value is the approximate concentration of the analyte in the sample |
| HNRO | HNRO |  |  |  |  |  | high native analyte recovery in OPR (or LCS), potential high bias |
| HTE | H |  |  |  |  |  | Hold time exceeded. |
| HTH | HTH |  |  |  |  |  | Hard to Homogenize |
| HVER | HVER |  |  |  |  |  | high calibration verification standard recovery, estimated value |
| I | I |  |  |  |  |  | Estimated value; compound failed initial calibration value |
| ICA | ICA |  |  |  |  |  | Incorrect Initial Calibration Associated with Sample |
| IDL | IDL | LT |  |  |  |  | Instrument Detection Limit, less than |
| INT | INT |  |  |  |  |  | Interference suspected. Analyte may not be present. |
| IQCOL | IQCOL |  |  |  |  |  | ICV,CCV,ICB,CCB, ISA, ISB, CRI, CRA, DLCK or MRL standard: Instrument related QC is outside acceptance limits |
| ISAC | ISAC |  |  |  |  |  | Internal standard acceptance criteria not met |
| ISP | ISP |  |  |  |  |  | Improper Sample Preservation |
| ITNA | ITNA |  |  |  |  |  | Incubation time not attained |
| ITNM | ITNM |  |  |  |  |  | Incubation temperature not maintained |
| J | J |  |  |  |  |  | Estimated: The analyte was positively identified and the associated numerical value is the approximate concentration of the analyte in the sample. |
| J- | J- |  |  |  |  |  | The analyte was positively identified; the associated numerical value is the approximate concentration of the analyte in the sample, and may have a potential negative bias. |
| J+ | J+ |  |  |  |  |  | Estimated: The analyte was positively identified and the associated numerical value... +++. |
| JB | B | J |  |  |  |  | Estimated: The analyte was positively identified and the associated numerical value is the approximate concentration of the analyte in the sample. Detection in blank. |
| JCN | JCN |  |  |  |  |  | Sample Container Damaged, no sample lost |
| JCW | JCW |  |  |  |  |  | Sample Container Damaged, sample lost |
| JDE | J | DE |  |  |  |  | Estimated: The analyte was positively identified and the associated numerical value is the approximate concentration of the analyte in the sample. And serial dilution acceptance criteria not met. |
| JH | H | J |  |  |  |  | Estimated: The analyte was positively identified and the associated numerical value is the approximate concentration of the analyte in the sample. Holding time exceeded. |
| J-HT | H | J |  |  |  |  | Approximate value analysis exceeded the holding time |
| JHTF | J | HTF |  |  |  |  | Estimated value. Holding time exceeded in the field |
| JHTQC | H | J | QC |  |  |  | Estimated value, exceeded holding time and QC problems |
| JL | J | L |  |  |  |  | Estimated: The analyte was positively identified and the associated numerical value is the approximate concentration of the analyte in the sample. Lowest available reporting limit for the analytical method used. |
| JL,U | MDL | J | LT |  |  |  | Result estimated low. Result is less than the MDL. |
| J-MI | J | MI |  |  |  |  | Approximate value due to matrix interference |
| JMQC | J | MI | QC |  |  |  | Estimated value, matrix interference, QC problems |
| J-QC | J | QC |  |  |  |  | Approximate value due to quality control problems |
| J-R | J | R |  |  |  |  | Approximate value result is below the reporting level but greater than the method detection limit |
| J-RB | B | J | R |  |  |  | Approximate value result is below the reporting level but greater than the method detection limit. Detection in blank. |
| J-RH | H | J | R |  |  |  | Approximate value result is below the reporting level but greater than the method detection limit. Holding time exceeded. |
| JRHQC | H | J | R | QC |  |  | Estimated value, between detection limit and reporting limit, holding time exceeded, QC problems |
| JRHT | H | J | R |  |  |  | Estimated value, between detection limit and reporting limit and exceeded holding time |
| JRQC | J | R | QC |  |  |  | Estimated value, between detection limit and reporting limit and QC problems |
| K | K |  |  |  |  |  | Value below the detection Limit. For BOD: depletion is less than 1.0 |
| KB | B | K |  |  |  |  | Not detected, compound also detected in LRB |
| KBJHQ | B | H | J | K | QC |  | Estimated value, not detected, compound also detected in LRB, holding time exceeded, QC problems |
| KBJQC | B | J | K | QC |  |  | Estimated value, not detected, compound also detected in LRB, QC problems |
| KCF | KCF |  |  |  |  |  | Known Contamination, field |
| KJ | J | K |  |  |  |  | Estimated value, not detected |
| KJHQC | H | J | K | QC |  |  | Estimated value, not detected, holding time exceeded and QC problems |
| KJHT | H | J | K |  |  |  | Estimated value, not detected and exceeded holding time |
| KJQC | J | K | QC |  |  |  | Estimated value, not detected and QC problems |
| KK | KK |  |  |  |  |  | True bacterial concentration is assumed to be less than the reported value. |
| KN | K | N |  |  |  |  | Not detected, presumptive evidence of nontarget compound |
| KNBJQ | B | J | K | N | QC |  | Estimated value, not detected, compound also detected in LRB, presumptive evidence of nontarget compound, QC problems |
| KNJHT | H | J | K | N |  |  | Estimated value, not detected, presumptive evidence of nontarget compound, holding time exceeded |
| KNJQC | J | K | N | QC |  |  | Not detected, presumptive evidence of non-target compound, estimated value, QC problems |
| KRMDL | MDL | LT | = |  |  |  | Reported value was at or below the method detection limit (MDL) and entered at the MDL |
| KRPQL | PQL | LT | = |  |  |  | Reported value was at or below the reporting limit (PQL or LQL) and entered at the PQL |
| L | L |  |  |  |  |  | Lowest available reporting limit for the analytical method used. |
| L5B | J | L5B |  |  |  |  | Estimated Value, less than 5x blank |
| LAC | LAC |  |  |  |  |  | No Result Reported, lab accident |
| LBF | LBF |  |  |  |  |  | Lab Failed, sample not analyzed |
| LBJQC | B | J | L | QC |  |  | Estimated value, greater than quantitation limit, compound also detected in LRB, QC problems |
| LICC | LICC |  |  |  |  |  | Initial calibration criteria not met – low |
| LIS | LIS |  |  |  |  |  | Lab internal standard(s) added to sample. |
| LJHQC | H | J | L | QC |  |  | Estimated value, greater than quantitation limit, holding time exceeded, QC problems |
| LJQC | J | L | QC |  |  |  | Estimated value, greater than quantitation limit and QC problems |
| LL | LL |  |  |  |  |  | True bacterial concentration is assumed to be greater than the reported value. |
| LLBL | J | LLBL |  |  |  |  | low labeled compound recovery in sample, estimated value |
| LLRO | J | LLRO |  |  |  |  | low labeled compound recovery in the OPR (or LCS), estimated value |
| LLS | LLS |  |  |  |  |  | Value less than lower quality control standard. |
| LMSD | LMSD |  |  |  |  |  | Matrix spike duplicate acceptance criteria not met – low |
| LMSR | LMSR |  |  |  |  |  | low matrix spike recovery, potential low bias |
| LMSRJ | MQL | MDL | GB | MSR | LL | J | Low matrix spike recovery, potential low bias. Result is less than the MQL but greater than or equal to the MDL and the concentration is an approximate value. |
| LMSRU | MDL | LT | MSR | LL |  |  | low matrix spike recovery, potential low bias. Result is less than the MDL. |
| LNJQC | J | L | N | QC |  |  | Estimated value, greater than quantitation limit, presumptive evidence of nontarget compound, QC problems |
| LNRO | LNRO |  |  |  |  |  | low native analyte recovery in OPR (or LCS), potential low bias |
| LOB | LOB |  |  |  |  |  | Likely Biased Low |
| LOPR | LOPR |  |  |  |  |  | low OPR (or LCS) recovery, potential low bias |
| LSSR | LSSR |  |  |  |  |  | Surrogate standard acceptance criteria not met – low |
| LTL | LR | LT |  |  |  |  | Operating Range, less than |
| LVER | LVER |  |  |  |  |  | low calibration verification standard recovery, potential low bias |
| LXB | J | LXB |  |  |  |  | Estimated Value, between 5-10x blank |
| M6F | M6F |  |  |  |  |  | More Than 6 Flags Applied |
| MI | MI |  |  |  |  |  | Matrix Interference |
| MSR | MSR |  |  |  |  |  | Matrix spike acceptance criteria not met |
| MTHD2 | MTHD2 |  |  |  |  |  | 2nd method (USGS) |
| MTRX | J | MI |  |  |  |  | possible matrix interference, estimated value |
| MTX | MTX |  |  |  |  |  | Matrix Interference. |
| N | N |  |  |  |  |  | Presumptive evidence of a nontarget compound |
| NA | NA |  |  |  |  |  | Not Applicable |
| NAI | NAI |  |  |  |  |  | No Result Reported, interference |
| NB | B | N |  |  |  |  | Presumptive evidence of non-target compound; detected in blank |
| NBJQC | B | J | N | QC |  |  | Estimated value, presumptive evidence of nontarget compound, compound also detected in LRB, QC problems |
| NFNSI | NFNS | RV |  |  |  |  | While comparison of nutrient fractions (e.g. filtered > unfiltered) or nutrient species (e.g. PO4 > TP) results are not consistent, the results are within precision limits and are analytically equal. |
| NFNSU | NFNS | RA |  |  |  |  | Comparison of nutrient fractions (e.g. filtered > unfiltered) or nutrient species (e.g. PO4 > TP) are not consistent. Results fall outside the normal limits of variability and do not meet Data Quality Objectives. Reanalyses were performed with consistent results. All QA limits and results were valid |
| NHS | NHS |  |  |  |  |  | Non-homogenous sample |
| NJ | J | N |  |  |  |  | Estimated value, presumptive evidence of nontarget compound |
| NJHT | H | J | N |  |  |  | Estimated value, presumptive evidence of nontarget compound, holding time exceeded |
| NJQC | J | N | QC |  |  |  | Estimated value, presumptive evidence of nontarget compound, QC problems |
| NL | L | N |  |  |  |  | Estimated value, presumptive evidence of nontarget compound, greater than quantitation limit |
| NLBL | NLBL |  |  |  |  |  | no labeled compound recovery in sample, rejected |
| NLRO | NLRO |  |  |  |  |  | no labeled compound recovery in OPR (or LCS), rejected |
| NMSR | MSR | R |  |  |  |  | no matrix spike recovery, rejected |
| NN | NN |  |  |  |  |  | authentic recovery is not within method/contract control limits |
| NRO | NRO |  |  |  |  |  | Control sample acceptance criteria not met |
| NRP | NRP |  |  |  |  |  | No Result Possible |
| NRR | NRR |  |  |  |  |  | No Result Reported, other |
| NRS | NRS |  |  |  |  |  | Non Representative Sample |
| NSQ | NSQ |  |  |  |  |  | No Result Reported, insufficient quantity of sample |
| NWL | LR | NW |  |  |  |  | Operating Range, not within |
| OA3 | OA3 |  |  |  |  |  | Outlier, across stations |
| OS3 | OS3 |  |  |  |  |  | Outlier, single station |
| OTH | OTHER |  |  |  |  |  | Other |
| OTHER | OTHER |  |  |  |  |  | Other |
| OUT | OUT |  |  |  |  |  | Result value is defined as an outlier by data owner |
| PBH | PB | HQ | J | F |  |  | Estimated value, continuous probe biased high Quality control measures failed high at audit check during deployment. Could include pre, mid or post deployment check. |
| PBL | PB | LQ | J | F |  |  | Estimated value, continuous probe biased low Quality control measures failed low at audit check during deployment. Could include pre, mid or post deployment check. |
| PNQ | PNQ |  |  |  |  |  | No Quantifiable Result Reported |
| PPD | PPD |  |  |  |  |  | Spiked Blank Duplicate, failed |
| PRE | PRE |  |  |  |  |  | Presumptive evidence that analyte is present. |
| Q | Q |  |  |  |  |  | The result did not pass the lab quality checks and there was an insufficient amount of the sample for re-analysis. |
| QC | FQC QC |  |  |  |  |  | QC problems |
| QCI | QCI |  |  |  |  |  | Quality Control incomplete |
| R | R |  |  |  |  |  | Rejected: The sample results are unusable due to the quality of the data generated because certain criteria were not met. The analyte may or may not be present in the sample. |
| REX | REX |  |  |  |  |  | Re-Prepared |
| RHT | TOC |  |  |  |  |  | Sample received hot |
| RIN | RIN |  |  |  |  |  | Re-Analyzed |
| RLRS | RLRS |  |  |  |  |  | Reporting limit raised, low total solids |
| RMAX | RMAX |  |  |  |  |  | result is a maximum value |
| RNAF | RNAF |  |  |  |  |  | result not affected by noted QC issue |
| RNON | RNON |  |  |  |  |  | result reported as non-detect due to blank contamination |
| RPDX | RPDX |  |  |  |  |  | RPD is MS/MSD pair exceeds criterion, estimated value |
| RPO | RPO |  |  |  |  |  | % RPD outside of acceptable limits |
| RPON | N | RPO |  |  |  |  | % RPD outside of acceptable limits. Presumptive evidence of a nontarget compound. |
| RRUDL | RR | UDL |  |  |  |  | Lab reported a result value, however the lab's detection limit is not known/available for validation or comparison to the result. |
| RRUDQ | RR | UDQ |  |  |  |  | Lab reported a result value, but lab's detection limit not known/available for comparison to the result; however with parameter specific evaluation determined to be quantified. |
| RSM | VVR | SM |  |  |  |  | Value verified by rerun, same method (USGS) |
| SBB | SBB |  |  |  |  |  | Estimated Value, less than blank |
| SCA | SCA |  |  |  |  |  | Suspected Contamination, lab analysis |
| SCF | SCF |  |  |  |  |  | Suspected Contamination, field |
| SCP | SCP |  |  |  |  |  | Suspected Contamination, lab preparation |
| SCX | SCX |  |  |  |  |  | Suspected Contamination, unknown |
| SD%EL | SD%EL |  |  |  |  |  | MS/MSD RPD exceeds control limits |
| SD%SS | MSD | + | SS |  |  |  | MS/MSD RPD exceeds control limits due to sample size difference. (TestAmerica Laboratory with a qualifier code of 'F4') |
| SDROL | SDROL |  |  |  |  |  | MS and/or MSD Recovery is outside acceptance limits |
| SLB | SLB |  |  |  |  |  | Spike level low compared to background |
| SM | SM |  |  |  |  |  | same method (USGS) |
| SSR | SSR |  |  |  |  |  | Surrogate standard acceptance criteria not met |
| SUS | SUS |  |  |  |  |  | Result value is defined as suspect by data owner |
| T | T |  |  |  |  |  | Hardness by Calculation Method - Standard Methods 2340B - 19th Ed |
| TMLF | TMLF |  |  |  |  |  | Time missing in logger file. |
| TOC | TOC |  |  |  |  |  | Temperature outside of criteria |
| TT | TT |  |  |  |  |  | analyte recalculated against alternate labelled compound(s |
| U | U |  |  |  |  |  | Not Detected: The analyte was analyzed for, but was not detected at a level greater than or equal to the level of the adjusted Contract Required Quantitation Limit (CRQL) for sample and method. |
| UB | B | U |  |  |  |  | The analyte was detected in the sample and in either the associated laboratory blank or field blank. If detected below the reporting limit (RL) the analyte result was reported as non-detected. If detected above the RL, the analyte result was reported as non-detected. |
| UD | U | DI |  |  |  |  | U D; Not Detected. Dilution required. |
| UH | H | U |  |  |  |  | Not Detected: The analyte was analyzed for, but was not detected at a level greater than or equal to the level of the adjusted Contract Required Quantitation Limit (CRQL) for sample and method. Holding time exceeded. |
| UJ | J | U |  |  |  |  | Not Detected/Estimated: The analyte was not detected at a level greater than or equal to the adjusted CRQL or the reported adjusted CRQL is approximate and may be inaccurate or imprecise. |
| UNC | UNC |  |  |  |  |  | Value Not Confirmed |
| UQ | FQC | U |  |  |  |  | Not Detected: The analyte was not detected at a level >= to the Reporting Level for the analysis. Also, the result did not pass the lab quality checks and there was an insufficient amount of the sample for re-analysis. |
| UREJ | K | R |  |  |  |  | Value below the detection Limit. Rejected: The sample results are unusable due to the quality of the data generated because certain criteria were not met. The analyte may or may not be present in the sample |
| V | V |  |  |  |  |  | Surrogate recoveries out of range |
| VVRR | VVRR |  |  |  |  |  | Value verified by rerun |
| Z | VRR | MTHD2 |  |  |  |  | Value verified by rerun, 2nd method (USGS) |